SA WG2 Meeting #168	S2-2503322

7 - 11 April 2025, Goteborg, Sweden

Source:	ZTE

Title:	UE policy enhancement to support data collection from UE

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes ways for UE Policy enhancements.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support standardized user plane transmission for AI/ML-related UE data collection." To enable the establishment of user plane connections from the UE (User Equipment) to NWDAF for AI/ML-related user data collection, this paper proposes a UE policy enhancement method that supports the establishment of user plane connections for AI/ML-related data collection.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: UE policy enhancement to support data collection from UE

6.x.1	Functional Description

To enable the establishment of user plane connections from the UE to specific network functions within the core network for AI/ML-related user data collection, this paper proposes to enhance UE policy(URSP rule). This paper specifies that user data is collected via the user plane for AI/ML purposes, such as model training. Additionally, it supports third-party AFs (Application Functions) in providing AI/ML-related user data collection parameters to core network functions (NFs) like UDR (Unified Data Repository) through the NEF (Network Exposure Function), allowing the UDR to notify the PCF (Policy Control Function). Based on these parameters, the PCF generates URSP rules and delivers them to the UE, ensuring seamless AI/ML data collection through optimized user plane connections.

6.x.2	Procedures

UEr

The untrusted AF can provide AI related data collection parameters to core Network as follow:

0a. UE Policy Association

AF

NEF

UDR

PCF

AMF

0b. Subscribe to UE policy change

1. Nnef_ServiceParameter_Create request

2. Request Verification and parameters mapping

3. Store AI/ML data collection parameters

5. Nudr_DM_Notify

4. Nnef_ServiceParameter_Create response

7. UE Policy

6. Generate URSP

8. Npcf_EventExposure_Notify

9. Nnef_ServiceParameter_Notify

Figure 6.x.2-1: Untrusted AF provide AI related data collection parameters

- 0a. The UE has registered to the network, and the AMF has established a UE policy association with the PCF.

- 0b. The PCF subscribes to notifications regarding changes in UE policy information from the UDR by invoking the Nudr_DM_Subscribe operation  (AF service parameter provisioning information, SUPI, Data Set setting to "Application Data", Data Subset setting to "Service specific information").

- 1. The AF invokes Nnef_ServiceParameter_Create to provide parameters related to AI/ML data collection to the NEF, which may include:

- Application information, such as Application ID and AF Service ID;

- UE information, such as UE ID (e.g. GPSI), UE address (e.g. IP address), UE group information (e.g. external group ID), and indication applicable to any UE;

- Indication for AI/ML data collection

- Location/area information (e.g. Geographical coordinates (latitude/longitude) or Street-based city addresses) for the data collection

- NOTE: Only when UE in the location provided, the data collection and PDU session establishment can be triggered.

- Data volume-related information (e.g. Data size, Data threshold, traffic offloading indication, allowing traffic offload (e.g. to non-3GPP access WLAN)) when the data volume exceeds a threshold or is significantly large;

- subscription to UE policy delivery result notifications, along with a notification address

- 2. The NEF may perform the following operations:

- Authorization and validation of the AF request

- Mapping external parameters to core network internal parameters, such as:

- Mapping Application ID, AF Service ID and indication for AI/ML data collection to a combination of S-NSSAI and DNN of an entity inside core network based on local configuration, Mapping external Application ID to an internal core network Application ID, Mapping location information to core network parameters such as TAI (Tracking Area Identifier) and Cell ID.

- Invoking UDM (Unified Data Management) services, such as Nudm_SDM_Get, for following parameter mapping:

- Mapping GPSI to SUPI, Mapping external group ID to an internal group ID

- Authorizing the service specific parameter provisioning request with the UDM by sending a Nudm_ServiceSpecificAuthorisation_Create

- Assigning a transaction reference ID

- 3. The NEF stores the mapped AF request parameters (from Steps 1 and 2) in the UDR, including: AI/ML data collection parameters (Indication for AI/ML data collection, location/area information, data volume-related information, DNN, S-NSSAI), Assigned transaction reference ID

- The data is stored in the "Service-Specific Information" subset within the "Application Data" dataset.

- 4. NEF responds to the AF with a Nnef_ServiceParameter_Create response message, which includes the transaction reference ID assigned by NEF.

- 5. The PCF receives a data change notification from the UDR via the Nudr_DM_Notify service. The notification may contain the AI/ML data collection parameters from Steps 1 and 2, such as AI/ML data collection indications, location/area information for data collection, Data volume-related information, DNN and S-NSSAI.

- 6. The PCF generates URSP rules based on the received information. The URSP rules may include:

- AI/ML data collection indications, possibly as a connectivity capability value in a URSP rule's traffic descriptor

- Location/area information, possibly as a parameter in the Routing Selection Descriptor (RSD)

- Traffic offloading indications, possibly as a parameter in the RSD (e.g. If the AF requested data volume is large)

- DNN and S-NSSAI

- 7. The PCF delivers the UE policy to the UE (see the next procedure)

- 8. If the AF has subscribed to UE policy delivery result notifications, and the PCF has received the UE policy container notification from AMF, then PCF sends a UE policy result notification to the NEF, using the Npcf_EventExposure_Notify message (UE information, Location/area information, AI/ML data collection indications).

- 9. The NEF maps the received PCF information to external information (e.g., mapping SUPI to GPSI). NEF then notifies the AF of the received information, including UE policy delivery result, Location/area information, AI/ML data collection indications. This is done using the Nnef_ServiceParameter_Notify message.

PCFr

AMFr

RAN

UEr

Following is the UE policy distribution procedure:

0a. Decide to update UE policy

6. UE Policy update result

5. UE Policy update result

4. UE update local policy

3. UE Policy distribution

2. Network initiated service request procedure

1. UE Policy Container

0b. Subscribe to UE policy update

Figure 6.x.2-1:  UE Policy distribution

0a. The PCF determines  to update the UE policy based on trigger conditions (e.g., initial registration, UE policy update required).

1. The PCF subscribes to the AMF for notifications regarding the UE’s response to the UE policy update.

2. The PCF invokes Namf_Communication_N1N2MessageTransfer to AMF with send a message containing:

SUPI , UE policy container, which includes:

AI/ML data collection indications (which may be a connectivity capability value in a URSP rule's traffic descriptor)

Location/area information (which may be a parameter in the URSP rule’s routing selection parameters)

Traffic offloading indications (which may be a parameter in the Routing Selection Descriptor (RSD))

DNN and S-NSSAI of an entity in core network

3. If the UE is in connected mode, the AMF delivers the UE policy container/UE policy information received from the PCF to the UE.

The UE policy container includes:

AI/ML data collection indications (which may be a connectivity capability value in a URSP rule's traffic descriptor)

Location/area information (which may be a parameter in the routing selection parameters of a URSP rule)

Traffic offloading indications (which may be a parameter in the RSD)

DNN and S-NSSAI of an entity in core network

4. The UE updates its locally stored UE policy based on the PCF-provided UE policy, which includes:

AI/ML data collection indications, Location/area information, Traffic offloading indications, DNN and S-NSSAI of an entity in core network

The UE utilizes the AI/ML data collection indication within the URSP rule's Traffic Descriptor (TD) to match the corresponding Routing Selection Descriptor (RSD) parameters and decides whether to establish a new PDU session or use an existing PDU session for transmitting AI/ML-related application traffic.

5. The UE sends the reception result (e.g. success indication) to the AMF.

6. The AMF forwards the UE’s response to the PCF by sending a Namf_Communication_N1MessageNotify message.

6.X.3	Impacts on services, entities and interface

To implement the solution, the AF needs to provide AI/ML data collection indication, location information of UE that for the training data collection, data size information as well as offload to non-3GPP WLAN indication if the data size is too large.

The NEF needs to do mappings.

The UDR needs to store the relevant information and inform PCF for policy update.

The PCF generate URSP rule which containing AI/ML data collection indications (which may be a connectivity capability value in a URSP rule's traffic descriptor), UE Location/area information (which may be a parameter in the URSP rule’s routing selection parameters), Traffic offloading indications (which may be a parameter in the Routing Selection Descriptor (RSD). The traffic offloading indication may be generated when the data size is too large. After generate the URSP rule, it distributes to UE using existing procedure.

---------- End of changes ----------