3GPP TSG-WG SA2 Meeting #168	S2-2503344

# Goteborg, Sweden, April 07 – 11, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1434 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates of NWDAF discovery and ML model provisioning for LMF-based AI/ML Positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Regarding the following EN: Editor's note: Whether the AI/ML positioning model needs to be trained per case (i.e. case 2b, case 3b) is FFS, this needs coordination with RAN WGs. According to the Reply LS from RAN2 (R2-2501507), RAN2 does not have any plan to work on Case 2b in Rel-19, and there will be only one case (i.e. Case 3b) in Rel-19 for LMF-based AI/ML Positioning, so we don’t need to introduce the positioning case information in Rel-19. - For ML Model provisioning from NWDAF for LMF-based AI/ML Positioning, the parameters of relevant services should be aligned with TS 23.273. - The name of the indication of supporting model training for LMF-based AI/ML Positioning should be aligned with TS 23.501 and TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the following EN, and remove the positioning case information for NWDAF registration and discovery. Editor's note: Whether the AI/ML positioning model needs to be trained per case (i.e. case 2b, case 3b) is FFS, this needs coordination with RAN WGs. - Update the parameters of Nnwdaf_MLModelProvision and Nnwdaf_MLModelInfo services operations based on TS 23.273. - Change the name of indication to LMF-based AI/ML positioning indication to align with TS 23.501 and TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with the progress of RAN WGs and other specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2A.1, 6.2A.2, 6.2A.3, 7.5.2, 7.5.4, 7.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If supporting model training for the LMF-based AI/ML Positioning, the NWDAF containing MTLF provides its NF profile to the NRF during registration with an LMF-based AI/ML Positioning indication. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

NOTE 4:	How to implement the LMF-based AI/ML Positioning indication in a backward compatible and extensible manner is up to stage 3.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 5:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. If the consumer is an LMF, it shall include in the request an LMF-based AI/ML Positioning indication that indicates the ML model is for LMF-based AI/ML Positioning. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Horizontal Federated Learning (HFL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 6:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID. The FL capability type only applies for HFL to maintain backward compatibility.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 7:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

In order to support VFL training and inference, the NWDAF supporting VFL shall include its VFL capability information per supported Analytics ID during registering to NRF. The VFL capability information includes VFL capability type (i.e. VFL server or VFL client or both) and Time interval supporting VFL if available.

To discover NWDAF supporting VFL from NRF, the consumer should consider the VFL capability information. For NWDAF as VFL server or NWDAF as VFL client, NWDAF also includes its VFL interoperability indicator(s) per Analytics ID, optionally with supported feature ID(s) and it may also include NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s), where data can be collected as input for local model training during registration to NRF. When NFs (i.e. NWDAF and/or AF) perform VFL together, only NFs sharing same VFL interoperability indicator can understand feature ID(s) and other VFL configurations from each other. Feature ID(s) represent the feature information supported by the NWDAF as VFL client per Analytics ID, i.e. VFL training and/or VFL inference. To discover NWDAF as VFL client via NRF, the consumer should also consider VFL interoperability indicator(s) and optionally consider Serving Area information and/or NF set ID(s) of the data source(s) and/or feature ID(s). The detailed procedure and parameters of NWDAF registration and discovery for VFL are as defined in clause 6.2H.2.1.

NOTE 8:	There are no standardized feature ID in this release, and only feature IDs related to static/preconfigured feature information can be registered to NRF.

# * * * * Next change * * * *

### 6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e.:

-	an NWDAF containing AnLF to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF;

-	an NWDAF containing MTLF to subscribe/unsubscribe at another NWDAF containing MTLF, i.e. an FL server NWDAF; or

-	an LMF to subscribe/unsubscribe at an NWDAF containing MTLF,

to be notified when ML Model Information related to the ML Model subscription becomes available, using Nnwdaf_MLModelProvision services as defined in clause 7.5. When the service consumer is an NWDAF containing AnLF, the ML Model Information is used by the NWDAF containing AnLF to derive analytics. When the service consumer is an LMF, how the ML Model Information is used by the LMF is described in TS 23.273 [39]. The service is also used by an NWDAF Service Consumer to modify existing ML Model Subscription(s). An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (e.g. NWDAF containing AnLF, NWDAF containing MTLF) subscribes to, modifies, or cancels subscription for a (set of) trained ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. If the NWDAF service consumer is LMF, it includes an LMF-based AI/ML Positioning indication.

NOTE 1:	How to implement the LMF-based AI/ML Positioning indication in a backward compatible and extensible manner is up to stage 3.

The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML Models if available.

When a ML Model subscription is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription; or

-	determine whether triggering further training for the existing trained ML Models is needed for the subscription.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from 5GC NFs (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML Model. For ML model training for LMF-based AI/ML positioning, the NWDAF containing MTLF collects input data from the LMF as described in TS 23.273 [39].

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) trained ML Model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer by invoking Nnwdaf_MLModelProvision_Notify service operation with:

-	For each Analytics ID requested by the service consumer, a set of pair(s) of unique ML Model identifier and the associated Information.

NOTE 2:	The structure and format of the ML Model identifier and its uniqueness are up to stage 3.

NOTE 3:	Parameters defined for Multiple ML Models are for Analytics accuracy enhancement.

The content of trained ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify an available re-trained ML Model when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When step 1 is for a subscription modification (i.e. including Subscription Correlation ID), the NWDAF containing MTLF may provide either a new trained ML Model different to the previously provided one, or a re-trained ML Model by invoking Nnwdaf_MLModelProvision_Notify service operation.

When the content includes ML Model provide indicator, the NWDAF service consumer (i.e. an NWDAF containing AnLF, an NWDAF containing MTLF or LMF) may trigger ML Model retrieval procedure in clause 6.2B.7 to retrieve ML Model(s) from ADRF.

# * * * * Next change * * * *

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML Model provisioning services (e.g. NWDAF containing AnLF, NWDAF containing MTLF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	A list of Analytics IDs: identifies the analytics for which the ML Model is used.

-	[OPTIONAL] Vendor ID: identifies the vendor of the consumer (e.g. of NWDAF containing AnLF, NWDAF containing MTLF).

For each Analytics ID, the following parameters may be provided:

-	[OPTIONAL] Use case context: indicates the context of use of the analytics to select the most relevant ML Model.

NOTE 1:	The NWDAF containing MTLF can use the parameter "Use case context" to select the most relevant ML Model, when several ML Models are available for the requested Analytics ID(s). The values of this parameter are not standardized.

-	[OPTIONAL] ML Model Interoperability Information. This is vendor-specific information that conveys, e.g., requested model file format, model execution environment, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes.

-	[OPTIONAL] ML Model Filter Information: indicates the applicable conditions of the trained ML model and enables the consumer to select which ML Model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model Filter Information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of ML Model Reporting: indicates the object(s) for which ML Model is requested, e.g. specific UEs i.e. a list of SUPIs, a group of UEs i.e. a list of Internal-Group-Ids or any UE (i.e. all UEs).

-	[OPTIONAL] Requested representative ratio: a minimum percentage of UE(s) in the group whose data is a non-empty set and can be used in the model training when the Target of ML Model Reporting is a group of UEs i.e. a list of Internal-Group-Ids.

-	[OPTIONAL] ML Model Target Period: indicates time interval [start, end] for which ML Model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	[OPTIONAL] Inference Input Data information: contains information about various settings that are expected to be used by the consumer (e.g. NWDAF containing AnLF) during inferences such as:

-	the "Input Data" that are expected be used, each of them optionally accompanied by metrics that show the granularity with which this data will be used (i.e., a sampling ratio, the maximum number of input values, and/or a maximum time interval between the samples of this input data).

NOTE 2:	This can be a subset of the possible Input Data specified for a certain analytics type.

-	the data sources that are expected to be used, indicated as a list of NF instance (or NF set) identifiers.

-	[OPTIONAL] Number of ML model(s), indicating the maximum number of ML models that the NWDAF containing MTLF could provide to the consumer (e.g. NWDAF containing AnLF) for the Analytics ID.

-	ML Model Reporting Information with the following parameters:

-	(Only for Nnwdaf_MLModelProvision_Subscribe) ML Model Reporting Information Parameters as per Event Reporting Information Parameter defined in Table 4.15.1-1, TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in clause 4.15.1 of TS 23.502 [3], allowing to correlate notifications received from the NWDAF containing MTLF with this subscription.

-	[OPTIONAL] Indication of supporting multiple ML Models.

-	[OPTIONAL] Accuracy level(s) of Interest.

NOTE 3:	Multiple ML Models Filter Information are composed by Accuracy level(s) of Interest and Number of ML Model(s).

-	[OPTIONAL] Time when model is needed: indicates the latest time when the consumer expects to receive the ML Model(s).

-	[OPTIONAL] ML Model Monitoring Information:

-	desired ML Model metric.

NOTE 4:	In this Release, only "ML Model Accuracy" is defined as ML Model metric.

-	[OPTIONAL] (only for Nnwdaf_MLModelProvision_Subscribe service operation) ML Model monitoring reporting mode: such as Accuracy reporting interval or pre-determined status. Depending on the reporting mode, the NWDAF containing MTLF reports the ML Model accuracy to NWDAF containing AnLF either periodically or when the ML Model accuracy is crossing an ML Model Accuracy threshold, i.e. the accuracy either becomes higher or lower than the ML Model Accuracy threshold.

-	[OPTIONAL] ML Model Accuracy Threshold: indicating the accuracy threshold of the ML Model requested by the consumer (as a kind of pre-determined status). It also can be used as an indication that the MTLF is triggered to execute the accuracy monitoring operations for the ML Model provisioned to AnLF.

-	[OPTIONAL] DataSetTag and ADRF ID if available: indicates the inference data (including input data, prediction and the ground truth data at the time which the prediction refers to) stored in ADRF which can be used by MTLF to retrain or reprovision of the ML Model.

-	[OPTIONAL] ML Model identifier: indicates the Model that the data corresponding to the DataSetTag is related to (in the case of subscription modification).

The NWDAF containing MTLF provides to the consumer of the ML Model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	For each Analytics ID requested by the service consumer, a set of pair (s) of unique ML Model identifier and the following information.

-	ML Model Information, which includes:

-	the ML Model file address; or

-	ADRF (Set) ID.

When ADRF (Set) ID is provisioned and the MTLF authorizes the NF Service Consumer to retrieve all ML Models corresponding to a Storage Transaction ID, a Storage Transaction ID may also be provisioned.

-	[OPTIONAL] ML Model provider information, includes the NF Instance Identifer to identify the NWDAF containing MTLF which is the ML Model provider and is responsible for training/updating this ML Model.

NOTE 5:	For example, if one MTLF provides the Model which is generated by FL server NWDAF, the NF Instance Identifier identifies the FL server NWDAF but not the MTLF.

-	[OPTIONAL] ML Model Filter Information and/or Target of ML Model Reporting may be also provided. They are provided when the ML Model provisioning request include same Analytics IDs but with different Targets of ML Model Reporting and/or ML Model Filter Information.

-	[OPTIONAL] ML Model provide indicator: indicates that the ML Model corresponding to the ML Model identifier is updated (e.g. re-trained ML Model).

-	[OPTIONAL] ML Model degradation indicator: indicates whether the provided ML Model is degraded.

-	[OPTIONAL] Validity period: indicates time period when the provided ML Model Information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML Model Information applies.

NOTE 6:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML Model Filter Information and of ML Model Target Period, respectively.

-	[OPTIONAL] ML Model representative ratio: indicating the percentage of UE(s) in the group whose data is used in the ML Model training when the Target of ML Model Reporting is a group of UE(s).

-	[OPTIONAL] Training Input Data Information: contains information about various settings that have been used by MTLF during ML model training, such as:

-	the "Input Data" that have been used, each of them optionally accompanied by metrics that show the data characteristics and granularity with which this data has been used (i.e. a sampling ratio, the maximum number of input values and/or a maximum time interval between the samples of this input data, data range including maximum and minimum values, mean and standard deviation and data distribution when applicable) and the time, i.e. timestamp and duration, when this data was obtained.

-	the data sources related to the "Input Data" that were used for ML Model training, which have been identified by a list of NF instance (or NF set) identifiers.

NOTE 7:	This can be a subset of the possible Input Data specified for a certain analytics type.

NOTE 8:	Data source information enables ML Model selection when different models are available for an Analytics ID, or it enables a consumer to avoid selecting a ML Model that used data from a specific data source at a particular time or used data characterized by specific data characteristics.

-	[OPTIONAL] ML Model Accuracy Information: indicates the accuracy of the ML Model if related ML Model Monitoring Information was provided, which includes:

-	the metric value of the ML Model.

-	[OPTIONAL] used ML Model metric.

To calculate the UE location, the LMF as the consumer can also use the ML Model provisioning services as described in clause 7.5 and clause 7.6 to retrieve ML model from NWDAF containing MTLF.The ML Model retrieving procedure by the LMF from NWDAF containing MTLF and the applicability of the parameters used in the ML Model provisioning procedure are as defined in TS 27.273 [39].

# * * * * Next change * * * *

### 6.2A.3	ML Model request

The procedure in Figure 6.2A.3-1 is used by an NWDAF service consumer, i.e.:

-	an NWDAF containing AnLF to request and get ML Model Information from another NWDAF, i.e. an NWDAF containing MTLF;

-	an NWDAF containing MTLF to request and get ML Model Information from another NWDAF containing MTLF working as FL server; or

-	an LMF to request and get ML Model Information from NWDAF containing MTLF,

using Nnwdaf_MLModelInfo services as defined in clause 7.6. When the service consumer is an NWDAF containing AnLF, the ML Model is used by the NWDAF containing AnLF to derive analytics. When the service consumer is an LMF, the ML Model Information is used by the LMF as described in TS 23.273 [39]. An NWDAF can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.3-1: ML Model Request

1.	The NWDAF service consumer (e.g. NWDAF containing AnLF, NWDAF containing MTLF) requests a (set of) ML Model(s) associated with a/an (set of) Analytics ID(s) by invoking Nnwdaf_MLModelInfo_Request service operation. If the NWDAF service consumer is LMF, it includes an LMF-based AI/ML Positioning indication. The parameters that can be provided by the NWDAF Service Consumer are listed in clause 6.2A.2. The service consumer optionally indicates its support for multiple ML Models if available.

Whe an ML Model Information request is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the request; or

-	determine whether triggering further training for the existing trained ML Models is needed for the request.

If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from 5GC NFs (e.g. AMF/DCCF/ADRF), UE Application (via AF) or OAM as described in clause 6.2, to generate the ML Model. For ML model training for LMF-based AI/ML positioning, the NWDAF containing MTLF collects input data from the LMF as described in TS 23.273 [39].

2.	The NWDAF containing MTLF responds to the NWDAF service consumer by invoking Nnwdaf_MLModelInfo_Request response service operation including:

-	a set of pair(s) of unique ML Model identifier, the ML Model Information for each Analytics ID that the NWDAF service consumer requests.

The content of ML Model Information that can be provided by the NWDAF containing MTLF is specified in clause 6.2A.2.

# * * * * Next change * * * *

### 7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe

Description: Subscribes to NWDAF ML Model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2 (conditional, required if the ML model is requested for analytics generation), or an LMF-based AI/ML Positioning indication (conditional, required if the ML model is requested for UE location caculation), Notification Target Address (+ Notification Correlation ID).

NOTE:	Whether the LMF-based AI/ML Positioning indication is implemented in a new information element is up to stage 3.

Inputs, Optional:

-	Subscription Correlation ID (in the case of modification of the ML Model subscription).

-	For each Analytics ID, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-    In case of the LMF-based AI/ML positioning indication, the parameters as defined in clause 5.18 of TS 23.273 [39] may be provided.

-	NF consumer information, ML Model Reporting Information, Time when model is needed, ML Model Monitoring Information (including e.g. desired ML Model metric, ML Model monitoring reporting mode, ML Model Accuracy Threshold, DataSetTag and ADRF ID, ML Model identifier), Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator's policy).

Outputs, Optional: None.

# * * * * Next change * * * *

7.5.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify

Description: NWDAF notifies the ML Model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Notification Correlation Information, Set of:

-	(required if the ML model is provided for analytics generation) the tuple (Analytics ID, one or more tuples of unique ML Model identifier and the information as defined in clause 6.2A.2).

-    (required if the ML model is provided for UE location caculation) one or more tuples of unique ML Model identifier and the information as defined in clause 5.18 of TS 23.273 [39].

Inputs, Optional: ML Model Accuracy Information.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * Next change * * * *

### 7.6.2	Nnwdaf_MLModelInfo_Request service operation

Service operation name: Nnwdaf_MLModelInfo_Request

Description: The consumer requests NWDAF ML Model Information.

Inputs, Required: (Set of) Analytics ID(s) defined in Table 7.1-2 (conditional, required if the ML model is requested for analytics generation), or an LMF-based AI/ML Positioning indication (conditional, required if the ML model is requested for UE location caculation).

NOTE:	Whether the LMF-based AI/ML Positioing indication is implemented in a new information element is up to stage 3.

Inputs, Optional:

-	For each Analytics ID, the following parameters may be provided: ML Model Filter Information to indicate the conditions for which ML Model for the analytics is requested and Target of ML Model Reporting as defined in clause 6.2A.2, Requested representative ratio, ML Model Target Period, Use case context, Inference Input Data Information, indication of support for multiple ML Models, multiple ML Models Filter Information to indicate the conditions for which multiple ML Models are requested, ML Model Interoperability Information.

-    In case of the LMF-based AI/ML positioning indication, the parameters as defined in clause 5.18 of TS 23.273 [39] may be provided.

-	Vendor ID.

-	ML Model Monitoring Information.

Outputs, Required: Set of:

-	(required if the ML model is provided for analytics generation) the tuple (Analytics ID, one or more tuples of unique ML Model identifier and the information as defined in clause 6.2A.2.

-	(required if the ML model is provided for UE location caculation) one or more tuples of unique ML Model identifier and the information as defined in clause 5.18 of TS 23.273 [39].

Outputs, Optional: ML Model Accuracy Information.

# * * * * End of changes * * * *