3GPP TSG-SA2 Meeting #168	S2-2503363

# Goteborg, Sweden, April 7 – 11, 2025	(revision of S2-250XXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3911 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transitioning between operation in S&F mode and normal mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the last meeting, it was agreed to capture a description for transition between S&F mode and non S&F mode. It also reads that the satellite can change the operate mode depending on the feeder link availability and/or operator policy and regulatory. We propose that when a satellite changes its operation modes, the MME-onboard may decide to trigger the S1 release procedure with the existing cause "load balancing TAU required" so that the UE to initiate TAU procedure by considering, e.g. the UE's preferred Network Behaviour, MT data to be delivered, and/or subscription data. For idle UEs, the MME may wait until the UE become ECM-CONNECTED, or may paging the UE to be CONNECTED, and the MME can control that the procedures are not triggered simultaneously for serving UEs to avoid signaling congestion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | MME-onboard may trigger S1 release procedure with the existing cause "load balancing TAU required" when transition between the S&F mode and normal mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE and network expected behaviour when transitioning between operation in S&F mode and non-S&F remains undefined and may lead to mis-operation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

4.13.9.2	Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

When the satellite changes its operation modes as above, the MME may decide to initiate the S1 Release procedure with release cause "load balancing TAU required" (clause 5.3.5), e.g. determined by considering the Network Behaviour the UE can support, MT data to be delivered, and/or subscription data. The S1 and RRC connections are released and the UE initiates a TAU.

* * * * End of Changes * * * *