3GPP TSG-SA WG2 Meeting #168	S2-2503381

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	Assistance Information

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes normative text for Assistance Information.

1. Discussion

In SA2#167, study conclusions are updated including Assistance Information.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request.

-	Aggregation assistance information used to assist in the aggregation of AIoT responses received from multiple AIoT devices.

-	Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF.

NOTE:	AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) as pre-configured without input from the AF.

* * * End of Change * * * *