3GPP TSG-SA WG2 Meeting #168	S2-2503384

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	AIoT Readers

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes normative text for AIoT Readers.

1. Discussion

AIoT Reader needs to be described in the normative text in TS 23.369, as it is a new function. The LS from RAN3 (R3-250905) also needs to be considered to align with RAN WGs.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

4.5.2	AIoT Readers

An AIoT Reader is a function that communicates with the AIoT device by means of AIoT radio. An AIoT Reader includes AIoT RAN Reader and AIoT UE Reader.

-	AIoT Reader:	Common reader function in TR 38.769[X]. A function that communicates with the AIoT device by means of AIoT radio.

-	AIoT RAN Reader:	AIoT RAN Reader is an AIoT Reader included in AIoT RAN.

-	AIoT UE Reader:	AIOT UE Reader is an AIoT Reader included in an AIoT-eabled UE.

-	AIoT RAN:	A-IoT RAN in TR 38.769[X]. A logica element which hosts certain functions for AIoT as part of the functional in RAN. AIoT RAN includes AIoT RAN Reader and AIoT RAN node.

-	AIoT radio:	A-IoT radio in TR 38.769[X]. Radio interface between AIoT device and AIoT RAN node in topology 1 and between AIoT device and AIoT-enabled UE in topology 2.

-	AIoT RAN node:	A-IoT RAN node function in TR 38.769[X]. A function that contains e.g., the control of the A-IoT radio resources used towards the AIoT device. AIoT RAN node is an aggregated gNB and this gNB may or may not only provide the AIoT radio. In Topology 1, an AIoT RAN node may serve one or more readers. The AIoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

* * * Start of 2nd Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[X]	3GPP TR 38.769: "Study on solutions for Ambient IoT (Internet of Things) in NR".

* * * End of Change * * * *