3GPP TSG-SA WG2 Meeting #168	S2-2503385

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes normative text for AIoT Reader selection.

1. Discussion

In SA2#167, study conclusions are updated including AIoT Reader selection. LS from RAN3 (R3-250905) needs to be considered regarding AIoT Reader Selection.

1.1 We need to resolve the following EN.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

If AF request includes target device, the AIOTF may determine targeted RAN reader(s) directly based on the last known serving RAN reader(s).

If AF request includes target area, the AIOTF may determine targeted RAN reader(s) directly based on the optional RAN reader information.

If AF request does not include neither target device information nor targe area information, the AIOTF may not determine RAN reader(s) directly. AIOTF select the AIoT RAN as pre-configured.

It is proposed to delete the EN and add text accordingly.

2.2 We need to resolve the following EN.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

If AF request includes target device, the AIOTF may determine targeted RAN reader(s) directly based on the last known serving RAN reader(s). Then, AIOTF select the AMF and the AIoT RAN that serves the targeted RAN reader(s).

If AF request includes target area, the AIOTF may determine targeted RAN reader(s) directly based on the optional RAN reader information. Then, AIOTF select the AMF and the AIoT RAN that serves the targeted RAN reader(s).

If AF request does not include neither target device information nor targe area information, the AIOTF may not determine RAN reader(s) directly. Then, AIOTF select the AMF and the AIoT RAN as pre-configured.

It is proposed to delete the EN and add text accordingly.

3.3 We need to resolve the following EN.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

NEF converts target area information from geographical information into cell/TA or service area information that AIoT devices and AIoT Readers are deployed.

It is proposed to delete the EN and add text accordingly.

4.4 We need to resolve the following EN.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

If AF request includes target device, the AIOTF may determine targeted RAN reader(s) directly based on the last known serving RAN reader(s).

If AF request does not include neither target device information nor targe area information, the AIOTF may not determine RAN reader(s) directly. Then, AIOTF select the AMF and the AIoT RAN as pre-configured.

It is proposed to delete the EN and add text accordingly.

5.5 We need to resolve the following EN.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Currently, RAN3 considers OAM based mechanism to provide AIoT RAN RAN reader information to AIOTF.

It is proposed to keep the EN and wait for RAN feedback(s).

5.6 We need to resolve the following EN.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Currently, RAN3 considers OAM based mechanism to provide AIoT RAN RAN reader information to AIOTF.

It is proposed to keep the EN and wait for RAN feedback(s).

5.7 We need to resolve the following EN.

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

Currently, RAN3 considers OAM based mechanism to provide AIoT RAN RAN reader information to AIOTF.

It is proposed to keep the EN and wait for RAN feedback(s).

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

5.3	AIoT Reader Selection

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. For example, NEF converts target area information from geographical information into cell/TA information as AIoT devices and AIoT Readers are deployed. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

If AF request includes target device, the AIOTF may determine targeted RAN reader(s) directly based on the last known serving RAN reader(s). The, the AIOTF may select the AMF and/or the AIoT RAN that serves the targeted RAN reader(s).

If AF request includes target area, the AIOTF may determine targeted RAN reader(s) directly based on the optional RAN reader information. Then, the AIOTF may select the AMF and/or the AIoT RAN that serves the targeted RAN reader(s).

If AF request does not include neither target device information nor targe area information, the AIOTF may select the AMF and/or the AIoT RAN as pre-configured.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	RAN reader information is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

* * * End of Change * * * *