3GPP TSG-WG SA2 Meeting #168	S2- 2503392

# Goteborg, Sweden, 07 – 11April 2025

Source: 	vivo

Title: 	KI#2, new sol: Service adjustments for network energy saving based on AF request and energy information

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose the new solution for KI#2 for TR 23.700-67.

1. Introduction

This contribution proposes a new solution for WT#2 for the r20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

FIRST CHANGE

6.x	Solution # x: Service adjustments for network energy saving based on AF request and energy related information

6.x.1	Description

This solution applies to Key Issue #2.

In the release 19 features, the AF could subscribe the energy consumption information exposure from the network at specific granularity, e.g, UE level, service data flow level, PDU session level. The AF may also provide the reporting information, e.g., reporting period, reporting frequency, or reporting threshold.

Besides the energy information exposure, certain application providers may have the following considerations:

- Certain application providers may be highly concerned with the energy consumption for its applications, and would like to aware the energy consumption information of its application. If the energy consumption per UE per application is too high, it could accept certain network energy saving behaviours to minimize the energy consumption for its application.

- Some other application providers, e.g., some factories, may have its mobile terminals accessing the local non-pubic 5G network. These application providers are considered with the network energy consumption of its mobile terminals. If the energy consumption per UE is too high, it could accept certain network energy saving behaviours to minimize the energy consumption.

In this solution, it is proposed that the AF may indicate to the network the expected network energy saving behaviours if the network energy consumption at certain granularity is above certain value. In this case, the possible network energy saving behaviours, e.g., policy control decisions, may be negotiated between the AF and operators. So, the possible network energy saving behaviours are considered as the preferred or acceptable from the application point of view, if the energy consumption information at certain granularity is very high. The network could also consider whether the UE is subject to network energy saving. So the final network energy saving behaviours may be based on the AF indication and UE’s subscription.

6.x.2	Procedures

Figure 6.x.3-1: Network energy saving behaviours negotiation between the AF and network

1.	The AF may provide the network with the preferred or expected network energy saving behaviours for its application’s service data flows or its users. The AF sends Nnef_NetworkEnergySaving_Creat Request to the NEF to provide the indication on the expected network energy saving behaviours, e.g., QoS adjustment. The energy consumption value at certain granularity for applying the network energy saving behaviours may also be provided.

2.	The NEF may interact with the UDM to authorize the AF for the requested service. If the network accepts the request from the AF, the NEF will respond to the AF to indicate the result. If the network can not accept certain network energy saving behaviours, the NEF may respond to the AF about the result.

3.	If the NEF accepts certain network energy saving behaviours, the NEF sends the information to the UDR via Nudr_DM_Create including the expected network energy saving behaviours and the related energy consumption value for applying the behaviour.

4.  The UDR further sends the information to the PCF via Nudr_DM_Notify.

5.  Based on the UE’s subscription for network energy saving and the AF request, the PCF may subscribe the energy consumption information notify from EIF via Neif_EventExposure_Subscribe/Notify.

6.	The PCF makes the policy decisions based on the UE subscription (Energy saving indicator), EIF notification and AF request.

6.x.3	Impacts on existing services, entities and interfaces

AF:

-	providing the expected network energy saving behaviours and the related energy consumption value for applying the network energy saving behaviours;

NEF:

-	supporting the inaction with AF about the expected network energy saving behaviours and the energy related information for applying the behaviours.

PCF:

-	make the policy decisions based on the UE’s subscription, AF request and EIF notification.

UDR:

-	New parameters addition related with network energy saving.

End of CHANGES