3GPP TSG-SA2 Meeting #168	S2-2503397

# Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0304 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the AF trafficInfluence information from SMF to I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently, only user plane latency requirement and indication of considering N6 delay is provided from the SMF to the I-SMF as part of policy update in the local offloading management scenario. As per the trafficInfluence information provided from the H-SMF to the V-SMF specified in clause 6.1.6.2.79 of TS 29.502, Stage 3 defines user plane latency requirement and indication of considering N6 delay to be sent from SMF to I-SMF within the trafficInfluence information When the Local Offloading Management is applied, the AF is not aware of whether the I-SMF is inserted for a PDU Session when as described in clause 5.6.7.1 of TS 23.501: "When an I-SMF is inserted for a PDU Session and if Local Offloading Management is applied (see TS 23.548 [130]), the I-SMF insertion shall be transparent (i.e. not aware) to the AF but the insertion of I-SMF is not transparent to the PCF." Therefore, it is required to incorporate the following legacy as part of general support for Edge computing when the Local Offloading Management is applies. - AF triggered EAS re-discovery or edge relocation; - AF request information including requirements for the Simultaneous Connectivity over the source and target PSA at Edge Relocation; - AF subscription for the UP path management events. It is proposed to clarify that the AF provided trafficInfluence information related to above is also provided from the SMF to the I-SMF when the local offloading management applies. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §6.10.1 - Propose to clarify that the AF provided traffic influence information is sent from SMF to the I-SMF.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear how the AF request related to legacy support for Edge computing applies in the Local Offloading Management scenario. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR yyy2 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.10.1	General

In edge computing deployment, to reduce the impact on a centrally deployed SMF on management of the edge computing related information, 5GS may support the Local Offloading Management.

In Distributed Anchor Point and Multiple PDU Sessions connectivity models (see clause 4.3), the existing means such as URSP and SSC mode are used to support Local Offloading Management.

In Session Breakout connectivity model, the Local Offloading Management only applies to the non-roaming PDU session. For Session Breakout connectivity model, if the AMF determines that the UE is within the Local Offloading Management service area and Local Offloading Management is allowed in the SMF selection subscription data, AMF selects/re-selects an I-SMF to support Local Offloading Management. This also includes the case when the UE is within the SMF service area. For the Local Offloading Management, the I-SMF is in charge of edge computing handling and supports following functionalities:

-	EASDF configuration and DNS message handling, as defined in clauses 6.2.1 and 6.2.3, instead of SMF.

-	EASDF discovery and selection is performed by the I-SMF instead of SMF as described in clause 6.3.23 of TS 23.501 [1].

-	Management and retrieval of EDI from NEF as described in 6.2.3.4.

NOTE 1:	The SMF can be also preconfigured with the EDI.

-	Receiving of the Local Offloading Information from PCF via SMF, which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 2:	When the I-SMF is inserted to support Local Offloading Management, the SM NAS message is still terminated to the SMF.

When the I-SMF is inserted for the Local Offloading Management, i.e. the inserted I-SMF providing Local Offloading allowed indication to SMF, the SMF does not discover, select and configure EASDF. as well as DNS message handling for the locally offloaded traffic towards the local part of DN.

The SMF sends to the I-SMF the received Application Function influence on traffic routing Enforcement Control (i.e. received in PCC rules from the PCF) including DNAI(s), EAS IP replacement information provided by AF due to AF triggered EAS re-discovery or edge relocation, AF request information including requirements for the Simultaneous Connectivity over the source and target PSA at Edge Relocation and/or subscription for the UP path management events when the local offloadding management applies. If the I-SMF cannot serve the target DNAI, it invokes a Nsmf_PDUSession_SMContextStatusNotify service operation to send the target DNAI to the AMF to trigger the I-SMF reallocation. In this case, the SMF triggered I-SMF selection or removal specified in clause 4.23.5.4 of TS 23.502 [3] does not apply.

The UE is unaware of whether the Local Offloading Management is applied or not for a PDU Session.

* * * * End of Changes * * * *