3GPP TSG-WG SA2 Meeting #168 	S2-2503403

Goteborg, Sweden, April 07 – 11, 2025	(revision of S2-25xxxxx)

Source:	Huawei, HiSilicon

Title:	Clarification of the selected I-SMF for Local offloading traffic handling

Document for:	Approval

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract: this paper clarifies how the I-SMF is selected for local offloading traffic handling.

1. Introduction

To support Local Offloading handling the I-SMF is inserted into the signaling path when the UE is in LOM service area regardless whether there are related traffics to be offloaded. One question has been raised before on whether the inserted I-SMF is selected randomly first and need be reallocated later. In this paper we analyze this issue.

2. Discussion

When the UE is in the LOM service area and allowed to be offloaded locally per subscription data, the AMF selects an I-SMF for local offloading traffic handling. This is independent of whether there are ongoing traffics for this UE. There is a proposal to do the I-SMF reallocation per DNAI notified by SMF. The argument is that when the I-SMF is selected by AMF, it may be selected randomly as the traffic information is unknown by the AMF. So it need be reallocated later. From our view this is incorrect understanding and need be clarified.

How AMF select I-SMF for local offloading traffic management?

For one PDU session, it may include serval different AF traffic within this PDU session. From network view it does not know when and which AF traffic will be transferred within this PDU session. However, it is expected that the selected I-SMF should be able to steer the AF traffic to different L-PSA within the LOM service area. This means that the I-SMF is not expected to be selected per which traffic is ongoing.

In one LOM service area, the potential offloading point is limited and be determined. If the traffic is offloaded, it is always gone via those limited number dedicated offloading point, i.e. DNAI within this area. Hence if one SMF is always able to select and configure one related UPF for each DNAI within that area, this SMF can offload all AF traffic locally regardless which AF traffic is to be offloaded. And this SMF is the one to be discovered and selected as I-SMF in this LOM service area.

Observation 1: The I-SMF for LOM is not expected to be selected per which traffic is ongoing.

Observation 2: If one SMF can select and configure one related UPF for each DNAI within LOM service area, it can be selected as I-SMF for LOM.

What is the impact of I-SMF reallocation?

On the other hand, if the I-SMF is selected randomly early and rely on later I-SMF reallocation, this is not a suitable behaviour. The impact can be analysed as following:

-	How to do the I-SMF reallocation? One proposal is that only the DNAI provisioned by the SMF is considered for I-SMF re-selection by the AMF, i.e. change the I-SMF selection per AF traffic.

If there already other traffic has been offloaded via some dedicated DNAI, how to handle those ongoing AF traffic as the AMF has no those DNAI(s) information? Does the I-SMF need combine those selected offloading DNAI and the DNAI notified by SMF as final target DNAIs to the AMF? If yes, how to consider other offloading DNAI not selected now but maybe selected later, i.e., whether some further I-SMF reallocation also need be considered? Also does the I-SMF need keep monitoring AF traffic via UPF to avoid that some AF traffic which are not transferred anymore but the related DNAI has been selected before?  Does the I-SMF reallocation can always be re-trigged during the lifetime of the PDU session?

-	Extral signaling are introduced. For each reallocation the extral signaling are introduced. If I-SMF is selected like this, this will cause a lot of extra siganling per AF traffics during the lifetime of PDU session.

Considering above we do not see this I-SMF reallocation is a simple issue. Indeed, several issues need further discussion and get agreement. The fundamental issue is whether the I-SMF for LOM is selected per LOM area or per AF traffic?

To avoid any further reallocation the best way is to take all potential DNAI within LOM service area into account. If that, why not do it from the beginning, i.e. I-SMF selection per LOM service area?

Observation 3: if I-SMF is selected per AF traffic, several issues need be considered. It is not a simple task and open the door for endless extra signalling.

Way forward on the selected I-SMF

Taking all above analysis into account, the I-SMF selected for LOM shall be done correctly from the beginning, i.e., the “best” one to serve that area when this I-SMF is inserted into the signaling path. This “best” one can also be reflected as it can serve the LOM service area and be part of its NF profile.

At the S2#166AH meeting, the SMF which can serve as I-SMF for local offloading management is agreed to be identified via its NF profile, i.e., capability and service area. Hence it is possible to ensure those dedicated SMF be planned to serve the LOM service area early by network configuration. If one I-SMF cannot cover the whole LOM service area, it can adjust its LOM service area within the NF profile. This can avoid later unnecessary reallocation due to different ongoing AF traffic. If we do like this, we do not see the need of the I-SMF reallocation triggered by SMF.

Proposal 1: The I-SMF for LOM is identified by its NF profile and possible to be planned early by network configuration.

Proposal 2: I-SMF for LOM is selected per LOM area not per AF traffic. Any I-SMF reallocation triggered by SMF due to AF traffic is not needed.

3. Proposal

It is proposed to discuss and agree above Proposal. The related accompanied paper (S2-2503404/2503405/2503406) are also proposed on the concrete changes.