SA WG2 Meeting #168	S2-2503419

# 07 - 11 April 2025, Goteborg, SE	(revision of)

Source: 	vivo

Title: 	KI #3, New Sol: AMF (re-)selection based on energy related information

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys / Rel-20

Abstract of the contribution: This contribution is trying to provide a sol for KI#X on the AMF selection aspect.

- Introduction

This solution applies to WT#3 on the AMF selection aspects based on energy related information.

SP-250417 … WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information

This solution includes the following sub-aspects:

- Registration with AMF re-allocation considering energy saving

- AMF provides energy related information to NG-RAN for AMF selection

2	Proposal

It is proposed to capture the following solution in TR 23.700-67.

First change

6.x	Solution #X: AMF (re-)selection based on energy related information

6.x.1	Description

This is a solution related to KI#3:

‐	Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information.

This solution leverages some existing procedure for AMF selection/reselection based on energy related information including following parts

-	When the initial AMF receives the UE initial message or uplink NAS transport message as in clause 4.2.2.2.3 of TS23.502[a], the AMF may need to re-route the message to another AMF, e.g. when the initial AMF is not the appropriate AMF to serve the UE due to energy related information (e.g. under EnergySaving state as in TS28.310[b] or Energy inefficiency in TS 28.104[c]), the initial AMF may trigger the AMF re-allocation to re-route the UE message to a suitable Target AMF.

-	During the Target AMF query from the NRF, the NRF may take the energy related information in the AMF NF profile into account for Target AMF querying and feedback. Optionally, the initial AMF may indicate the NRF that energy saving or efficiency for Target AMF querying is needed, then the NRF considers it for target AMF discovery and feedback (e.g., returns the AMF with high Renewable Energy Ratio or Energy priority)

-	During NGAP interaction, AMF provides its energy related information (e.g., the Energy States, Renewable Energy Ratio, energy priority) to NG-RAN for subsequent of AMF selection.

6.X.2	Procedures

6.X.2.1	AMF re-allocation considering energy saving

Figure 6.x.2.1-1: AMF re-allocation procedure considering energy related information

1.	Initial AMF receives a UE initial message or Uplink NAS transport message.

2.	If the initial AMF detects that it can not serve the UE based on the energy related information, e.g., the initial AMF is under e.g., EnergySaving state or Energy inefficiency, the initial AMF triggers the AMF re-allocation as in the subsequent steps.

3.	Initial AMF queries NRF, the NRF may consider the energy related information in NF profile and returns available AMF instances (e.g., not in EnergySaving state, high energy priority, high Renewable Energy Ratio).

Optionally, during the querying, the initial AMF may indicate the NRF that energy saving or efficiency for Target AMF query is needed, the NRF considers it for target AMF discovery and feedback (e.g., returns the AMF with high Renewable Energy Ratio or Energy priority).

4.	Initial AMF selects the Target AMF based on the NRF feedback, then reroutes the UE registration request or Uplink NAS transport message to the Target AMF by Namf_Communication_N1MessageNotify message directly, or sends a NGAP Reroute NAS Request message with Target AMF info to the (R)AN, the (R)AN reroutes the UE initial NAS message to Target AMF.

6.x.2.2	Energy related information of AMF provisioned to NG-RAN

To satisfy the NG-RAN selecting appropriate AMF when considers the energy saving and efficacy, the AMF may provide its energy related information (e.g., the Energy States, Renewable Energy Ratio, Energy priority) to NG-RAN using NG-AP message as specified in TS38.413[d]. Following is an example for energy related information of AMF provisioned to NG-RAN using the NG setup procedure in TS38.413[d]. The AMF provides its Energy related information in the NG SETUP RESPONSE message.

When performs the AMF selection subsequently, the AMF selection functionality in the 5G-AN may consider the energy related information provided by AMF for Target AMF selection.

Figure 6.x.2.2-1: NG setup: successful operation for Energy related information provisioning

6.X.3	Impacts on existing services, entities and interfaces

AMF:

-	AMF may redirect UE initial NAS message by considering the energy related information.

-	AMF may provide energy related information to NG-RAN

-	AMF may indicate the NRF to consider that energy saving or efficiency for Target AMF query is needed.

NG-RAN:

-	receives the AMF provided energy related information.

-	performs AMF selection with AMF provided energy related information.

NRF:

-	takes the energy related information in NF profile into account for Target AMF querying.

-	when receives the indication that energy saving or efficiency for Target AMF query is needed from initial AMF, the NRF considers it for target AMF discovery and feedback.

Second change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[a]	3GPP TS 23.502 " Procedures for the 5G System (5GS); Stage 2".

[b]	3GPP TS 28.310 " Energy efficiency of 5G ".

[c]	3GPP TS 28.104 " Management Data Analytics (MDA)".

[d]	3GPP TS 38.413 " NG Application Protocol (NGAP)".

End of Changes