3GPP TSG-SA2 Meeting #168	S2-2503436

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

Source:	Nokia

Title:	Discussion on PDU set QoS explicit indication by NG-RAN or SMF

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 /Rel-19

Abstract of the contribution: This contribution discusses the necessity of an explicit indication by 5GS (the NG-RAN or the SMF) on whether PDU set QoS or Normal QoS has been applied as part of the notification control mechanism.

# Discussion

TR23.700-70 conclusions for “KI#1_QoS Notification without AQP” and “KI#1_AQP” indicate that the application needs to be aware of whether the requested PDU set QoS has been achieved (or not) and this can be achieved by notification control by the NG-RAN and AQP as follows:

“In order to support QoS Notification Control "GFBR cannot be guaranteed or can be guaranteed again" (see clause 5.7.2.4 of TS 23.501 [2]), for a GBR QoS flow with the QoS profile containing UL and/or DL PSDB and UL and/or DL PSER, instead of PDB and PER, the fulfilment or not of UL and/or DL PSDB or UL and/or DL PSER, or GRBR in the QoS profile are used by the NG-RAN to determine QoS notification if the PDU Set QoS handling is applied in the corresponding direction. Otherwise, the fulfilment or not of the PDB and/or PER in the corresponding direction is still used for the QoS notification.”

“The following conclusion principles are agreed for key issue #1_AQP.

1.	The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

2.	Based on the Alternative Service Requirements from AF, the PCF generates PCC rule with Alternative QoS parameters in a prioritized order, which can include UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER) and the corresponding reference to each Alternative QoS parameter Set.

3.	SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN. When the QoS profile includes UL and/or DL PDU Set QoS parameter, the Alternative QoS Profile shall include the corresponding PDU Set QoS parameters; otherwise, the Alternative QoS profile does not include the corresponding PDU Set QoS parameters(s) when the QoS profile doesn't include the UL and/or DL PDU Set QoS parameter.

4.	If QoS notification is determined as "GFBR cannot be guaranteed", the NG-RAN determines the AQP that can be used as a reference to the currently fulfilled QoS considering the priority order: If the PDU Set QoS handling is applied in the corresponding direction, the PSDB and PSER in the corresponding direction and GFBR in the AQP are used by the NG RAN to determine the AQP that can be used as a reference to the currently fulfilled QoS. …”

Observation 1: For a GBR flow, an application that provides PDU set QoS values assumes that they will be used in place of QoS values (as per PDU set QoS handling).

Existing QoS notification mechanism without AQP relies on NG-RAN providing a simple notification “GFBR can no longer (or can again) be guaranteed”. Updated specs (TS23.501 and TS23.503) implement the necessary changes to support PDU set QoS notifications by overriding the existing notification when PDU set QoS parameters are used by the NG-RAN.

Depending on whether the serving NG-RAN supports PDU set QoS handling (and notification) or not, either PDU set QoS handling or normal QoS handling will be applied respectively. This scenario will be very common as the upgrade of the access network is a gradual procedure that may take several years. The AF has no control over the selected NG-RAN and cannot be aware of its capabilities regarding PDU set QoS (Rel-19 or prior to Rel-19).

Observation 2: An application that provides PDU set QoS parameters cannot deduce whether PDU set QoS handling has been applied (using PSER and PSDB) or generic QoS (PER, PDB) has been used instead.

Unless NG-RAN (and/or the SMF) explicitly indicates that PDU set QoS parameters have been used, the AF cannot know what exactly has been fulfilled or not, and hence cannot decide on the best adaptation to perform.

For example, if an AF has requested {PSDB=25ms, PSER=10-4} but the tighter values {PDB=10 ms, PER=10-5} have been used by NG-RAN instead, the notification “GFBR not fulfilled” will be totally misleading for the application. A slight violation of PDB may generally be tolerable for the application and require no direct adaptation by the AF. Instead, in this scenario, the application would consider that since even the more relaxed PDU set QoS requirements have not been met, immediate and drastic adaptation is needed, whereas it is simply that PDU set QoS parameters have not been used at all. Similar issues arise if the notification “GFBR is guaranteed” is provided by the NG-RAN with the application being unaware of which is the QoS parameters that it refers to.

Observation 3: The existing notification mechanism as specified in TS23.501 is non-deterministic.

In notification control with AQP procedures, the NG-RAN provides a reference to the exact AQP that has been used. As per existing specifications of the AQP mechanism (without PDU set support), the NG-RAN provides only a reference to the QoS parameters index, as indicated in the following excerpt of TS38.413

“9.3.1.152 Alternative QoS Parameters Set Index

This IE indicates the QoS parameters set which can currently be fulfilled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Alternative QoS Parameters Set Index | M |  | INTEGER (1..8, ...) | Indicates the index of the item within the Alternative QoS Parameters Set List IE corresponding to the currently fulfilled alternative QoS parameters set. |

As the AQP is expected to include both QoS and PDU set QoS parameters (pending stage-3 implementation decisions), a clear indication is necessary regarding whether the PDU set or the existing QoS parameters of the AQP have been applied. This holds for all the interfaces from NG-RAN up to the AF.

Notice that the analysis provided throughout the document, and the observations also apply to notification control with AQP. In this case, either the NG-RAN needs to provide an explicit indication of whether the PDU set parameters of the AQP are applied or alternatively the SMF should indicate so based on PDU Set based handling support Indication received from NG-RAN.

Observation 4: All previous observations also apply to notification control with AQP.

The above observations indicate the necessity of an explicit indication by 5GS for notification control with(out) AQP. This new indication essentially discloses whether the NG-RAN supports PDU set QoS handling and can be implemented either by NG-RAN itself or by the SMF (is aware of the NG-RAN capability via PDU Set based handling support Indication).

Without this indication, the application cannot identify whether QoS notification mechanism with(out) AQP has used PDU Set QoS or normal QoS or PDU Set QoS parameters and hence can neither adapt accordingly nor properly evaluate the usefulness of PDU set QoS enhancements.

Proposal: Specify an explicit indication by 5GS (NG-RAN or SMF) on the use of the provided PDU set QoS values as part of the notification control (with AQP) mechanism as per CRs S2-2503433 (NG-RAN Alternative 1) and S2-2503434 (SMF Alternative 2).