3GPP TSG-SA2 Meeting #168	S2-2503437

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

Source:	Nokia

Title:	Discussion on PDU set based DSCP marking

Document for:	Discussion

Agenda Item:	19.3.2

Work Item / Release:	XRM_Ph2 /Rel-19

Abstract of the contribution: This contribution discusses the necessity of any new instruction from I-SMF to I-UPF over N4 to support PDU set based transport level marking as per “KI#3_ Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network”.

# Discussion

TR23.700-70 conclusions for “KI#3_ Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network” describe how the anchor SMF based on locally configured {DSCP values <=> PSIs} mappings may guide the PSA UPF to consider PDU Set Importance (PSI) via FAR.

“Leverage PDU Set QoS information for DSCP marking over N3/N9 in the transport network: the following conclusions are agreed for the baseline functionality:

-	For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF as described in clause 5.8.2.7 of TS 23.501 [2]. For a QoS Flow configured for PDU Set QoS handling, besides the above information, the SMF may further consider PDU Set Importance values to be used by the QoS Flow, to provide a list of DSCP marking values, each of the DSCP marking values corresponding to one or several PDU Set Importance values.

NOTE 1:	It is recommended that DSCP markings be used to vary the drop precedence between PDUs in the transport network nodes (e.g. IP routers) on the N3/N9 interfaces. If the Class Selector Codepoint of the DSCP markings varies within a QoS Flow, the packets of the QoS Flow can be reordered by the transport network.

NOTE 2:	The transport level marking values being provided on per-QoS Flow basis, it is up to operator deployments to enforce consistency of transport level marking in the transport network.

-	The SMF sends the mapping list of the transport level marking for DL packets (N3/N9 interface) and related PDU set importance value(s) in the FAR to the UPF, and the UPF performs the DSCP value marking based additionally on the PDU Set Importance per PDU Set.”

The conclusion has been implemented in TS23.501 as:

“For QoS Flows that are configured for PDU Set QoS handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values.”

Observation 1: It is only the anchor SMF that decides the mapping {DSCP values <=> PSIs} based on local configuration.

Transport level marking is supported since Rel-15 with the anchor SMF instructing the PSA UPF to perform transport marking considering ARP, 5QI, etc.. PDU sessions with I-SMF have been specified in Rel-16 as part of ETSUN work. Upon I-SMF/I-UPF insertion, as per existing specifications the I-UPF simply forwards any transport level marking received from N9 towards N3. This applies also to PDU set QoS handling as per TS23.501 excerpt below:

“I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.”

Observation 2: No new functionality has been specified in XRM_Ph2 regarding the I-SMF/I-UPF insertion.

Nevertheless, in Rel-19 an unnecessary and unclear condition has been included, namely that “When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to…”. Neither the instruction nor what would be the corresponding I-UPF DSCP handling in the absence of such an I-SMF instruction have been specified. Besides, how the I-SMF gets to know from the anchor SMF what to instruct the I-UPF for a QoS flow is also unspecified.

Observation 3: Addressing the handling of a new indication from I-SMF is beyond the scope of XRM_Ph2 and would require a new study.

The above observations indicate that the introduction of such an instruction is unnecessary, leaves several aspects unspecified and introduces significant risk that the transport level markings in N9 and N3 are not aligned.

Proposal: Clarify that the I-SMF has no role in determining the transport level marking as per CR S2-2503439 (TS 23.501 CR6221).