3GPP TSG-WG SA2 Meeting #168	S2-2503472

Goteborg, SE, 7 – 11 April, 2025	(revision of S2-250xxxx)

Source:	Sony

Title:	Update Command Procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Updates the Command procedure with clarifications and corrections

# 1. Introduction/Discussion

This pCR propose a number of updates as the following:

- In step 1, 2 and 3: The Target AIoT device information name should be aligned with the same name as in the Inventory procedure and this shall be a mandatory IE, as either this IE indicate a specific device or multiple devices. Even if this is a “address all” inventory, the IE would have a defined value.

- In step 1: the () brackets around “AIoT data” changed to [] brackets.

- In step 1: The text related to filtering shall be removed and replaced by the same EN as in the Inventory procedure on how a group of devices can be addressed.

- Step 4: Rewrite the steps to first have the positive part and then the negative outcome. Also remove the cause code, as this would be a stage 3 thing to decide.

- Correlation identifier changed the correlation ID as this is a term used before.

- Adding an EN whether the correlation ID is the same as in the Inventory Request or a new correlation ID unique for the AIoT device to receive the command request.

- Adding the AIoT device ID to the command request and the corresponding EN on the protection of this ID.

- Fix the term AIoT device to AIoT Device.

- Clarify that the AIoT Device ID sent to the AF is the permanent Device ID.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

### 6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, AIoT Device ID identification information, [Target area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], [AIoT data]) message to NEF.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

Editor's note:	AIoT Device ID identification information needs further definition especially when addressing multiple AIoT devices e.g. detailing how filtering can be done.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and AIoT Device ID identification information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device ID identification information, [Target area information], [Approximate number of AIoT devices], [Approximate D2R message size], [AIoT data]) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If the AIoT service operation cannot be processed, no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF. If the response was a reject the procedure stops here.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation ID AIoT Device ID, NAS Command Request) to the AIoT RAN. The correlation ID is unique correlation ID for each AIoT Device and corresponds to the AF service operation request, and is used for the AIOTF to correlate the service operation responses to the AF service operation request.

Editor's note:	Whether the correlation ID is the same correlation ID as in the Inventory Request is FFS. In this step the command Request and correlation ID is for an Individual device, but in the Inventory Request the correlation ID could be for a group of devices or an individual device. Meaning that if the Inventory request was for a group of devices the correlation ID should probably be updated and a new correlation ID and the individual AIoT Device ID must be sent to the AIoT RAN.

Editor's note:	Whether and how the AIoT Device ID is protected will be determined and aligned with SA WG3.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation ID, AIoT Device ID, NAS Command Request)) to AMF. The correlation ID is unique correlation ID for each AIoT Device and corresponds to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the AF service operation request.

Editor's note:	Whether the correlation ID is the same correlation ID as in the Inventory Request is FFS. In this step the command Request and correlation ID is for an Individual device, but in the Inventory Request the correlation ID could be for a group of devices or an individual device. Meaning that if the Inventory request was for a group of devices the correlation ID should probably be updated and a correlation ID and the individual AIoT Device ID must be sent to the AIoT RAN.

Editor's note:	Whether and how the AIoT Device ID is protected will be determined and aligned with SA WG3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation ID, AIoT Device ID, NAS Command Request) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

14.	The AIoT Device sends the AS D2R message (NAS Command Response) to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds a Command Response message (correlation ID, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed.

15b.	The AIoT RAN responds an Command Response message (correlation ID, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation ID, Command Response (correlation ID, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command_Notify message (AIoT Device Permanent ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command_Notify message (AIoT Device Permanent ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command_Notify.

# * * * * End of changes * * * *