3GPP TSG-WG SA2 Meeting #168	S2-2503477

Sweden, Goteborg, 07 – 11 Apr, 2025

Source:	China Mobile

Title:	pCR for TS 23.369: update on Assistance information provided to AIoT RAN Node

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC/ Rel-19

Abstract: This paper proposes an additional parameter – AIoT reader ID(s) in the Assistance information provided to AIoT RAN Node

# 1. Introduction/Discussion

This contribution proposes adding the parameter – AIoT reader ID(s)- to the Assistance information provided to the AIoT RAN Node in Section 5.4.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.369 v0.2.0.

# * * * * FIRST CHANGE* * * *

# 5	High level functionality and features

## 5.1	General

## 5.2	AIoT Services

The following AIoT Services are supported:

-	Inventory service;

-	Command service.

### 5.2.1	AIoT Inventory service

AIoT Inventory service is used to discover the AIoT devices, i.e. to obtain the AIoT device identifiers.

### 5.2.2	AIoT Command service

Three types of Command service operations are supported: Read, Write and Disable.

An AF uses the Command service Read operation to retrieve information from a specific AIoT device.

An AF uses the Command service Write operation to configure information into a specific AIoT device.

An AF uses the Command service Disable operation to permanently disable the capability of the AIoT device(s) to transmit RF signals

## 5.3	AIoT Reader Selection

The AIOTF selects AIoT RAN node(s) and, optionally, a list of the RAN readers. The AIOTF selects the AIoT RAN node(s) based on the internal area information.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	How AIOTF selects the AMF and the AIoT RAN in the indirect path is FFS.

The AF may provide the expected target area information in the AIoT service request. The NEF may convert the target area information received from the AF to internal area information. The NEF queries the NRF to obtain serving AIOTF(s) based on internal area information. Then, the NEF forwards the AIoT service request, including the internal area information, to the AIOTF.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

The AIOTF may have AIoT RAN information and the optional RAN reader information. For direct path, the AIOTF obtains the information from AIoT RAN by the NGAP or OAM configuration.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	RAN reader information is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) to determine targeted RAN reader(s) directly.

## 5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g., radio resource allocation, and AIoT reader selection.

The following assistance information may be provided:

-	AIoT service type (e.g. Inventory, Command);

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request;

-    Optionally, requested service area information (e.g., Area and/or AIoT RAN readers ID list).

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

## 5.5	AIoT Device Profile Management

The ADM may hold operator’s subscription data for the AIoT Device used in the network. If the AIoT Device is managed by the network, then the profile data for an AIoT Device is required in the network, otherwise the corresponding profile data (e.g. Device ID or credentials) is stored external to the network.

The AIoT Device ID is used by the AIOTF together with local configuration, 3rd party related context to locate the entity which stores the profile data of an AIoT Device.

In case the AIoT Device is managed by the network, the AIOTF checks whether the AIoT Device ID from AIoT Device has the profile data in the network and retrieves the profile data. The profile data for AIoT Device is different with UE subscription data as defined in TS 23.502 [4] clause 5.2.3, it is stored in the ADM (AIoT Data Management) network entity that exclusively supports management of AIoT Device’s profile data. The AIoT Device Permanent ID is the primary key for AIoT device profile data in the ADM.

Editor's note:	Whether ADM support ID security mechanism pending on SA WG3 decision.

The table 5.5-1 below describes information storage structures for AIoT device profile data.

Table 5.5-1: AIoT Device Profile Data

| Field | Description |
| --- | --- |
| AIoT Device Permanent ID | Uniquely identifies the AIoT Device. |
| Last known AIOTF information | Indicate the last known AIOTF that serves the AIoT device, or unknown |

Editor’s note:	SA WG2 will align security related materials with SA WG3 decision later.

## 5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform authorization of AIoT service requested by the AF, by retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1.

Editor's note:	Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

## 5.7	Identifiers

### 5.7.1	General

### 5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT device permanent identifier shall be allocated to each AIoT device. An AIoT device permanent identifier is assigned either by an operator or by a third party. The identifier is used to identify an AIoT device and locate the entity where the AIoT device related information is stored.

NOTE 1:	How to configure AIoT device with the permanent AIoT device identifier is out of 3GPP scope.

The permanent AIoT device identifier includes the following components:

-	The ID type, including:

-	Information indicating whether the ID includes a network identifier or not.

-	Information indicating whether the ID includes a third party identifier or not.

-	The identification information type, i.e. EPC or unstructured information.

-	The domain information includes none, one or both of the following:

-	A network identifier (i.e. PLMN ID (MCC and MNC) and/or Network Identifier (NID) as specified in TS 23.003 [6]), when the ID includes the network identifier.

-	A third party identifier used to identify a third party when the ID includes the third party identifier.

-	The identification information which is used to distinguish different AIoT devices within the scope identified by domain information (if available).

-	EPC as defined in clause 14 of [7];

-	The unstructured information.

Figure 5.7.2-1: AIoT device permanent identifier Structure

NOTE 2:	The coding for the above information and how to code the length for the identification information are left to stage 3.

For operator allocated ID, the network identifier is mandatory.

For third party allocated Part 1ID, the network identifier is not included and the third party identifier is optional.

NOTE 3:	The length of ID type, Network Identifier (if present) and the third party identifier (if present) are fixed. The length of the identification information is dynamic. The details are specified in TS 23.003 [6] and TS 29.xxx [xx].

Editor’s note:	The reference in NOTE 2 and NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

# * * * * END OF CHANGE* * * *