3GPP TSG-SA2 Meeting #168	S2-2503487

07 - 11 April 2025, Goteborg, SE

Source:	vivo

Title:	23.700-19: Architecture principles and assumptions

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: This paper is to propose the architecture assumptions for FS_5GSAT_Ph4_ARC WT#1, i.e. to support IMS voice call over GEO via NB-IoT NTN connecting to EPC, and UE-SAT-UE for 5GLAN services

# 1Discussion

The SID for the study on WT#1 IMS voice call over GEO via NB-IoT NTN connecting to EPC was agreed upon in SP-250400. Since this study involves multiple architectures, including IoT NTN, IMS regular calls, IMS emergency calls, and LCS, it is essential to discuss and establish baseline assumptions to facilitate a smoother discussion during the study phase.

#### 1.1 GEO voice service

##### 1.1.1 IoT NTN architecture:

As specified in TS 23.401 and TS 36.331, NB-IoT (GEO) satellite access via EPC generally supports data transmission through the Control Plane (S1-MME interface, SRB1bis) and/or the User Plane (S1-U interface, with a maximum of two DRBs). It can also be enhanced as an NB-IoT IP CAN to enable IMS voice calls.

Proposal#1: The EPC architecture with GEO satellite access for NB-IoT, as defined in TS 23.401 [x], is used as the reference architecture for supporting IMS voice calls.

Satellites generally utilize two types of payloads: transparent payloads and regenerative payloads. From deployment’s perspective, ss of now, no GEO satellites support regenerative payloads. Most operational GEO satellites rely on transparent payloads due to their simpler architecture and established use in communication network. From 3GPP standardization perspective, 3GPP has initiated discussions on regenerative payloads starting from Release 19, focusing primarily on NGSO (Non-Geostationary Satellite Orbits), including MEO (Medium Earth Orbit) and LEO (Low Earth Orbit) satellites. One advantage of using a GEO satellite to support IMS is the ability to leverage existing deployments.

Proposal#2: IMS voice calls assume that the serving GEO satellite operates with transparent payload only.

Fig.1 Example of IMS voice transmission path over NB-IoT (GEO) satellite access

As approved in SP-250400, the discussion on IMS voice calls over NB-IoT (GEO) satellite access connected to the EPC is limited to scenarios utilizing the SGi interface, as shown in Fig.1. NB-IoT traditionally operates with either CP or UP transmission. According to TS 36.331, clause 4.2.2, SRB1bis (used for CP-based data transmission) is disabled if AS security in the PDCP layer (which enables UP-based transmission) is activated. Furthermore, from the SA2 perspective, there is no standardized procedure to support establishing a single PDN connection for both CP and UP data transmission. Relying on a single plane simplifies the network architecture, reducing both implementation complexity and operational overhead.

Proposal#3: IMS signaling and IMS voice data are assumed to be transmitted simultaneously via either the Control Plane (CP) or the User Plane (UP), without a combination of both.

##### 1.1.2 IMS architectures for regular call and emergency call:

Voice calls using the IMS (IP Multimedia Subsystem) platform have been a standard feature for supporting VoIP as a replacement for voice over the CS domain in regular calls since Release 5 (as outlined in TS 23.228) and for emergency calls since Release 6 (as outlined in TS 23.167). As illustrated in Fig.1, both the CP and UP of NB-IoT IP-CAN can support NIDD and IP data transmission. However, there has been no commercial deployment of Voice service over NIDD. In contrast, the deployment of VoIP has seen widespread adoption across various sectors, becoming a key technology in modern communications, particularly with the transition to IP-based networks. To ensure a future-proof solution for this study, it is proposed:

Proposal#4: The IMS reference architecture, as specified in TS 23.228 for regular calls and TS 23.167 for emergency calls, is adopted as the baseline for supporting NB-IoT IP-CAN. This applies exclusively to VoIP services, with no support for VoNIDD or voice over CS domains.

IMS has been evolved to improve SIP signaling efficiency. The introduction of SIP compact form and SigComp within IMS ensures efficient signaling while minimizing overhead and resource consumption, especially in constrained network environments. This enhancement enables the delivery of IMS-based services even in low-bandwidth situations, such as NB-IoT with GEO satellite access, boosting the performance and scalability of VoIP across a wide range of platforms and network types.

The I1 interface (stage 3 protocol defined in TS 24.294, stage 2 in TS 23.292) was initially introduced to enable interworking between IMS-based networks (which utilize IP-based protocols like SIP) and non-IMS networks (such as legacy circuit-switched networks or PSTN). Although there has been no deployment yet, it could serve as a starting point for replacing SIP signaling if SIP-based optimizations fail to meet the required 30-second call setup time or better signalling performance is expected.

Proposal#5: Existing 3GPP-supported SIP compression mechanisms, including SigComp (as defined in TS 24.229, clause 8.1), SIP compact form [RFC 3261, clause 7.3.3], and the SIP substitution I1 protocol as specified in TS 24.294, are assumed to leveraged to the fullest extent possible to optimize IMS signaling efficiency.

As specified in GSMA NG.119-V3.0-4 and TS 23.167, there are two types of emergency calls:

-	UE-detectable emergency call: This requires the UE to verify if the dialed number is a valid emergency number and initiate the emergency call setup procedure. This call is prioritized in case of network congestion and is the standard type of emergency call, and suitable for international emergency numbers such as 911, 112.

-	Non-UE-detectable emergency call: The core network translates the dialed short number and routes the call to the appropriate PSAP. This is used when operators need to support local emergency numbers to comply with legislation, and these numbers are not detectable by the UE. This scenario is only feasible when emergency services are provided locally, i.e. suitable for local emergency numbers such as 119, 120 in China (as shown in Annex F, non-UE detectable emergency numbers).

Proposal#6: The study will prioritize scenarios supporting local emergency numbers with non-UE-detectable emergency calls.

##### 1.1.3 LCS architecture:

For NB-IoT NTN satellite terminals, the LPP protocol for transferring UE location information are not supported. Instead, the coarse location of the UE, derived from its GNSS information, is transmitted via the NAS protocol. Given the GEO scenario, additional constraints such as power consumption, limited bandwidth, further exacerbate the challenge. Therefore, it is proposed that:

Proposal#7: The positioning enhancements to the UE and network to support LPP is out of scope to support IMS emergency call using NB-IoT (GEO) satellite access.

As outlined in TS23.167, Location information is needed for 2 main reasons in emergency services. The initial purpose of the location information is to enable the IMS network to determine which PSAP serves the area where the UE is currently located, so that the IMS network can route the emergency session to the correct PSAP. The second purpose is for the PSAP to get more accurate or updated location information for the terminal during or after the emergency session where required by local regulation. The coarse location, derived from the accurate GNSS UE location, is designed to be similar to the size of a terrestrial cell (approximately 2km), which is sufficient for the initial purpose. For the second purpose, more precise location data based on GNSS UE location could be used.

Proposal#8: It is assumed that UE GNSS location information, including coarse location, will serve as the baseline for determining the UE location in emergency sessions.

#### 1.2 5G-LAN service

The 5G-LAN service is comprehensively defined in TS 23.501, which outlines its key features and capabilities within the 5G network architecture. If the service is to be realized through UE-satellite-UE communication, leveraging NR NTN architecture with a regenerative payload would be a suitable approach.

Proposal#9: The 5GC architecture with satellite access for NR NTN, as defined in TS 23.501, is used as the reference architecture for supporting 5G-LAN services.

# 2 Proposal

It is proposed to agree the architecture assumptions into TR 23.700-19 on FS_5GSAT_Ph4-ARC.

* First change *

# 2	References

[a]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[b]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[c]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[d]	3GPP TS 24.229: " IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP)".

[e]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[f]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[g]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[h]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[i]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

* Second change *

# 4	Architectural Assumptions and Principles

### 4.x	Architecture principles

In order to satisfy the normative stage-1 requirements defined for IMS voice call using GEO satellite access in TS 22.261 [a], the system shall:

-	ensure that the call setup time does not exceed 30 seconds for both regular and emergency calls.

### 4.y	Architecture assumptions

The following architecture assumptions are applied in this study:

-	The EPC architecture with GEO satellite access for NB-IoT, as defined in TS 23.401 [b] in Rel-18, is used as the reference architecture for supporting IMS voice calls.

-	IMS voice calls assume that the serving GEO satellite operates with transparent payload only.

-	IMS signaling and IMS voice data are assumed to be transmitted simultaneously via either the Control Plane (CP) or the User Plane (UP), without a combination of both.

-	The IP Multimedia Core Network Subsystem (IMS) reference architecture, as specified in TS 23.228 [c] for regular calls and TS 23.167 [g] for emergency calls, is adopted as the baseline for supporting NB-IoT IP-CAN. This applies exclusively to VoIP services, with no support for VoNIDD or voice over CS domains.

NOTE 1: 	Existing 3GPP-supported SIP compression mechanisms, including SigComp (as defined in TS 24.229 [d], clause 8.1), SIP compact form (as defined in clause 7.3.3 in [f]), and the SIP substitution I1 mechanism as specified in TS 24.294 [e], are leveraged to the fullest extent possible to optimize IMS signaling efficiency.

NOTE 2:	The study will prioritize scenarios supporting local emergency numbers with non-UE-detectable emergency calls.

-	The positioning enhancements to the UE and network to support LPP is out of scope to support IMS emergency call with NB-IoT IP-CAN

NOTE 3:	It is assumed that UE GNSS location information, including coarse location, will serve as the baseline for determining the UE location in emergency sessions.

-	The 5GC architecture with satellite access for NR NTN, as defined in TS 23.501 [i], is used as the reference architecture for supporting 5G-LAN services.

* End of change *