3GPP SA WG2 Meeting #168	S2-2403501

07 - 11 April, 2025, Goteborg, Sweden

Title:	KI #X: New Solution: CST reduction by pre-negotiate SDP parameters during SIP registration

Source:	MediaTek Inc.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract:	The contribution proposes a solution to KI #X: XXXXXXX

Introduction

This solution aims to reduce the call setup time by negotiate pre-configured SIP call SDP parameters in the SIP registration procedure.

Discussion

By moving the SDP negotiation (i.e. offer/answer) procedure during the Call Setup procedure “earlier”, i.e., move the negotiation phase to SIP Registration Procedure, can reduce later Call Setup procedure number of messages and message size. This can reduce the overall CST (Call Setup Time). Once this procedure is complete, the SDP parameters would be static in all the following calls, we may regard this “early” SDP negotiation procedure as a SDP pre-configuration procedure.

Proposal

A solution is proposed for KI#X for incorporation in the FS_5GSAT_Ph4-ARC TR23.700-19.

*** Start of changes (all new text) ***

6.X	Solution #X: CST reduction by pre-configured SDP parameters during SIP registration.

6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #X, " XXXXXXXXXXXXXXX".

6.X.2	Description

This solution aims to reduce the call setup time by pre-configured SIP call SDP parameters in the SIP registration procedure.

6.X.3	Procedures

Figure 6.X.3-1: Registration Procedure with “SDP pre-config” feature

Figure 6.X.3-1 is a signaling diagram for UE using GEO satellite access to successfully activate the “SDP pre-negotiation” mechanism, which includes the following steps:

0.	The UE is ATTACHED to a PLMN over GEO via NB-IoT NTN connecting to EPC and has established IMS PDN Connection. The UE acuiqres the IP address of the UE and the P-CSCF during the IMS PDN Connection establishment procedure.

1.	The UE sends a SIP REGISTER message, which includes a “SDP pre-config” feature tag to indicate that UE can support the UE support “SDP pre-config”, to the IMS CN.

2.	The P-CSCF supports “SDP pre-config” and determines to use the "SDP pre-config" mechanism and responds with a 401 Unauthorized message that also includes the"SDP pre-config" feature tag.

3.	The UE sends a (encrypted) REGISTER message, which includes an XML body containing the requested SDP parameters of UE side (e.g., IP address, port, and voice codec), to the P-CSCF.

4.	The P-CSCF selects the final SDP parameters, saves it in the P-CSCF and responds with a 200 OK message that also includes an XML body containing the pre-configured SDP parameters of P-CSCF side

5.	The pre-configured SDP parameters is saved in both the UE and P-CSCF sides, and the pre-configured SDP parameters determined during previous steps (i.e., during SIP Registration Procedure) will be used in all the following IMS calls during the IMS registration lifecycle

Figure 6.X.3-1a: Example of SDP negotiation during SIP registration procedure

Figure 6.X.3-1a is a more detailed SDP negotiation example during SIP registration procedure.:

Figure 6.X.3-2: Call Setup Procedure with “SDP pre-config” feature

Figure 6.X.3-2 is a signaling diagram of a UE using GEO satellite access makes a MO IMS call with another UE using TN access according, which includes the following steps:

0.	The UE successfully finished the IMS registration procedure with “SDP pre-config” feature (see Figure 6.X.3-1.

1.	The UE-1 using GEO access initiates an IMS call to the UE-2 using TN access and send INVITE without SDP.

2~9.	The IMS CN will use “SDP pre-config” parameters stored in the IMS CN to trigger SIP interactions between IMS CN and UE-2, the SIP interaction between IMS CN and UE-2 will apply the traditional SDP offer/answer process thus the IMS CN forwards the INVITE to UE-2 with SDP offer, and UE-2 responds 183 Session Progress with SDP answer. All the SIP interactions will not be forwarded to UE-1 until UE-2 has sent out the 180 Ringing.

10.	The IMS CN activate Dedicated EPS Bearer Context for voice with the UE-1 using the stored “SDP pre-config” parameters.

11.	The IMS CN forwards 180 Ringing to UE-1 without SDP.

12~13.	UE-2 answers the call, send 200 OK to UE-1 and UE-1 responds ACK to UE2.

14.	The media paths between UE-1 and IMS CN, and between UE-2 and IMS CN have been established

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	Support “SDP pre-config”

IMS Server

-	Support “SDP pre-config”

*** END of changes ***