3GPP SA WG2 Meeting #168	S2-2503504

07 - 11 April, 2025, Goteborg, Sweden

Title:	KI #X: New Solution: IMS PDN Activation with both default and dedicated EPS bearer contexts

Source:	MediaTek Inc.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract:	The contribution proposes a solution to KI #X: XXXXXXX

Introduction

This solution aims to reduce the time needed for Call Setup procedure.

Discussion

For UP, the dedicated EPS bearer context activation and related RRC procedures (e.g., RRCReconfig(ActivateDedicatedEPSBearerContextReqeust), RRCReconfigComplete, ActivateDedicatedEPSBearerContextAccept) contributes to time delay for the whole CST (Call Setup Time), the time can be saved if the dedicated EPS bearer context activation can be pre-activated (as a "shell" dedicated EPS bearer), e.g., activated before the SIP call setup procedure (i.e., before SIP INVITE), e.g., activated during the IMS PDN connectivity activation procedure instead of activated during the SIP call setup procedure (i.e., after SIP INVITE). Thus the procedure can be

Simplied From:

-	IMS PDN Establishment with only default EPS bearer

-	SIP Register

-	Enter IDLE Mode

-	User wants to dial call, enter CONNECTED mode, SERVICE Procedure with IMS PDN default DRB establishment

-	During SIP Invite, voice dedicated DRB establishment

To

-	IMS PDN Establishment with both default EPS bearer and a "shell"dedicated EPS bearer

-	SIP Register

-	Enter IDLE Mode

-	User wants to dial call, enter CONNECTED mode, SERVICE Procedure with IMS PDN default DRB and voice DRB establishment

-	During SIP Invite, voice dedicated DRB establishment and dedicated EPS bearer establishment are not needed ( One RRC Reconfiguration Procedure can be saved after SIP INVITE, which saves time)

Proposal

A solution is proposed for KI#X for incorporation in the FS_5GSAT_Ph4-ARC TR23.700-19.

*** Start of changes (all new text) ***

6.X	Solution #X: IMS PDN Activation with both default and dedicated EPS bearer contexts.

6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #X, " XXXXXXXXXXXXXXX".

6.X.2	Description

For UP, the dedicated EPS bearer context activation and related RRC procedures (e.g., RRCReconfig(ActivateDedicatedEPSBearerContextReqeust), RRCReconfigComplete, ActivateDedicatedEPSBearerContextAccept) contributes to time delay for the whole CST (Call Setup Time), the time can be saved if the dedicated EPS bearer context activation can be pre-activated, e.g., activated before the SIP call setup procedure (i.e., before SIP INVITE), e.g., activated during the IMS PDN connectivity activation procedure instead of activated during the SIP call setup procedure (i.e., after SIP INVITE).

6.X.3	Procedures

Figure 6.X.3-1: Pre-Activation of Voice Dedicated EPS Bearer Context during IMS PDN Activation Procedure

Figure 6.X.3-1 is a signaling diagram for UE using GEO satellite access to utilize Pre-Activation of Voice Dedicated EPS Bearer Context during IMS PDN Activation Procedure, which includes the following steps:

1.	The UE is accessing a PLMN over GEO via NB-IoT NTN connecting to EPC and triggers IMS PDN Connectivity Activation procedure. The IMS PDN Connectivity Activation procedure can be combined with the ATTACH procedure or can be a standalone PDN Connectivity Activation procedure (i.e., activated after the ATTACH procedure). The UE indicates support of Pre-Activation of Voice Dedicated EPS Bearer Context to the NW in ims PDN CONNECTIVITY REQUEST message.

2.	The NW determines to use Pre-Activation of Voice Dedicated EPS Bearer Context (e.g., due to checking UE’s subscription and detecting the UE is accessing the PLMN over GEO via NB-IoT NTN connecting to EPC) initiates establishment of a "shell" Dedicated EPS Bearer Context for voice for future usage (i.e., used when the UE dials a voice call). The NW initiates the default and dedicated EPS bearers context activation procedures at the same time for the UE. An indication that the dedicated bearer is used for voice (e.g., QCI is set to 1) is also sent to the UE.

3.	The UE accepts both the default and dedicated EPS bearers activation.

4.	The RRC connection is released (e.g., due to no activity)

5.	The MME preserves the dedicated EPS bearer context.

6.	The user wants to dial voice call, triggers the UE initiating Service Procedure. Two DRBs, for both default EPS bearer and dedicated EPS bearer are activated by the network in a single RRC reconfiguration procedure

7.	The SIP call setup procedure goes without the need to activate the dedicated EPS bearer (DRB for voice), so some time can be saved.

8.	When the P-CSCF analyses the SDP in the SDP Answer and sends the Authorize/Authenticate-Request message to the PCRF with the related service information (IP address, port numbers, information on media-type). The PCRF authorises the request and identifies the affected IP-CAN session (e.g. default bearer) that has been established, and initiates a Re-Auth-Request to the PGW to initiate the creation of a dedicated bearer for voice with related traffic flow template. The PGW stores the related traffic flow template (in TFT), but does not triggers the Dedicated Bearer Activation Procedure (i.e., not sending CREATE BEARER REQUEST to SGW) and use the stored traffic flow template (in TFT) to filter/identify the DL voice packets in the  PGW and steer those packets over the dedicated bearer associated with "indication that the dedicated bearer is used for voice"

9.	The UE filters/identifies the UL voice packets based on the port number negotiated during the SIP call setup procedure and send these voice packets over the dedicated EPS bearer, and the PGW identify the DL voice packets according the stored traffic flow template (in TFT) acquired in step 8.

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	support of Pre-Activation of Voice Dedicated EPS Bearer Context

-	identify UL voice traffic based on the port number negotiated during the SIP call setup procedure and send on Voice Dedicated EPS Bearer Context

MME:

-	support combined activation of default and dedicated bearers

PGW:

-	support of Pre-Activation of Voice Dedicated EPS Bearer Context: stores the related traffic flow template (in TFT) from PCRF, but does not triggers the Dedicated Bearer Activation Procedure (i.e., not sending CREATE BEARER REQUEST to SGW) and use the stored traffic flow template (in TFT) to filter/identify the DL voice packets

*** END of changes ***