3GPP SA WG2 Meeting #168	S2-2503505

07 - 11 April, 2025, Goteborg, Sweden

Title:	KI #X: New Solution: IMS PDN as first PDN connection

Source:	MediaTek Inc.

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract:	The contribution proposes a solution to KI #X: XXXXXXX

Introduction

This solution aims to reduce the time needed to reach IMS registered state.

Discussion

The UE is not aware of whether a NB-IoT NTN network supports voice or not, thus the UE may activate a non-IMS PDN connectivity during the ATTACH procedure, then usually, if the network indicates the IMS voice over PS session in S1 mode supported in ATTACH ACCEPT activate another 2nd PDN for IMS. The 2nd PDN activation procedure increases the time needed before the UE can starts to do IMS registration; and the non-IMS PDN connectivity may not be used by the UE at all. Thus, instead of activating non-IMS PDN first, if the UE activates the IMS PDN as the 1st PDN some time will be reduced.

Proposal

A solution is proposed for KI#X for incorporation in the FS_5GSAT_Ph4-ARC TR23.700-19.

*** Start of changes (all new text) ***

6.X	Solution #X: IMS PDN as first PDN connection.

6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #X, " XXXXXXXXXXXXXXX".

6.X.2	Description

The UE is not aware of whether a NB-IoT NTN network supports voice or not, thus the UE may activate a non-IMS PDN connectivity during the ATTACH procedure, then usually, if the network indicates the IMS voice over PS session in S1 mode supported in ATTACH ACCEPT activate another 2nd PDN for IMS. The 2nd PDN activation procedure increases the time needed before the UE can starts to do IMS registration; and the non-IMS PDN connectivity may not be used by the UE at all. Thus, instead of activating non-IMS PDN first, if the UE activates the IMS PDN as the 1st PDN some time will be reduced.

6.X.3	Procedures

1.	The UE can be, by Home PLMN, (pre-)configured with an indication whether a NB-IoT NTN network (e.g., PLMN) supports Voice over GEO via NB-IoT NTN connecting to EPC.

2.	If UE supports Voice over GEO via NB-IoT NTN connecting to EPC, when the UE initiates the ATTACH procedure to a PLMN supporting Voice over GEO via NB-IoT NTN connecting to EPC, the UE initiates the ATTACH procedure together with the IMS PDN connectivity activation procedure.

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	support of (pre-)configured indication whether a NB-IoT NTN network (e.g., PLMN) supports Voice over GEO via NB-IoT NTN connecting to EPC

*** END of changes ***