3GPP TSG-SA WG2 Meeting #168	S2-2503532

Goteborg, Sweden, April 07 – 11, 2025

Source:	Xiaomi

Title:	KI#X: New Sol: Support QoS differentiation for CP based IMS voice service over NB-IoT NTN

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#X.

1. Discussion

To enable IMS voice over PS session, the serving PLMN shall send an indication toward the UE during Attach procedure or TAU procedure if an IMS voice service over PS session is supported. A UE with “IMS voice over PS” capability should take this indication into account when esatablish a bearerthat supports the Conversational Voice. As specified in TS 23.401 currently, the feature is not supported when UE is using NB-IoT RAT.

To address WT#1, i.e. IMS voice call over GEO via NB-IoT NTN, if the serving network supports IMS voice over NB-IoT NTN, a support indication should be provided to the UE to inform IMS voice service is allowed when using the NB-IoT NTN.

Another aspect is dedicated bearer is not supported over NB-IoT as specified in TS23.401, which implies differentiate QoS through multiple EPS bearers for IMS service is not supported when using NB-IoT. If there is QoS requirement for an IMS service, e.g. IMS signalling may have higher QoS than IMS voice data, the EPS connecting to NB-IoT NTN needs to be enhanced to support multiple bearers (i.e. more than default bearer).

2. Proposal

A solution is proposed for KI#X for incorporation in FS_5GSAT_Ph4_ARC TR 23.700-19.

* * * Start of Change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | X |
| x | x |  |  |  |

6.X	Solution #x: Support of QoS differentiation for IMS voice service over NB-IoT NTN

6.x.1	Description

As specified in TS 23.401, if an IMS voice service over PS session is supported by the serving PLMN, it shall send a support indication toward the UE during Attach procedure or TAU procedure. This indication is taken by the UE with “IMS voice over PS” capability into account when establishing a voice bearer for the IMS voice. However, the support indication is not applied to the NB-IoT RAT.

To support IMS voice over GEO via NB-IoT NTN connecting to EPC as listed in WT#1, the serving network should be enhanced to support this feature and provide the support indication to the UE as well in order to inform to the UE that IMS voice is allowed when using the NB-IoT NTN.

Moreover, according to the current status, only default bearer is supported to create when using NB-IoT (i.e. dedicated bearer is not supported over NB-IoT). This may not support the QoS requirement of an IMS voice service, e.g. the IMS signalling may have higher QoS requirement than IMS voice data. To meet different QoS requirement of various IMS data, creating multiple bearers, i.e. more than default bearer, needs to be enhanced by the EPS connecting to the NB-IoT NTN.

This solution describes the enhancemenet on multiple EPS bearer establishment over NB-IoT NTN to support IMS voice service. The solution is based on CP CIoT EPS Optimisation.

6.x.2	Procedures

Figure 6.x.2.1: IMS signalling and voice data delivery in CIoT EPS Optimisation over NB-IoT via GEO satellite

0. A GEO satellite of a NB-IoT access may broadcast its support of IMS voice service over NB-IoT.

1. A UE, camping on the NB-IoT cell, reads the related SIB information that the NB-IoT cell supports IMS voice service. If the UE can proceed to attach, it initiates the Attach procedure by the transmission of an Attach Request to the MME. The Preferred Network Behaviour indicating Control Plane CIoT EPS Optimisation is preferred is sent to the MME.

2. The MME procees the Attach request from the UE. If the attach is successful, it sends an Attach Accept message (Supported Network Behaviour, IMS voice over NB-IoT Support indication) to the UE. The MME indicates Control Plane CIoT EPS Optimisation is supported in the Supported Network Behaviour information. If the serving network supports IMS voice service over NB-IoT is supported when the UE is using the NB-IoT NTN, an IMS voice over NB-IoT Support indication is contained in the message as well.

3. Based on the IMS voice over NB-IoT Support indication, the UE can initiate a PDN connection establishement for IMS service by using PDN Contectivity Request. The request messeage includes an APN related to IMS service.

4. If the serving network supports IMS voice service over NB-IoT NTN, in order to provide differentiate QoS between IMS signalling and IMS voice data, the network can be enhanced to support establishing more than one bearer (e.g, default bearer for IMS signalling and one dedicated bearer for IMS voice data) for the IMS PDN connection over NB-IoT NTN. The creat session request/response is exchange between MME and S-GW and between S-GW and P-GW. Two EPS bearers (EPS bearer ID marked as EBI-1 and EBI-2) are established for the IMS PDN connection over NB-IoT.

5. The MME sends PDN Connectivity Accept including EBI-1 and EBI-2 to the UE.

6. The UE can initiate IMS signalling negotiation with remote UE by using CP CIoT EPS Optimisation as defined in clause 5.3.4B.2. The NAS PDU carries the encrypted uplink IMS signalling and EBI-1 to the eNodeB of the NB-IoT NTN.

6a. the eNodeB of the NB-IoT NTN, based on configuration, may retrieve the EPS negotiated QoS profile from the MME. The QoS profile may be applied to prioritise between requests from different UEs.

7. The NAS PDU sent in step 6 is relayed to the MME by the eNodeB using a S1-AP Initial UE message.

8. The MME checks the integirity of the incoming NAS PDU and decrypts the data it contains.

9. Based on the received EBI-1, the MME mappes the uplink IMS signalling data to the default bearer (i.e. EPS bearer-1) and deliveries it to the P-GW.

10. The Downlink IMS signalling is delivered via the default bearer, details can refer to steps 9-12 of Figure 5.3.4B.2-1 of TS 23.401.

11. The IMS signalling negotiation may trigger P-CSCF to send service requirement to the PCRF which provides the QoS requirement for the IMS voice data transmission. The PCRF may update the PCC rule for the IMS voice data and provides it to the P-GW.

12. The updated PCC rule may trigger P-GW to initiate bearer update to the S-GW and MME.

13. When the P-GW receives a downlink IMS voice data, it delivers the downlink data to the S-GW and further to MME via the dedicated bearer (i.e. EPS bearer-2).

14. The MME encrypts and integrity protects Downlink IMS voice data.

15. The MME sends the encrypt downlink IMS voice data along with EBI-2 using a NAS PDU carried by a Downlink S1-AP message.

16. The NAS PDUwith IMS voice data is delivered to the UE via a Downlink RRC message.

17. The uplink IMS voice data transmission via the dedicated bearer can refer to steps 16-19 of Figure 5.3.4B.3-1 of TS 23.401.

NOTE: 	Whether using one or more than one SRB to deliver NAS PDU of IMS signalling data and NAS PDU of IMS voice data is upto RAN implementation.

6.x.3	Impacts on services, entities and interfaces

UE:

-	Initiate IMS voice when using NB-IoT NTN access based on the received IMS voice service over NB-IoT Supported indication

-	Support to establish more than one bearer when using NB-IoT NTN and when using IMS service

MME:

-	Determine whether to support IMS voice over NB-IoT NTN for the UE based on the network operator policy

-	Support to establish more than one bearer for UE using IMS service over NB-IoT NTN

-	Support mapping IMS signalling data and IMS voice data to different EPS bearers and using differentiate QoS.

S-GW/P-GW:

-	Support to establish more than one bearer for UE using IMS service over NB-IoT NTN

- 	Support mapping IMS signalling data and IMS voice data to different EPS bearers using differentiate QoS

* * * End of Change * * * *