3GPP TSG-SA2 Meeting #168	S2-2503537

# 07 - 11 April, 2025, Goteborg, Sweden		(revision of )

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1444 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on VFL inference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some ENs should be addressed in VFL inference procedure. Editor's note: It is FFS whether additional parameters are needed to send from the VFL server to VFL client, e.g. parameters used in training phase and parameters from Analytics request. For this EN, some parameters used in training phase and parameters from analytics could be re-used in VFL inference procedure, and they should be described clearly. Thus, a new clause may be needed to describe the contents of VFL inference service. Editor's note: When the AnLF determine the VFL server AF is FFS. For example, during the training phase. When the AnLF determine the VFL server is based on the analytics consumer request, not related to the VFL phases. The AnLF may determine the VFL server e.g. when the AnLF received an analytics request from analytics consumer, but cannot generate the analytics output, it may discover the VFL server from NRF, as described in clause 6.2H.2.3. The name of results generated by the VFL client in VFL inference is confusing and inconsistent, it could be aligned as intermediate local inference results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe the contents of VFL inference service. Remove ENs in clause 6.2H.2.4.1 and 6.2H.2.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents of VFL related services are not clear and missing. The terminology for VFL inference results may lead to confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2H.2.4, 6.2H.2.4.1, 6.2H.2.4.2, 6.2H.X(new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.2H.2.4	Inference procedure for vertical federated learning

##### 6.2H.2.4.1	Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when NWDAF or Trusted AF is acting as VFL server

The inference procedure when trusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	The analytics consumer NF sends an Analytics request/subscribe (Analytics ID, Target of Analytics Reporting= e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period and Analytics Filter, Analytics Accuracy Request if the analytics consumer NF requests Analytics Accuracy Monitoring, and other parameters as defined in clause 6.1.3.) to NWDAF containing AnLF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

1.	If the NWDAF containing AnLF can be the VFL server to generate the VFL inference results for the requested analytics ID, then step 1 is skipped.

If the NWDAF containing AnLF cannot generate the analytics output, the NWDAF containing AnLF determines the VFL Server NWDAF or AF for the requested analytics (e.g. case A/B/C as described in clause 6.2H.2.3).

If the VFL server is trusted AF, the NWDAF containing AnLF sends a request to trusted AF VFL server using Naf_Inference_subscribe/request including Analytics ID, Target of Analytics Reporting = e.g. UE IDs and optionally Analytics Reporting Information=Analytics target period Analytics Filter and Analytics Accuracy Request.

2.	Based on the information received in the step 0 or 1, VFL server decides to initiate the VFL inference procedure with the VFL clients. Before the VFL server initiate the VFL inference procedure, the VFL server may initiate the VFL training procedure if no VFL model is already trained as described in the clause 6.2H.2.3, based on the information received in the step 0 or 1 or VFL server local configuration. VFL Server selects clients(s) using information stored in the VFL training process. The VFL server may select some or no clients, e.g. depending on the accuracy of the VFL model, the contribution to the training result and the current status of the VFL clients. The VFL client may report their status and other cause value for rejecting the VFL inference process (e.g. overload, target UE moved out of NWDAF serving area) in the VFL inference response service.

When no VFL Clients are selected, the VFL server may generates the VFL inference results based only on its local trained ML model associated with the determined VFL correlation ID, skipping the steps 2 - 6 (and 7 if step 1 was also skipped).

NOTE 1:	If the server does not select VFL clients for all features, it can internally store information about the features considered when deriving the analytics results together with a timestamp to identify the analytics results, to enable to subsequently trace the origin of analytics results and to interpret possible feedback on analytics accuracy.

NOTE 2:	If a client is not available for inference, e.g. depending on the contribution to the training result, the server can decide to select another available VFL client with a well-trained model, or the VFL server can start a new training procedure for a new VFL client.

VFL server NWDAF or trusted AF determines and sends a VFL Inference request/subscription to each VFL client with the parameters according to clause 6.2H.X.

2a.	For each NWDAF VFL client, the VFL Server NWDAF or trusted AF sends an Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client.

2b.	For each trusted AF VFL client, the VFL Server NWDAF sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the VFL client.

2c.	For each untrusted AF VFL client, the VFL Server NWDAF sends an Nnef_VFLInference_Subscribe or Nnef_VFLInference_Request to the NEF serving the AF.

2d.	For each untrusted AF VFL client, the NEF converts any internal identifiers to external identifiers and sends an Naf_VFLInference_Subscribe or Naf_VFLInference_Request to the untrusted AF VFL client.

NOTE 3:	For the trusted AF is acting as VFL server, the VFL client can only be the NWDAF.

3.	Each VFL Client collects its local data by using the current mechanism if the VFL Client does not have local data available already.

4.	Based on the VFL correlation ID, each VFL Client determines the VFL local model to generate the intermediate local inference results.

NOTE 4:	In this Release, it is assumed that intermediate local inference results are shared between VFL server and VFL client.

5.	VFL Client sends the client intermediate local inference results to the VFL server.

The intermediate local inference results, which are sent from the VFL Client to the VFL Server during the VFL inference process, are the information for the VFL Server to combine and generate the VFL inference results.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

5a.	Each NWDAF VFL client sends an Nnwdaf_VFLInference_Notify or Nnwdaf_VFLInference_Request response to the VFL Server NWDAF or trusted AF.

5b.	Each trusted AF VFL client sends an Naf_VFLInference_Notify or Naf_VFLInference_Request response to the VFL Server NWDAF.

5c.	Each untrusted AF VFL client sends an Naf_VFLInference_Notify or Naf_VFLInference_Request response to the NEF.

5d.	For each untrusted AF VFL client, the NEF converts any external to internal identifiers and sends an Nnef_VFLInference_Notify or Nnef_VFLInference_Request response to the NWDAF VFL server.

6.	The VFL server may collect its local data and generate its intermediate local inference results. When the VFL Server selected VFL clients to participate in the VFL Inference process, it combines all the intermediate local inference results to generate the VFL inference results based on the VFL correlation ID. The VFL server takes into account the participation of each VFL client during the ML training process and the importance of the intermediate local results when generates the VFL inference results.

The VFL server may compute the VFL accuracy based on all the intermediate local inference results received from VFL clients and the label, if it receives Analytics accuracy request in step 0.

7.	Depending on request, the NWDAF or trusted AF VFL server sends Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify or Naf_Inference_Response/Notify to the consumer (i.e. NWDAF containing AnLF) including the VFL inference results, optionally, VFL accuracy.

8.	The NWDAF containing AnLF provides the analytics output to the analytics consumer NF based on the VFL inference results by means of either Nnwdaf_AnalyticsInfo_Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 0.

The NWDAF containing AnLF may provide VFL accuracy if the consumer NF provides Analytics Accuracy request in step 0.

NOTE 3:	There may be some time delay between the time when the AnLF provides the analytics output to consumer NF and the time when the AnLF provides VFL accuracy, as the labels are collected after making predictions. The analytics result and VFL accuracy may be send in different response messages.

# * * * * Next changes * * * *

##### 6.2H.2.4.2	Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

Figure 6.2H.2.4.2-1: Inference procedure for vertical federated learning when untrusted AF is acting as VFL server

The inference procedure when untrusted AF is acting as VFL server may be triggered by a request or subscription from a 5GC consumer NF or internal service logic of the AF acting as VFL server. If triggered by internal service logic of the AF acting as VFL server, the steps 0, 1,7 and 8 are skipped.

0.	Same as step 0 in clause 6.2H.2.4.1.

1.	Same as step 1 in clause 6.2H.2.4.1, to NEF using Nnef_Inference_subscribe/request. NEF forwards the subscription request to AF using Naf_Inference_subscribe/request.

2.	Same as step 2 in clause 6.2H.2.4.1, to NEF using Nnef_VFLInference_subscribe/request. An untrusted AF includes the external NWDAF ID and sends the request to the NEF.

NEF converts any received external identifiers to internal identifiers and forwards the subscription request to NWDAF using Nnwdaf_VFLInference_subscribe/request.

2c	An NWDAF VFL client may send Nnwdaf_VFLInference_Subscribe or Nnwdaf_VFLInference_Request to the VFL client to other one or more NWDAF VFL client(s) as configured from which it desires to receive the client intermediate local inference results.

NOTE 1:	To support the intermediate results sharing between VFL clients in step 2c, the VFL clients might be selected based on the VFL Interoperability Indicators and/or the parameters in VFL Interoperability Information (e.g. gradient dimension of local model, split point of the preconfigured initial model, etc.).

3.	Same as step 3 in clause 6.2H.2.4.1.

4.	Same as step 4 in clause 6.2H.2.4.1.

NOTE 2:	In this Release, it is assumed that local intermediate inference is shared between VFL server and VFL client.

5.	Same as step 5 in clause 6.2H.2.4.1, to NEF using Nnwdaf_VFLInference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription notify to the untrusted AF using Nnef_VFLInference_Notify/Request Response.

5c-5d.	The NWDAF VFL clients share the client intermediate local inference results with the NWDAF VFL client from which it received request in step 2c. This NWDAF VFL client aggregates the received client intermediate local inference results, performs local computation using the aggregated intermediate local inference results, then sends one notify message to the NEF by including its client intermediate local inference result.

If the VFL server used an inference subscription in step 2, step 5 may be repeated.

6.	Same as step 6 in clause 6.2H.2.4.1.

7.	Same as step 7 in clause 6.2H.2.4.1, to NEF using Naf_Inference_Notify/Request Response.

NEF converts any received internal identifiers to external identifiers and forwards the subscription Notify to NWDAF using Nnef_Inference_Notify/Request Response.

8.	Same as step 8 in clause 6.2H.2.4.1.

# * * * * Next change (all new text) * * * *

### 6.2H.X	Contents of VFL Inference service

The consumers of the VFL inference services may provide the input parameters in Nnwdaf_VFLInference service or Naf_VFLInference service or in Nnef_VFLInference service as listed below:

-	Analytics ID: identifies the analytics for which the ML Model is requested to be used for inference.

-	(Only for Inference subscribe service) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the VFL client with the subscription.

-	VFL Correlation ID: indicate the VFL client which previously well-trained VFL local model associated with this ID will be used

-	Target of VFL inference: indicates the object(s) for which data for VFL inference is requested, i.e. a list of SUPIs, group of UEs identified by a list of Internal-Group-Ids or External-Group-Ids.

-	(Only for Inference subscribe) Subscription Correlation ID.

-	[Optional] Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1 of TS 23.502 [3].

-	[Optional] VFL inference filter.

- Inference target period: indicates the time interval for VFL inference;

- Time when intermediate local inference result is needed: indicates to the VFL client the latest time to receive intermediate local inference result;

- Data time window;

The VFL client provides to the consumer of the VFL inference service operations the output information in as listed below:

- (Only for Inference notify) Notification Correlation Information.

- Intermediate local inference results: the output of the client VFL local model, which is used for the VFL server to generate the VFL inference results.VFL correlation ID.

- Analytics ID.

# * * * * End of changes * * * *