3GPP TSG-SA WG2 Meeting #167	S2-2503546

17 - 21 Feb 2025, Athens, GR

Agenda item:	9.11.2

Source:	NTT Docomo, Nokia

Title:	Discussion on always-on PDU sessions for network slices subject to area restrictions.

WI/Release:	eNS_Ph3/Rel-18

Document for:	Discussion and Decision

# 1	Discussion

In rel-18 the features defined in TS 23.501 clause 5.15.17 "Partial Network Slice support in a Registration Area" and TS 23.501 clause 5.15.18 "Support for Network Slices with Network Slice Area of Service not matching deployed Tracking Areas" have been introduced. Inadvertently, these two new features have an impact on the support of "always-on PDU sessions" in the network slices that are subject to area restrictions defined by these features.

Clause 5.6.13 of TS 23.501 states:

- An always-on PDU Session is a PDU Session for which User Plane resources have to be activated during every transition from CM-IDLE mode to CM-CONNECTED state.

- The UE shall request activation of User Plane resources for always-on PDU Sessions even if there are no pending uplink data for this PDU Session or when the Service Request is triggered for signalling only or when the Service Request is triggered for paging response only.

Clause 5.6.8 of TS 23.501 indicates:

- If a PDU Session is an always-on PDU Session, the SMF should not deactivate a UP connection of this PDU Session due to inactivity.

The rationale of this is that there should not be a situation where an established PDU session that is marked as always-on needs to be reactivated when the UE is CM-CONNECTED. This improves the latency in accessing the services of the PDU session and avoids CM-CONNECTED SR before sending UL data. By extension, whenever a UE is CM-CONNECTED, the always-on behavior should still apply also when the S-NSSAI of the PDU sessions is subject to area restrictions. However, the features introducing the concept of S-NSSAI area restrictions have impacted the behaviour for all PDU sessions and it is the following:

- PDU sessions are deactivated when the UE exits the area of availability and support of the S-NSSAI

- It is expected that the UE is in charge of reactivating the PDU sessions when it enters areas of support and availability of the S-NSSAI only when there is data pending.

- The network takes no action on any PDU sessions upon returning to area of availability for any PDU session.

In addition, SA2 CR 5345r5 of TS 23.501/ S2-2409332 it was agreed that "When UE is outside of slice service area then UE shall not include Always-on PDU session in the list of PDU session to be activated.". This however means the Always-On PDU sessions become de facto normal PDU sessions as because - if nothing is done (see solutions A/B further below) - also the UE has to explicitly activate these upon receiving UL data for transmission when coming back to area of availability and support. In short, the always-on feature is lost as this means that even if the UE is CM connected in the area of availability of a slice, the always-on PDU sessions for the network slice are not necessarily activated and need to be handled as any other PDU session. The text in the current specification also fails to state that the Always on behavior needs to change as now the UE needs to check whether the DRBs for the PDU session are established even for Always on PDU sessions and if not proceed to service request (this is a dramatic change to always on PDU sessions handling).

Since we believe the always-on PDU session feature semantic needs to be retained, we propose that one of the following approaches is followed:

Solution A

Proposal A1: To avoid deactivating a PDU session that is always-on in the NG-RAN when the UE exists the area of availability, the SMF provides an always-on indication to the NG-RAN. The NG-RAN then keeps the PDU session activated but the SMF still receives the "out indication" from the AMF due to location reporting.

Proposal A2: The UE shall not send UL data and when it is outside the area of availability and support of a network slice (for DL the existing behavior of not sending DL data now applies when the UE is outside the area of availability also for active always-on PDU sessions),

Proposal A3: The UE can still request User Plane resources have to be activated during every transition from CM-IDLE mode to CM-CONNECTED state for always-on PDU sessions even when the always PDU sessions belong to network slices that are not available at the location where the transition occurs (i.e. the UE behavior is not impacted)

Solution B

Proposal B1: the SMF provides an always-on indication to the NG-RAN for PDU Sessions. When the UE exists the area of availability, the NG-RAN deactivates the PDU session but keeps in the UE context the PDU session information with Always on indication.

Proposal B2: When the UE comes back into the NS-AoS of the network slice, the NG-RAN triggers the reactivation of the PDU session by sending to the AMF a PDU Session Resource Notify for the Always on PDU session(s) for the network slice, if the UE context includes the PDU session information with Always on indication for a deactivated PDU session in the network slice.

Proposal B3: When the UE is not inside of the NS-AoS of a network slice, it shall not indicate to the network in a Service Request that has UL data for the PDU session of the always on PDU sessions for the network slice.

Solution A may impact the UE, solution B seems to not require any UE impact other than the one already in the specification (the UE shall not reactivate always on PDU sessions when not in the NS-AoS of a network slice of the PDU session).

# 2	Conclusions

It is recommended that one of solution A or solution B is adopted.