3GPP TSG-WG SA2 Meeting #168	S2-2503549

7th – 11th April 2025, Goteborg, Sweden	(revision of 25XXXXX)

Source:	Nokia

Title:	Updates on aggregation

Document for:	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: This paper proposes to add the descriptions about aggregation in the clause 4.5 of the TS 23.369 for Ambient IoT.

# Introduction

This paper is prepared for the normative work of Ambient IoT, to add more descriptions on the aggregation.

# 2. Text Proposal

It is proposed to capture the following changes to 3GPP TS 23.369 for Ambient IoT.

# * * * * First change * * * *

### 4.5.2	AIoT Readers

Editor’s note:	AIoT RAN and AIoT Reader will be aligned later with RAN WGs. It is FFS whether to specify the relationship between AIoT RAN and AIoT Reader and which term to use in this TS.

The AIoT Readers may aggregate data collected from multiple AIoT devices in accordance with the aggregation assistance information provided by the AIOTF.

### 4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an AIOT RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the AIoT RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-  Performing aggregation of the AIoT service responses. The AIoT service responses aggregated from the AIoT devices share the same correlation identifier. The AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

-	Determining the AIoT aggregation assistance information based on operator policy and information provided by  the AF and providing it to the AIoT RAN that contains time interval (The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF).

Editor’s Notes: Further details on how to use the AIoT aggregation assistance information will be captured in the procedure clauses and will require coordination with RAN3.

# * * * * End of changes * * * *