3GPP TSG-SA2 Meeting #168	S2-2503557

Gothenburg, Sweden, 7th April – 11th April, 2025

Source:	Huawei, HiSilicon

Title:	Discussion on enhancement to IMS network capability exposure

Document for:	Discussion

Agenda Item:	30.6

Work Item / Release:	Rel-20

Abstract of the contribution: Discusses why the enhancement to IMS network capability exposure to support IMS audio/video call and messaging need to be studied further in Rel-20.

# Introduction

This document analyses why the enhancement to IMS network capability exposure to support IMS audio/video call and messaging need to be studied further in Rel-20.

# Discussion

The following figure refers to Figure B.1-1 in 3GPP TR 23.722 [1] and shows the OMA Service Exposure Framework.

According to the analysis in Annex B of 3GPP TR 23.722 [1], OMA APIs provides an abstracted view of network through the Exposure Layer to the Service Provider, Developer Program, Content Provider, Systems Integrator, Advertiser, etc. located in the application layer. It’s northbound APIs of the Exposure Layer where the SCEF in EPC network and the NEF in 5GC network is located. The southbound of the Exposure Layer are the native network signaling protocols. According to 3GPP architecture for service capability exposure defined in clause 4.2 of 3GPP TS 23.682 [2], the SCEF invokes IMS network capability through S-CSCF via ISC interface.

Since OMA APIs do not support IMS data channel capability exposure. Rel-19 studies and specifies the architecture and procedures of IMS data channel capability exposure and the related northbound and southbound APIs of NEF, which includes N33 interface (northbound APIs) and N91 and N92 interface (southbound APIs) according to Annex AD.2.1 of 3GPP TS 23.228 [3].

Though OMA APIs has supported audio call (https://www.openmobilealliance.org/release/AudioCallREST/ ) and messaging (https://www.openmobilealliance.org/release/MessagingREST/ ) capability exposure, but there are still some gap need to be studied further.

Observation 1:  The southbound APIs of NEF to support IMS audio/video call and messaging capability exposure, is not covered by Rel-19 FS_NG_RTC_Ph2 SI.

Observation 2: Since OMA APIs do not support IMS data channel capability exposure, the 3rd party cannot establish IMS data channel session with audio/video. This prevents the 3rd party customer invoking some data channel based services, such as CAT and CRS service enhanced with data channel.

Observation 3: The application developers have to face OMA APIs and the northbound APIs of NEF which have different styles. It’s not user-friendly to application developers.

# Proposal

It proposes to study whether and how to enhance IMS network capability exposure to support IMS audio/video call and messaging in the scope of Rel-20 FS_NG_RTC_Ph3 SI based on the gap analysis of the current OMA APIs.

# Reference

- [1]	3GPP TR 23.722: "Study on Common API Framework for 3GPP Northbound APIs (Release 15)".

- [2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

- [3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".