SA WG2 Meeting #168	S2-2503576

# April 07 – 11, 2025, Gothenburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Support of capability exchange and NAS timer adjustment for IMS voice service over GEO satellite

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for WT-1 on IMS voice service over GEO satellite, in particular addressing capability negotiation and NAS timer. The solution utilizes the existing parameters, i.e. the UE's usage setting and the IMS voice over PS session indicator.

2	Proposal

It is proposed to add the following text to TR 23.700-19.

********** First Change (All new text) **********

6.y	Solution #Y: Support of capability exchange and NAS timer adjustment for IMS voice service over GEO satellite

6.y.1	Key Issue mapping

This solution addresses an aspect of WT-1 focusing on capability exchange and NAS timer adjustment.

TS 24.301 clause 4.7 has the following texts:

---

A UE in NB-S1 mode (see 3GPP TS 36.331 [22]) shall calculate the value of the applicable NAS timer:

-	indicated in table 10.2.1 plus 240s; and

-	indicated in table 10.3.1 plus 180s.

---

When an MME that supports NB-S1 mode performs NAS signalling with a UE, which is using NB-S1 mode, the MME shall calculate the value of the applicable NAS timer:

-	indicated in table 10.2.2 plus 240s; and

-	indicated in table 10.3.2 plus 180s.

---

This solution is written based on the assumption that these timer values are not appropriate for a voice call.

Editor's note:	WT and its numbering in the above will be replaced with Key issue and its numbering, once Key issues are defined.

NOTE:	This solution is intended to be used together with other solutions for WT-1.

6.y.2	Description

High-level solution principles are as follows:

- UE adjusts the NAS-MM timer and the NAS-SM timer. UE sends to MME the UE's usage setting that has the value of voice centric.

- MME adjusts the NAS-MM timer and the NAS-SM timer. MME sends to UE the IMS voice over PS session indicator that has the value of IMS voice over PS session in S1 mode supported.

6.y.3	Procedures

Figure 6.y.3-1: Capability exchange and NAS timer adjustment during the Attach procedure

- When UE using NB IoT decides to or is configured to use IMS voice service over GEO satellite, the UE adjusts the NAS-MM timer and the NAS-SM timer to the values defined for IMS voice service over GEO satellite.

- UE includes UE's usage setting that has the value of voice centric in the Attach request. UE sends the Attach request to MME.

- MME recognizes that UE's usage setting is voice centric and that eNB serves nb-IoT-GEO. MME decides to set IMS voice over PS session indicator to be IMS voice over PS session in S1 mode supported, if the MME supports IMS voice service over GEO satellite. MME adjusts the NAS-MM timer and the NAS-SM timer to the values defined for IMS voice service over GEO satellite.

- MME sends to UE the Attach accept containing the IMS voice over PS session indicator that has the value of IMS voice over PS session in S1 mode supported.

6.y.4	Impacts on existing services, entities and interfaces

UE:

- NAS timer adjustment. Use of the existing parameter UE's usage setting also for NB IoT.

MME:

- NAS timer adjustment. Use of the existing parameter IMS voice over PS session indicator also for NB IoT.

********** End of Changes **********