3GPP TSG-SA WG2 Meeting #168	S2-2503586

Gothenburg, Sweden, 7th – 11th April 2025

Agenda item:	19.4.2

Source:	Nokia

Title:	Discussion on the collection and exposure of data usage

WI/Release:	EnergySys /Rel-19

Document for:	Discussion and Decision

# 1	Discussion

The calculation of the energy consumption at EIF depends on the EIF retrieving from the SMF data usage at the desired granularity in association to the gNB ID(s) and UPF ID(s) that are used within a time interval T. There is no existing exposure Event ID that allows to do so at SMF. Hence the first thing that we should observe is:

Observation 1: The EIF requires the SMF to provide via the exposure API information that no existing exposure Event ID can provide

As a consequence, we propose this:

Proposal 1: The SMF exposure service is enhanced with a new Event ID dedicated to exposing usage data associated to the gNB ID(s) and UPF IDs within a time interval T.

It is then to be evaluated whether, to retrieve information from the UPF at SMF, it is best to use the existing N4 usage reporting via PFCP to SMF or the UPF exposure service should be used. The authors of this paper believe it is preferable to use the N4 approach as we need to be able to trigger reporting from UPF when events occur at AMF like change of gNB ID (detected via ULI reporting) and change of I-UPF. This is rather challenging to perform via a Nupf_EventExposure_Notify service operation at the UPF since the UPF itself is unaware of gNB-ID changes and the Nupf_EventExposure service does not support means for the SMF to retrieve on demand (e.g. upon a change of gNB) the on-going volume measurements of an on-going event exposure subscription (and to reset corresponding counters in the UPF), while this functionality is already supported by the PFCP protocol e.g. for charging.

Proposal 2: The SMF interacts with the UPF using PFCP  over the N4 interface to retrieve the necessary usage data for the exposure of usage data at desired granularity to EIF.

The SMF can configure in the UPF usage reporting rules (URR) with granularity corresponding to the requested usage reporting by EIF. In addition, periodic reporting can be configured aligned with the time interval T starting from an initial time T0.

When the SMF detects a gNB change, it can request an immediate reporting over N4 session for the intended Usage Reporting Rules (URR). The received Usage Report information then can be stored in the SMF in association with the old gNB-ID and new usage data for the URR will be associated to the new gNB-ID. When the periodic report is received from the UPF at the end of time interval T, then the SMF can report a list of gNB-IDs and related data usage at the granularity of the Usage Reporting Rule.

# 2	Conclusions

It is recommended that the following proposals are agreed:

Proposal 1: SMF exposure is enhanced with a new Event ID dedicated to exposing usage data associated to the gNB ID and UPF IDs within a time interval T.

Proposal 2: the SMF interacts with the UPF using PFCP over the N4 interface to retrieve the necessary usage data for the exposure of usage data at desired granularity to EIF.

These are implemented in CRs in S2-2503588 and S2-2503593.