SA WG2 Meeting #168	S2-2503600

7 - 11 April, 2025, Gothenburg, Sweden

Source: 	Xiaomi

Title: 	Key issue for WT#1 UE data transfer via UP for UE-side model training for AIML NR air interface operation

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/Rel-20

Abstract of the contribution: This contribution proposes the KI description for WT#1.

1. Introduction

1) SA2 Rel-20 5G-A SID FS_AIML_CN_Ph2

The Rel-20 5G-A SID on Core Network Enhanced Support for Artificial Intelligence (AI) / Machine Learning (ML) Phase 2 is approved in SA plenary as SP-250413. The following is included:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

2) Related Requirement to SA2 from RAN

- In RP-243316, the following is stated:

From RAN perspective, transfer of data over UP for Solution 2 in SA2 should be studied and provide feedback on fulfillment of visibility and controllability requirements.

TSG RAN kindly asks TSG SA to start studying the transfer of data over UP for solution 2 in WG SA2

- In RP-242389, the following is stated:

RAN2 has discussed the topic of UE side data collection for UE-side model training and agreed on a TP for TR 38.843 captured in the annex of this LS.

7.2.1.3.2	Data collection for UE-side model training

The following options were discussed in RAN2:

1a.	UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected.

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

3.	UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

The options listed above were analysed to understand potential specification impact. The analysis included aspects such as termination entities, data transfer path, whether control plane (CP) or user plane (UP) should be used to transfer the data, protocol layers involved, MNO controllability and visibility over the collected data, etc.

…

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | FFS: level of controllability (Note 5) (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure, FFS other procedures | Via RRC procedure |
| Possible Options for data content visibility in MNO (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. UE proprietary information can only be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of possible MNO controllability and visibility via Option 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 cannot reach consensus on the feasibility of UP tunnel in Options 2 and 3 |  |  |  |  |

- In RP-243316, the following is clarified from RAN:

- Clarification on controllability for data transfer

RAN2 wants to clarify the following for the discussion of the UE side data collection for UE side model training:

- Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements).

- Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server (as capture in Table 7.2.1.3.2-1) in TR38.843.

SA2 can assume that the NG-RAN is involved in providing radio measurement configuration (if needed) for the UE side data collection at least for the beam management use case, as captured in the following agreement in RAN2-127bis:

Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

Data transfer can happen at any point in time under network control if there is collected data available in the UE. Furthermore, for data transfer, RAN2 hasn’t studied/concluded on the level of NG-RAN involvement.

RAN2 will continue the discussion on data collection configuration and RAN2 will appreciate SA2/SA5 input on the data transfer process.

SA2 can assume that the gNB is involved in providing radio measurement configuration (if needed) for beam management use case and LMF is involved in providing PRS measurement configuration (if needed). However, RAN2 has not agreed that the gNB/LMF is in charge of “initiating, terminating and fully managing data transfer”.

- Clarification on visibility for data transfer

As stated in the LS sent from RAN, visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

3) Observations for WT#1:

Based on the above, there are the following observations:

Observation 1:	The data to be transferred is the UE collected data for UE-side model training.

Observation 2:	How the data for UE-side model training is collected is out of scope of WT#1.

Observation 2:	Only the solution for the standardised data transfer over UP is required to be studied.

Observation 3:	The data transfer over UP refers to the sending/transfer of the collected data from the UE to Server via UP with 5GC as intermediary node of the data transfer path between UE and the server.

Observation 4:	The data transfer over UP from UE to server is the primary object otherwise the server cannot receive the UE collected data to perform the UE-side model training, while fulfilment of visibility and controllability requirements is optional.

Therefore, the Key issue for WT#1 needs to be clarified based on the above, it is suggested to reach a common understanding on the requirement for WT#1 before dig into solutions, appreciating our effort on discussion for real requirement.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

5.x	Key Issue #x: UE data transfer via UP for AI/ML for NR air interface operation with UE-side model training

5.x.1	Description

In order to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training, the following aspects are to be studied:

-	Whether and how to support the standardized transfer of standardized UE data from UE to server over user plane.

-	Whether and how to support the visibility for the standardized UE data transferred over user plane.

-	Whether and how to support the controllability of the standardized transfer of standardized UE data to server over user plane.

NOTE 1:	The user data privacy and user consent aspects with respect to the visibility and controllability of the standardized transfer of standardized UE data to from UE to server require coordination with SA WG3.

NOTE 2:	How the data for UE-side model training is collected is out of scope of the WT#1.

* * * End of Change * * * *