3GPP TSG-WG SA2 Meeting #168	S2-2503604

Goteborg, SE, 7th April – 11th April, 2025

Title:	[Draft]LS out on support for multiple non-3GPP device identifiers

Response to:	LS (C1-250822) on support for multiple non-3GPP device identifiers from CT1

Release:	Release 19

Work Item:	UIA_ARC

Source:	[Huawei to be] SA2

To:	CT1

Cc:	CT3

Contact Person:

Name:	Marco Spini

Tel. Number:

E-mail Address:	m DOT spini AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2503605, S2-2503606, S2-2503607

1. Overall Description:

SA2 thanks CT1 for their LS on support for multiple non-3GPP device identifiers.

SA2 discussed the questions and would like to provide the following answer:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

Answer: The multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST.

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

Answer: The SMF will reject the whole PDU SESSION MODIFICATION REQUEST if any Non-3GPP device identifier is not available and send the unavailable non-3GPP device identifiers in the PDU SESSION MODIFICATION COMMAND message.

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

Answer: If the PCF has received the notification from UDR that the Non-3GPP Device Identifier Information has been deleted, the PCF shall delete the PCC rules associated with the Non-3GPP Device Identifier(s). All the non-3GPP device identifiers which influence the established PDU Session (e.g. the deleted PCC rules are associated with the existing QoS flow(s)) will be contained in one PDU SESSION MODIFICATION COMMAND message.

Based on the agreement, SA2 updated TS 23.501, TS 23.502 and TS 23.503 accordingly, please see the attachment S2-2503605, S2-2503606, S2-2503607 for details

2. Actions:

To CT1 group.

ACTION: 	SA2 asks CT1 to take the above response into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #169		19th May – 23rd May, 2025		Fukuoka City, Fukuoka, JP

TSG-SA2 Meeting #170		25th Aug – 29th Aug, 2025		Goteborg, SE