3GPP TSG-WG SA2 Meeting #168	S2-2503605

# Goteborg, SE, 7th April – 11th April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6236 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarificaiton on supporting multiple non-3GPP device identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the C1-250822, it is not clear that: - whether the multiple non-3GPP device identifiers are allowed in a single PDU session modification procedure - whether the SMF rejects the Non-3GPP Device Identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message - whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message when more than one non-3GPP device identifier are removed from the subscription. And only a single non-3GPP Device Identifier is allowed in one message may lead to increase signalling overhead within the system if the UE/5G-RG needs to request QoS differentiation for multiple devices in a short time. CT1 also checked that it is allowed the SMF can provide multiple non-3GPP device identifiers to the PCF in one message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification on supporting multiple non-3GPP Device Identifiers in PDU SESSION MODIFICATION message and in PDU SESSION MODIFICATION COMMAND message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no consensus in CT1 based on the existing description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the Non-3GPP Device Connection Information may be signalled by the UE as defined in TS 24.501 [47]. When a non-3GPP device is connected to the UE, the UE may include the Non-3GPP Device Connection Information in PDU Session Modification Request to SMF. The SMF forwards the Non-3GPP Device Connection Information to the PCF for policy control.

NOTE 1:	It is up to UE implementation to determine when to initiate PDU Session Modification procedure for updating the Non-3GPP Device Connection Information.

For an Ethernet PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	MAC address used in PDU session;

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU session.

For an IPv4 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv4 Address associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes:

-	Non-3GPP Device Identifier;

-	IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU session;

-	Optionally, port ranges associated with the non-3GPP device used in PDU session.

NOTE 2:	If IPv4v6 PDU Session is applied, it is up to UE implementation to determine to use IPv4 or IPv6 or both Address/prefix(sub) based on the associated traffic of the non-3GPP device and the IP allocation by the network to the PDU Session.

Editor's note:	The detailed encoding of Non-3GPP Device Connection Information will be defined in stage-3.

If the UE requests QoS differentiation for multiple non-3GPP devices at the same time, the multiple Non-3GPP Device Identifier(s) and their corresponding user plane information are allowed in one PDU Session Establishment Request.

If the PCF indicates to the SMF that there is Non-3GPP Device Identifier(s) which is not available for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the whole PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier is not available for the UE and includes the unavailable Non-3GPP Device Identifier(s) in the non-3GPP device connection information within the PDU Session Modification Command.

# * * * * End of changes * * * *