3GPP TSG-SA2 Meeting #168	S2-2503617

07 - 11 April, 2025, Goteborg, Sweden

Title:	[draft] Reply LS on OSAppID usage by AppToken use case

Response to:	LS (S2-2502797) on OSAppID usage by AppToken use case

Release:	Rel-19

Work Item:	TEI19

Source:                  SA2

To:                          SA

Cc:                          SA3

Contact Person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 provides the following updates to SA and asks to consolidate the responses (SA2 and SA3) and respond to GSMA TSGNS LS that was received in TSGNS44_003v6 (S2-2501377).

TSGNS Questions:

- Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

SA2 Answer:

AF guidance on URSP allows AFs to provide URSP rules with any OS App Id; please refer clause 4.15.6.10 of TS 23.502 for details. SA2 has not discussed how the OS App Id can be generated by an application backend and would like to ask question how to ensure the uniqueness of OS App Id if it is dynamically generated by an application backend.

- Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

SA2 Answer:

In the proposed solution OS notifies the App Token to the Operator Entitlement Server therefore the network is aware of the third party application information running in the UE. This may cause end user privacy issue. However SA2 recommend SA3 to provide answer on alternate mechanism to preserve end user privacy.

- Consider the architectural implications of the proposed mechanism and provide feedback.

SA2 Answer:

Please refer to the above answers of 1 and 2.

In addition when to enforce the URSP evaluation is per UE implementation. It is not guaranteed that the UE will use the S-NSSAI X for the 3rd party APP. Therefore the proposed solution may not work all the time. Also the change of S-NSSAI during the session may cause service interruption.

Further, SA2 shares below observations and alternate mechanisms to be considered.

SA2 specifications provide a number of traffic descriptors as defined in table 6.6.2.1-2 of TS 23.503 that allow applications to request for specific network slices and DNNs. Moreover, in Release 18, SA2 allows applications to indicate traffic categories, defined by GSMA NG.141, to accommodate network requirements, and in Release 19, SA2 has defined mechanisms that allow application servers to request to replacement of specific slices as defined in 5.15.5.2.2a in TS 23.501.

Additionally, SA2 would like to ask GSMA to clarify the validity of the AppToken and whether there are any associated timers, in order to identify if there are any impacts on the implementation on the UE side.

2. Actions:

To SA.

ACTION:

SA2 kindly asks SA to take above SA2 information into account and accordingly respond to GSMA TSGNSA LS that was received in TSGNS44_003v6 (S2-2501377).

3. Date of Next SA2 Meetings:

SA2#169	19th – 23th May 2025 		Fukuoka, Japan

SA2#170	25th – 29th Aug 2025 		Goteborg, SE