3GPP TSG-WG SA2 Meeting #168	S2-2403631

# Gothenburg, SE, 7th – 11th April, 2025		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5464 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On the report of SMF to EIF for user-plane energy consumption calculation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description on Energy related information collection for UP EC calculation becomes stable. However, one minor issue leftover is the details on the update of energy consumption information subscription, e.g., when the PDU Session of the UE is established/released during the reporting time period, the subsequent behavior regarding EIF/UDM/SMF is unclear. It is proposed to add the details on this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the details on the procedure upon the change on the PDU Session of the UE for energy consumption calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.29.2, 5.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specificsations |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * The First change * * * *

### 4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e., trusted AF or 5GC NF) subscribes to the energy consumption event with the EIF by using Neif_EventExposure_Subscribe service for the required granularities (UE, PDU Session and/or QoS flow) as described in clause 5.51.2.3 of TS 23.501 [2].

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF subscribes to be notified using Nudm_SDM_Subscribe regarding the establishment/release of the PDU Session(s) of the UE.

4.	The UDM notifies the EIF via Nudm_SDM_Notification including the PDU Session ID and SMF ID when PDU Session(s) of the UE are established or released at any time.

5.	The EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to request the information as defined in clause 5.51.2 of TS 23.501 [2]. The SMF ID is retrieved in step 2 or step 4.

6.	The SMF informs the UPF with requesting the PSA UPF to report the data volume of the required granularity.

7. 	The SMF provides the information for energy consumption calculation (defined 5.51.2 of TS 23.501 [2]) to EIF by invoking Nsmf_EventExposure_Notify service operation for the required granularity.

NOTE:	The SMF can derive gNB ID(s)s according to the AMF provided ULI as described in clause 4.3.2.

8.	EIF unsubscribes the subscriptions upon the expiration of the request time period.

9. 	If there is no related Node-level energy consumption information and Node-level data volume, the EIF obtains the Node-level energy consumption information and Node-level data volume from OAM with providing gNB ID(s) and UPF ID(s).

10.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

# * * * * The Second Change* * * *

### 5.2.3	UDM Services

#### 5.2.3.1	General

The following table illustrates the UDM Services and Service Operations.

Table 5.2.3.1-1: NF services provided by UDM

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Subscriber Data | Get | Request/Response | AMF, SMF, SMSF, NEF, 5G DDNMF, TSCTSF, NWDAF |
| Management | Subscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF, NWDAF, EIF |
| (SDM) | Unsubscribe | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF, NWDAF, EIF |
|  | Notification | Subscribe/Notify | AMF, SMF, SMSF, NEF, GMLC, 5G DDNMF, NWDAF, EIF |
|  | ModifySubscription | Subscribe/Notify | AMF, SMF, SMSF, NEF, 5G DDNMF, NWDAF |
|  | Info | Request/Response | AMF, NEF |
| UE Context | Registration | Request/Response | AMF, SMF, SMSF |
| Management | DeregistrationNotification | Subscribe/Notify | AMF |
| (UECM) | Deregistration | Request/Response | AMF, SMF, SMSF |
|  | Get | Request/Response | NEF, SMSF, GMLC, NWDAF, SMF, TSCTSF |
|  | Update | Request/Response | AMF, SMF, SMSF |
|  | PCscfRestoration | Subscribe/Notify | AMF, SMF |
|  | SendRoutingInfoForSM | Request/Response | SMS-GMSC |
| UE | Get | Request/Response | AUSF |
| Authentication | ResultConfirmation | Request/Response | AUSF |
| EventExposure | Subscribe | Subscribe/Notify | NEF (NOTE), NWDAF, SMS-GMSC |
|  | Unsubscribe |  | NEF (NOTE), NWDAF, SMS-GMSC |
|  | Notify |  | NEF (NOTE), NWDAF, SMS-GMSC |
|  | ModifySubscription |  | NEF (NOTE), NWDAF, SMS-GMSC |
| Parameter | Update | Request/Response | NEF, AMF |
| Provision | Create | Request/Response | NEF |
|  | Delete | Request/Response | NEF |
|  | Get | Request/Response | NEF |
| NIDDAuthorisation | Get | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| ServiceSpecificAuthorisation | Create | Request/Response | NEF |
|  | UpdateNotify | Subscribe/Notify | NEF |
| ReportSMDeliveryStatus | Request | Request/Response | SMS-GMSC |
| NOTE: Other NFs are allowed to consume the service based on roaming agreement or operator policy. |  |  |  |

# * * * * End of the Change* * * *