3GPP TSG-WG SA2 Meeting #168	S2-2503634

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	WT#1: EC improvement for Passive and dynamic EC

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: A solution, which defines a methodology to improve the evaluation of Energy Consumption taking into account the passive and dynamic part of energy consumption, is proposed. A second solution proposes to define a method to consider the contribution of Control Plane and other contribution to the energy consumption

# 1. Introduction/Discussion

This document proposes to several improvement to the energy consumption estimations as follow:

-	Solution #x: improvement considering the Passive Energy Consumption (i.e. the minimal amount of energy a system, device, or electronic component uses when it is in an idle or inactive state, without performing its primary function)  and the and the Dynamic energy consumption (i.e. the additional quantity of energy consumption used by a system, device, or electronic component during active operation, when it is performing its primary functions)

-	Solution #w: improvement of Energy Consumption considering the contribution of Control Plane and other contributions

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.xxx

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X | X |  |  |
| W | X |  |  |

# * * * * second change * * * *

## 6.X	Sol #X: Energy Consumption calculation considering passive and dynamic energy Consumption

This solution defines a methodology to improve the evaluation of Energy Consumption taking into account the passive and dynamic part of energy consumption.

### 6.X.1	Description

The principles of this solutions are described below:

1)	Use the mathematical approach defined in Rel-19

2)-	Introduce the following classification of the Energy Consumption at NF level:

a.	Energy Consumption (EC)  The Energy Consumption is the actual energy consumed by a system, device, or electronic component during active operation. This EC represent the actual energy consumption of the system which is considered in the evaluation in Rel-19.

b.	Passive Energy Consumption (PenC): the minimal amount of energy a system, device, or electronic component uses when it is in an idle or inactive state, without performing its primary function. This consumption often occurs when a device is in standby or sleep mode, where it remains partially powered to maintain essential functions (like memory retention or background monitoring) but doesn't actively perform tasks.

c.	Dynamic Energy Consumption (DenC): the additional quantity of energy consumption used by a system, device, or electronic component during active operation, when it is performing its primary functions, such as processing data or executing tasks This type of energy consumption fluctuates based on the workload or the level of activity within the system. Unlike quiescent energy consumption, which is the baseline power usage when idle, dynamic energy consumption depends on factors like processing speed, data throughput, and active component usage.

d.	Maximum Energy Consumption (EnCMax): The highest amount of energy consumption that a system, device, or electronic component can consume under peak operational conditions, i.e under 100% load condition. This typically occurs when the system is fully loaded or operating at its maximum capacity, utilizing all available resources (e.g., maximum CPU, GPU, or memory usage). Maximum energy consumption is an important factor in system design, as it determines the power supply requirements, cooling needs, and overall energy efficiency under the most demanding conditions.

e.-	Dynamic Energy Consumption Range (DenCRange): This interval represents the range of energy consumption that a system, device, or component consumes during active, variable operations, fluctuating according to the workload. This range captures all possible values of energy consumption when the system is actively performing tasks but not at peak load continuously, encompassing the variability in energy use based on real-time operational demands.

3)	the Energy Consumption of a NFs considered in Rel-19 can be expresses as equal to the PenC plus the DenC , i.e. the  DenC=EC-PenC. while the Dynamic Energy Consumption Range can be expressed as DenCRange=EnCMax - PenC

4) The resulting estimation formula for each granularity are:

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi=

=gNBi∈gNBUETDenCgNbDVUE,gNBDVgNB+ gNBi∈gNBUETf(DenCgNbi)+UPFi∈UPFUETDenCUPFDVUE,UPFDVUPF+UPFi∈UPFUETf(DenCUPFi)

(1)

The Esession is the same formula as in TS 23.501 Annex T

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi	(2)

but where

ESession,gNB=DenCgNBDVSession,gNBDVgNB+h(DenCgNB)	(3)

ESession, UPF=DenCUPFDVSession,UPFDVUPF+h(DenCUPF)	(4)

The Eflow is the same formula as in TS 23.501 Annex T

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi	(5)

but where

EFlow,gNB=DenCgNBDVFlow,gNBDVgNB+g(DenCgNB)	(6)

EFlow,UPF=DenCUPFDVFlow,UPFDVUPF+g(DenCUPF)	(7)

5)	The functions f(DenCUPF), f(DenCgNB), g(DenCgNB), g(DenCUPF), h(DenCUPF) and h(DenCgNB) for allocation of the Passive Energy Consumption are determined by the operator in order to fulfil its strategy

### 6.X.2	Detailed description

This clause describe din details how the formulas are obtained and some numerical example to show the difference with the approach in rel-19.

Based on rel-19 formula the EC amount is allocated to the lower granularity proportionally to the data volume, therefore in low load condition the EC assigned to each UE is proportionally higher than when more UE are present, since the Passive Energy Consumption (QenC) is shared among more UE/Data volume.

If we consider the above definition the Rel-19 formula can be modified as follow:

- for the gNb

EgNB=DenCgNB+PenCgNB	(8)

DenCgNB=EgNB-PenCgNB	(9)

- for the UPF

EUPF=DenCUPF+PenCUPF	(10)

DenCUPF=ECUPF-PenCUPF	(11)

Now for seek of simplicity consider only the per UE granularity, which is

EUE,gNB=EgNBDVUE,gNBDVgNB=DenCgNbDVUE,gNBDVgNB+f(PenCgNb)	(12)

EUE,UPF=EUPFDVUE,UPFDVUPF=DenCUPFDVUE,UPFDVUPF+f(PenCUPF)	(13)

The contribution of Passive Energy Consumption is shown as a generic function f(PenC) since how this contribution is allocated to the UE is proposed to be left to operator decision based on operator policy.

Substituting the equation (12) and (13) in the equation  in Annex T of TS 23.501 for the UE granularity the equation (1) is obtained. Following the same approach for the other granularity the equations (2) and (5) are obtained.

In case of gNB TS 38.864 in clause 5.1 defines the energy consumption model for BS as follow:

- For active DL transmission, the BS power consumption is provided by

PDL=PstaticDL+PdynamicDL	(14)

where

-	PstaticDL is a static part of power for BS in active, which is not scaled based on reference configurations.

-	PdynamicDL is a dynamic part of power for BS in active, which is scaled based on reference configuration.

- For active UL transmission, the BS power consumption for is provided by

PUL=PstaticUL+PdynamicUL	(15)

where

-	PstaticULis a static part of power for BS in active, which is not scaled based on reference configurations.

-	PdynamicUL=sa*Pdyn,anteUL a dynamic part of power for BS in active, which is scaled based on reference configuration, and s_a is the fraction of active TRxRUs.

For details description on how to evaluate the above parameters value refer to TS 38.864 in clause 5.1. However it can be assumed that the value used per gNB can also be determined via measurement or other means and configured in OAM or in EIF.

In case of UPF, the same approach can be used, but how to derive the dynamic and static energy consumption contribution can also be determined via measurement or other means and configured in OAM or in EIF.

At this stage let consider some numerical examples for 3 generic NFs, NF A, NF B and NF C which has the EC characteristics shown in table 5.x.x-1 and in figure 5.x.x-1. Now let assume to have 2 UE, where UE1 transfer a data volume of 150 Mbit and UE2 500 Mbit. The EC resulting based on above equation is shown in the table 5.x.x.-2 considering when the NF has a load of 10% and a load of 90%.  The results shows that the proposed approach of considering only the dynamic part of the energy consumption the load does not influence the evaluation that is only proportional to the user data transmitted.

It shall be noted that it is assume that the energy consumption is proportional to the load which is good approximation of real case.

The table shows the amount of Energy Consumption corresponding to the Passive Energy Consumption which remains to be allocated bythe operator based on its strategy, such as the operator may decide to not allocate any amount to the UE traffic and consider it as a consumption due to network functioning, or to allocate a fixed amount to all UE as a “fixed functioning fees” (for example a 5%) due to have the network running, or any other criterias. The operator may decide to have different criteria for different NF type (UPF and gNB), based on location, based on energy source or whatever is considered suitable and consistent with the operator policy and strategy.

Table 6.x-1: Energy consumption characteristics

|  | ECmax [kJ] | PenC [kJ] | DV max [Gb] |  | EC1 at 10% load | DenC at 10% | EC2 at 90%load | DenC at 90% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NF A | 720 | 180 | 10 |  | 1 Gb 234 kJ | 54 kJ | 9 Gb 666 kJ | 486 kJ |
| NF B | 1440 | 180 | 10 |  | 1 Gb 306 kJ | 126 kJ | 9 Gb 1314 kJ | 1134 kJ |
| NF C | 1440 | 72 | 15 |  | 1.5 Gb 209 kJ | 136.8 kJ | 13.5 Gb 1303 kJ | 1231.2 kJ |

Figure 6.x-1 Energy consumption characteristics

Table 6.x-2: Energy consumption contribution for UE1 and UE2

|  | EC for UE at 10% load of NF |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rel-19 |  | Dynamic EC portion |  | PenC to be allocated according to operator policy |  |
|  | UE1 | UE2 | UE1 | UE2 | UE1 | UE2 |
| NF A | 35 | 117 | 8.1 | 27 | 180 | 180 |
| NF B | 45.9 | 153 | 18.9 | 63 | 180 | 180 |
| NF C | 20.88 | 69.6 | 13.68 | 45.6 | 72 | 72 |
|  | EC for UE at 90% load of NF |  |  |  |  |  |
|  | Rel-19 |  | Dynamic EC portion |  | PenC to be allocated according to operator policy |  |
|  | UE1 | UE2 | UE1 | UE2 | UE1 | UE2 |
| NF A | 11.1 | 37 | 8.1 | 27 | 180 | 180 |
| NF B | 21.9 | 73 | 18.9 | 63 | 180 | 180 |
| NF C | 14.48 | 48.27 | 13.68 | 45.6 | 72 | 72 |

### 6.X.3	Considerations on usage

The introduction of the evaluation based on the Passive and Dynamic Energy Consumption allows to split the effect of the data volume consumed by a UE without the impact of the different load condition at which the system work when the Energy Consumption is evaluated. The advantage is the capability to measure the contribution of traffic consumed by the user and to have the Energy Consumption which is comparable for any load.

The Passive Energy consumption contribution allocation to the user is left to the operator who has the flexibility to modulate where and how to use such value. Some example of use cases are:

-	for charging purpose the energy consumption considered can be the dynamic energy consumption and only a portion of the passive energy consumption

-	for QoS only the Dynamic part can be considered, since the change of QoS impacts, eg. change of bit rate, impact directly the Dynamic Energy Consumption contribution, while  the Passive Energy consumption remains unchanged

-	for NF selection the Passive energy Consumption and the load or other parameters may be more important, since to select a node with high Passive Energy Consumption in low load is probably not the preferable choice.

### 6.X.4	Impact

In order to use such improved evaluation impacts are the following:

-	OAM impact

-	provide to the EIF per each NF  the EC per NF and one between the DenC or the PenC. The PenC is invariant over the load and the time period and it is a characteristic of the NF instance and of the hardware, therefore at each measurement interval time both based can be passed by OAM to EIF or the PeNC can be passed once a time by OAM to EIF and only provided an update in case of NF resource modifications.

-	EIF impact

-	support the new formulas

-	support the 2 values passed by OAM, i.e. the EC per NF and one between the DenC or the PenC

# * * * * 3rd change * * * *

### 6.W	Sol #w: Energy Consumption contribution of Control Plane and other contributions

This solution proposes to define a method to consider the contribution of Control Plane and other contribution to the energy consumption.

### 6.X.1	Description

In Rel-19 there are several proposals for defining formulas to evaluate the EC of other CN NFs, such as SMF, AMF, PCF, UDM, etc, etc where a variety of possibility have been proposed

The Energy consumption for mobile network from literature is mention that RAN contribution is around 50-70% of overall energy consumption, then the rest if due to data centers, where data center may be a wide concept including several nodes.

There are elements that in any case can not be considered since they are outside the 3GPP scope, such as the contribution of transport network, the contribution of air condition, where from literature may represent a 10-20% of the overall RAN Energy Consumption

Taking into account the above elements and to avoid to add to much complexity it is proposed:

-	To add a constant factor to the Rel-19 formula to enable the operator to add other contributions to EC estimation as below

EUE=gNBi∈gNBUETEUE, gNBi+ UPFi∈UPFUETEUE, UPFi+KUE,EC	(1)

ESession=gNBi∈gNBSessionTESession, gNBi+ UPFi∈UPFSessionTESession, UPFi+KSession,EC	(2)

EFlow=gNBi∈gNBFlowTEFlow, gNBi+ UPFi∈UPFFlowTEFlow, UPFi+Kflow,EC	(3)

-	The definition of these factors KUE,EC, KSession,EC, Kflow,EC   is left to the operator implementation and decision, for example it can be a value proportional to the DV or to UE or to whatever parameter the operator considers suitable for its scope. . It is not precluded to have a single value for all scenario or to have different values for different UPF, or gNB in different areas.

The value may also be different in different area or per gNb ID and UPF ID for considering different local condition.

The constant values can be configured by operator in EIF based on its strategy and decision

In case of evaluation on a partial measurement interval T, the constant value K can be scaled proportionally to the effective measurement interval duration.

This approach can also be considered in the solution #x.

### 6.W.2	Considerations on usage

The introduction the constant value improve the estimation of energy consumption not limited to the user plane in a simpler way. From this point of view the drawback of the Rel-19 are not addressed. However if this approach is used in conjunction with the usage of Passive and Dynamic Energy Consumption described in solution #x the improvement is significant and the precision of the estimation is improved for all the purposed described in clause 6.x.3.

### 6.W.3	Impact

In order to use such improved evaluation impacts are the following:

-	OAM impact

-	optionally provide to the EIF the constant values KUE,EC, KSession,EC, Kflow,EC

-	EIF impact

-	support the new formulas

-	optionally receive from OAM or have preconfigured the constant values KUE,EC, KSession,EC, Kflow,EC

# * * * * End of changes * * * *