3GPP TSG-SA2 Meeting #168	S2-2503641

07 - 11 April 2025, Goteborg, SE	(revision of S2-25xxxxx)

Source:	CATT

Title:	23.700-14: New KI for FS_Sensing_ARC WT#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC

Abstract of the contribution: This paper is to propose new key issue for FS_Sensing_ARC.

1. Proposal

It is proposed to agree the new key issue into TR 23.700-14 based on the following WT#4 of FS_Sensing_ARC.

- WT-4: Sensing data and the associated information collection and transport mechanisms for result calculation.

* First change *

# 5	Key Issues

## 5.x	Key Issue #x: Sensing Data and Information Collection and Transmission

### 5.X.1	Description

Based on TS 22.137[x], several kind of data and information is specified to support sensing, as follows:

-	3GPP sensing data: data derived from 3GPP radio signals impacted (e.g., reflected, refracted, diffracted) by an object or environment of interest for sensing purposes, and optionally processed within the 5G system.

-	Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data.

NOTE 1:	Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc.

-	Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data.

NOTE 2:	Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

-	Sensing result: processed 3GPP sensing data requested by a service consumer.

Based on the definition above, the 3GPP sensing data is generated and used inside of 5G system, the sensing contextual information and sensing result is generated in 5G system and is exposed to the third party. However the sensing assistance information is provided from a trusted third-party to 5G system. Furthermore, the usage of different data and information is also different. Considering the generation and usage of different data and information is different, the collection and transmission mechanism is also different.

In order to support sensing data and information collection and transmission, the following aspects need to be addressed for this key issue:

-	For different kind of sensing data and information, which NF is responsible to generate and provide the information? Which NF is responsible to collect the information and how?

-	How to transfer the different kind of sensing data and information, including:

-	How to transfer the sensing data and information via control plane?

-	Whether user plane can be used to transfer the sensing data and information, if yes, how and the coordination mechanism with control plane?

* End of change *