3GPP TSG-SA2 Meeting #168	S2-2503643

# 07 – 11 April, 2025, Goteborg, SE                  			             			(revision of S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0198 | rev |  | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Altitude reporting cancellation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-21 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UAS NF/NEF can trigger the altitude reporting, but it is not clear how the UAS NF/NEF cancels the reporting, especially for the aerial UE(s) which has reached the destination of the flight path or the aerial UE(s) flying out of the service area of the UAS NF/NEF. To solve the issue, a new procedure on altitude reporting cancellation is added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new procedure on altitude reporting cancellation to support the UAS NF/NEF cancels the ongoing altitude reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3.1, 4.4.1.1.3.3, 4.4.1.1.3.5, 5.16.1, 5.16.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

4.4.1.1.3.1	General

Service Description: This service enables the consumer to request assistance for USS changeover if/when a UAV/UE moves from a geographical area served by one USS to an area served by another USS. For this purpose, a USS requests a core network to provide the assistance with the USS changeover, to inform the core network about the flight trajectory assigned and time schedule for the UAV/UE and to receive notifications when the UAV/UE moves out/in the geographical area served by a USS triggering and completing the changeover. This service also enables the UAS NF/NEF to trigger the UAV's altitude reporting, altitude reporting cancellation and receive the UAV's altitude from USS.

* * * Second Change * * *

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a next point in the flight path (even when the next point is located in geographical area served by different USS) or providing a NEF/UAS NF information about the in-flight monitoring of a target UAV. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF determines that UAV will leave a geographical area served by the USS, (2) the UAV enters a TA/Cell with a border-crossing point to another USS, (3) the UAV deviates from the assigned flight plan and (x) the altitude reporting cancellation.

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the request (i.e. USS changeover, in-flight monitoring).

Input, Optional: Flight path information, which may include planned flight path information for each of the segment of the paths served by different USS, e.g. a list of TA, UAV's speed, candidate border-crossing point(s), flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells).

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

* * * Third Change * * *

4.4.1.1.3.5	Nnef_UAVFlightAssistance_Notify service operation

Service operation name: Nnef_UAVFlightAssistance_Notify

Description: Notifying a USS about UAV leaving its serviced area and which border crossing point will be used for this purpose, informing a target USS about the UAV entering its serviced area and confirming the completion of the changeover, notifying the USS about the information related to altitude reporting (e.g. the altitude thresholds and information about the required periodicity of the reporting) and altitude reporting cancellation, notifying a USS about the monitoring results for the UAV.

Input, Required: Flight plan configuration ID, status of the UAV according to the assigned flight path (leaving, entering, deviating).

Input, Optional: Information about border-crossing point that will be used for USS changeover, altitude thresholds, indication about the required periodicity of the reporting, indication about altitude reporting cancellation.

Output, Required: Indication whether the USS changeover is completed successfully when used by the target USS when responding to the notification.

Output, Optional: None.

* * * Fourth Change * * *

5.16.1	General

In order to assist a USS/UTM with aerial UE's flight planning and monitoring, the 5GS may enable configuration of aerial UEs (UAVs) to perform altitude reporting. This may include providing aerial UEs with threshold values for altitude reporting and/or request reporting periodicity (e.g. threshold-based, event-based, periodic). For that, the 5GS may determine altitude thresholds and periodicity with which an aerial UE should report its altitude. Altitude thresholds can be derived, for instance, based on the flight path information provided by the serving USS/UTM during a pre-flight planning request or a USS changeover request. Delivery of the altitude thresholds and reporting periodicity can be done via application layer or via node-level signalling as specified in clause 5.16.2. The 5GS should forward the altitude measurements results to the AMF and NEF/UAS NF in the case of node-level signalling. When the aerial UE's flight altitude meets the altitude reporting conditions, the aerial UE's altitude information is reported as specified in clause 5.16.3. Furthermore, the 5GS can cancel the aerial UE’s altitude reporting as specified in clause 5.16.x.

* * * Fifth Change * * *

5.16.x	Altitude reporting cancellation

For the aerial UEs (UAVs) with ongoing altitude reporting as specified in clause 5.16.3, the procedure shown in Figure 5.16.x-1 specifies how the 5GS instructs the aerial UEs (UAVs) to stop the altitude reporting, considering the completion of 5GC assistance with UAV’s flight.

Figure 5.16.x-1: Procedure for altitude reporting cancellation

1.	The aerial UE reports the altitude as specified in clause 5.16.3.

2.	When the procedure (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2) is finished or the aerial UE reaches the destination of the flight path, the UAS NF/NEF determines to cancel the altitude reporting.

Option A: Application layer.

3a.	The UAS NF/NEF invokes Nnef_UAVFlightAssistance_Notify to inform the USS/UTM to cancel the altitude reporting.

4a.	The USS/UTM sends the altitude reporting cancellation indication to the UAV over the application layer.

NOTE X:	Application layer reporting itself is outside of 3GPP scope.

Option B: Node-level signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF sends an Namf_EventExposure_UnSubscribe request to the AMF to unsubscribe the altitude reporting.

4b.	The AMF sends the corresponding N2 messages to NG-RAN node.

5b.	The NG-RAN node instructs the UE to cancel altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE Y:	Aspects related to AMF informing NG-RAN regarding instructing UEs to cancel altitude reporting is specified in TS 38.413 [24].

* * * End of Changes * * *