3GPP TSG-WG SA2 Meeting #168	S2-2503657

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[AA] Architecture assumptions

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Architecture Assumptions for 5GSAT_Ph4-ARC based on those used in phase 3.

# 1. Introduction/Discussion

The following architecture assumptions and principles are based on the ones used during the 5GSAT_Ph3-ARC study, and as the scope of the work in Rel-20 for many WTs in the SID is the same, many of the previous assumptions remain true for this phase 4 study.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (all new) * * * *

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1], TS 23.401 [5] and TS 23.501 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Serving satellite: a satellite providing the satellite access to a UE (e.g. providing the serving cell(s)). Depending on the orbit, the serving satellite is covering a given geographic area for a limited period of time.

UE-Satellite-UE Communication: refers to a communication between UEs under the coverage of one or more serving satellites, using satellite access without the user traffic transiting through the ground segment.

NOTE:	These definitions are different from those in TS 22.261 [6].

NTN Gateway: an earth station located at the surface of the earth, providing connectivity to the NTN payload using the feeder link. An NTN Gateway is a TNL node.

# * * * * Second change (all new) * * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ISL	Inter-Satellite Link

NTN	Non-Terrestrial Network

# * * * * Third change (all new) * * * *

# 4	Architectural Assumptions and Principles

The following architecture assumptions are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [2] is used as a baseline.

-	The EPC architecture for satellite access for NB-IoT as defined in TS 23.401 [5] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [7] are used as a baseline. WB-E-UTRAN and LTE-M access types is not considered in this study.

-	Mobility enhancements for NB-IoT are not in scope and are not considered in this study.

-	Impacts to UE, network functions and entities are minimised. To the extent possible, existing procedures and functionality is reused.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	For IMS voice call over GEO via NB-IoT NTN connecting to EPC only PDN types IPv4, IPv6 and IPv4v6 are supported via the SGi interface.

-

-	NR UE-Satellite-UE communication assumes a connection to the ground is always available.

-	NR UE-Satellite-UE communication assumes roaming for only IMS services including MMTEL and mission critical communications are considered.

-	NR UE-Satellite-UE communication roaming is restricted to two UEs served by same VPLMN and HPLMN.

-	The enhancement for NR UE-Satellite-UE communication for IMS assumes at least a gNB and the set of core network entities, as defined in Rel-19, is present onboard of the satellites.

-	The enhancement for NR UE-Satellite-UE communication for non-IMS assumes at least a gNB and a UPF is present onboard of the satellite.

-	NR UE-Satellite-UE communication assumes that the serving satellite will be of MEO or of LEO type.

-	NR UE-Satellite-UE communication for service assumes communication between two UEs under the coverage of the same satellite or different satellites.

NOTE:	The study of the scenario when two UEs are under the coverage of the same satellite will be prioritized.

# * * * * End of changes * * * *