3GPP TSG-WG SA2 Meeting #168	S2-2503658

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT1] WT1 Key Issues

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: Key Issues relates to WT1 in FS_5GSAT_Ph4-ARC SID

# 1. Introduction/Discussion

The FS_5GSAT_Ph4-ARC SID in SP-250400 has the following WTs:

The study items below aim at investigating on further 3GPP core network functions enhancements to support satellite access with the following work tasks: - WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC NOTE 1: both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope. NOTE 2: Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered. - WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite, - WT-1.2 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements. - WT-1.3 Support of IMS emergency call via NB-IoT NTN. - WT-1.4 How to obtain UE Geo-location for IMS emergency call and other regulatory services. NOTE 3: Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U). NOTE 4: There will be NO mobility enhancement developed for the NB-IoT NTN in this WT. NOTE 5: Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item. - WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available. - WT-2.1 Support of UE-SAT-UE communication for roaming case considering the requirements for lawful interception. NOTE 6: The scenario will be restricted to two roaming UEs served by same VPLMN and HPLMN. NOTE 7: GSMA should be consulted with regard to the roaming requirement for UE-satellite-UE communication, and then further clarification may be needed to specify the detailed roaming scenario.. - WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type. NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked. NOTE 9: No UE impact is expected for WT-2.

The following Key Issues are proposed based on the sub-work tasks for WT1, keeping together aspects which will likely have solutions are address all the parts and separating out aspects which will likely have the possibility of distinct and stand-alone solutions.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change (all new) * * * *

## 5.a	Key Issue #x: Supporting of IMS voice call over GEO NB-IoT NTN

### 5.a.1	Description

EPS system aspects and to support satellite access to provide mechanisms to optimize IMS voice via NB-IoT NTN connected to EPC is studied. The aspects to be studied include:

-	System impacts for supporting IMS voice call over GEO via NB-IoT NTN connecting to EPC.

- 	Whether and how to support QoS for IMS voice service over GEO satellite.

NOTE 1:	Both CP and UP based NB IoT NTN solutions will be studied.

NOTE 2:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

# * * * * Second change (all new) * * * *

## 5.b	Key Issue #x: IMS enhancement to utilize NB IoT as IP-CAN

### 5.b.1	Description

Satellite access to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size via NB-IoT NTN connected to EPC is studied. The aspects to be studied include:

-	Whether and how to define IMS enhancement to utilize NB IoT as IP-CAN

-	Whether and how to optimization IMS signalling to comply with the relevant SA1 requirements.

# * * * * Third change (all new) * * * *

## 5.c	Key Issue #x: Support of IMS emergency call via NB-IoT NTN

### 5.c.1	Description

Emergency calls via satellite access via NB-IoT NTN connected to EPC is studied. The aspects to be studied include:

-	Support of existing IMS emergency call architecture aspects and identify any missing functionality for via NB-IoT NTN.

-	Study whether and how to support any gaps identified.

# * * * * Fourth change (all new) * * * *

## 5.d	Key Issue #x: Obtaining UE Geo-location for IMS emergency call and other regulatory services

### 5.d.1	Description

As part of an emergency calls via satellite access via NB-IoT NTN connected to EPC, the UEs location may need to be obtained and how this is done is to be studied. The aspects to be studied include:

-	How to obtain UE Geo-location for IMS emergency call and other regulatory services.

# * * * * End of changes * * * *