3GPP TSG-SA2 Meeting #168	S2-2503661

Goteborg, Sweden, April 07 – 11, 2025

Source:	Nokia

Title:	Discussion paper for information on Loop prevention and Multi Domain Support in the study on Industrial Networks Enhancements

Document for:	Information

Agenda Item:	30.6

Work Item / Release:	Release 20

Abstract of the contribution: This paper explains the need of support at 5GS to support industrial ethernet networks in context to brownfield deployments, to support loop prevention and multi domains.

# 1.	Discussion

## 1.1	Overview

In industrial network, most of the communication takes place through the Ethernet based networks. These networks have been deployed over time as factories/industries evolved. To provide wireless communication in such a deployment, 5G systems need to be integrated transparently as a logical bridge as introduced in R16. Industrial ethernet networks are already available and deployed and 5GS must adapt to such industrial ethernet networks, needing to support such brownfield integrations. In such networks, loops may occur when two or more UEs are served by different 5G logical bridges and are connected to each other through a wired network.

The above figure shows such an example with three potential loops when two logical 5GS bridges are integrated in an Ethernet Network.

Additionally, frames that are received from a different configuration domain may use priority values that are reserved for TSN streams. To avoid interference with the traffic (especially TSN streams) in the configuration domain such priorities need to be downgraded as described in IEEE Std 802.1Q-2022, Annex U.2.

## 1.2	Motivation

IEEE 802.1 group already informed SA2 group that loop prevention according to IEEE Std 802.1Q-2018 is missing in the TS 23.501 when 5GS acts as a IEEE 802.1 bridge. Meanwhile the original STP as defined in IEEE Std 802.1D-2004 was replaced with the successor Rapid Spanning Tree Algorithm and Protocol (RSTP) and for Industrial Automation (IA) the TSN working group recommends the support of the Multiple Spanning Tree Algorithm and Protocol (MSTP) according to IEC/IEEE  60802, clauses 5.7.1 and 6.4.2.4, which extends RSTP to derive the Common and Internal Spanning Tree (CIST).

In 2023 and 2024 5G-ACIA investigated potential gaps of 5GS when it acts as one or more IEEE 802.1Q bridges in an Industrial Automation network and identified the missing MSTP functionality as a critical issue, especially for non-stream IA traffic.

Further, SA1 already specified in TS 22.261 Rel-16 the requirement to support STP. This requirement was modified in Rel-17 to support RSTP, and in Rel-19 to support MSTP as defined in IEC/IEEE 60802.

Hence it is proposed to enhance 5GS with the support of the spanning tree algorithms and protocols RSTP and MSTP. The support is required to provide the loop prevention functionality as defined in IEEE Std 802.1Q-2022 when 5GS is integrated in Ethernet network(s).

Further on the aspect of multi domain, the configuration of the priority regeneration is described in IEEE Std 802.1Q Annex U.2 step 7, the configuration of the Traffic Class Table (part of the PMIC to DS-TT and NW-TT)) is not shown.  In this table the CNC assigns the PCPs to the Traffic Queues of the port. IEC/IEEE 50802 clause 4.7.4.9 describes an example of Traffic class to traffic type mapping. The definition of which PCP is used for streams may differ in each configuration domain. For example, in conf domain 1 PCP 4 is defined for isochronous streams while in config domain 2 no streams are required, and PCP 4 is used for communicating critical alarms. If such an alarm in config domain 2 is transferred to another config domain via config domain 1 it violates the rule that PCP 4 is reserved for streams. To avoid such situations the CNC will simply use the priority regeneration at ports that connects to foreign domains to map PCP values that are reserved for streams in config domain 1 to a non-stream PCP value. If CNC will not do this, such frames may be added to the traffic queue of bridge port and the deterministic behavior is no longer guaranteed. Important aspect here is that CNC does not know of traffic from a foreign domain.

# 2	Way forward

Proposal 1:

Have a work task in the SID in the lines of “Study how to provide loop prevention functionality (i.e., MSTP/RSTP defined in IEC/IEEE 60802) when the 5G system is integrated in industrial Ethernet network as a bridge.”

Proposal 2:

Have a work task in the SID as “Study whether and how to implement different configuration domains to perform the priority regeneration according to IEEE Std 802.1Q-2022.”

# 3.	References

[1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[2]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”.

[3]	LS from 5G-ACIA “5G-ACIA-LS-2024-008” to SA and SA2.

[4]	LS Response from IEEE 802.1 group https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_139e_Electronic/Docs/S2-2004336.zip