SA WG2 Meeting #168	S2-2503683

07 - 11 April, 2025, Goteborg, Sweden                  	(revision of S2-2501950)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1557 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on DC Application Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paper is proposed to add the function description of DC Application Server and the related abbreviation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the function description of DC Application Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Function description of DC Application Server is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.3, AC.2.2.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

3.1	Definitions

Refer to TS 23.002 [1] for the definitions of some terms used in this document.

For the purposes of the present document the terms and definitions given in TR 21.905 [68] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [68].

For the purposes of the present document, the following terms and definitions given in TS 23.003 [24] apply:

-	Distinct Public Service Identity

-	Public User Identity

-	Wildcarded Public User Identity

-	Wildcarded Service User Identity

Alias Public User Identities: A set of Public User Identities that belong to the same alias group as specified in TS 29.228 [30].

ALG: Application Level Gateway (ALG) is an application specific functional entity that allows communication between disparate address realm or IP versions, e.g. an IPv6 node to communicate with an IPv4 node and vice versa, when certain applications carry network addresses in the payloads like SIP/SDP. NA(P)T-PT or NA(P)T is application unaware whereas ALGs are application specific translation entities that allow a host running an application to communicate transparently with another host running the same application but in a different IP version or IP address realm. See IETF RFC 2663 [34] for more details.

For IMS, an IMS ALG provides the necessary application function for SIP/SDP protocols in order to communicate between different address realms or IP versions, e.g. IPv6 and IPv4 SIP applications.

Application data channel: A data channel within an IMS session used to transfer data of data channel applications between UEs or between the UE and the network, and optionally supports data channel interworking with MTSI UE.

AR Application Server: An application server used to control the service logic related to AR communication via IMS data channel.

Bootstrap data channel: A data channel established within an IMS session between the UE and the network, to transfer a graphical user interface that can include a list of data channel applications.

Business trunking: as defined in TS 24.525 [81].

Data channel application: A HTML page including JavaScript(s) and optionally image(s) and style sheet(s). It is downloaded from the network to the UE through the bootstrap data channel.

DC Application Server: An application server that interacts with the DCSF and the DC media function for data channel traffic handling.

Distinct Public User Identity: used in relation to wildcarded Public User/Service Identities to denote an explicitly provisioned Public User/Service Identity. See more details in TS 23.003 [24].

Entry point: In the case that border control concepts are to be applied in an IM CN subsystem, then these are to be provided by capabilities within the IBCF and the IBCF acts as an entry point for this network (instead of the I-CSCF). In this case the IBCF and the I-CSCF can be co-located as a single physical node. If border control concepts are not applied, then the I-CSCF is considered as an entry point of a network. If the P-CSCF is in the home network, then the I-CSCF is considered as an entry point for this document.

Exit point: If operator preference requires the application of border control concepts then these are to be provided by capabilities within the IBCF and requests sent towards another network are routed via a local network exit point (IBCF), which will then forward the request to the other network (discovering the entry point if necessary).

Geographical Identifier: A Geographical Identifier identifies a geographical area within a country or territory. See more details in clause E.8.

Geo-local service number: A local service number that is used to access a service in the roamed network (a local service where the subscriber is located).

Home local service number: A local service number is used to access a service that is located in the home network of the user.

HSS Group ID: This refers to one or more SBI capable HSS instances managing a specific set of IMPIs/IMPUs.

IMC: IMS Credentials as defined in TR 21.905 [68].

IMS application reference: An IMS application reference is the means by which an IMS communication service identifies an IMS application.

IMS application: An IMS application is an application that uses an IMS communication service(s) in order to provide a specific service to the end-user. An IMS application utilises the IMS communication service(s) as they are specified without extending the definition of the IMS communication service(s).

IMS communication service identifier: An IMS communication service identifier uniquely identifies the IMS communication service associated with the particular IMS request.

IMS communication service: An IMS communication service is a type of communication defined by a service definition that specifies the rules and procedures and allowed medias for a specific type of communication and that utilises the IMS enablers.

IMS enabler: An IMS enabler is a set of IMS procedures that fulfils specific function. An IMS enabler may be used in conjunction with other IMS enablers in order to provide an IMS communication service.

Instance identifier: An identifier, that uniquely identifies a specific UE amongst all other UEs registered with the same Public User Identity.

Inter-IMS Network to Network Interface: The interface which is used to interconnect two IM CN subsystem networks. This interface is not constrained to a single protocol.

IP Flow: Unidirectional flow of IP packets with the following properties:

-	same destination IP address and port number;

-	same source IP address and port number;

-	same transport protocol (port numbers are only applicable if used by the transport protocol).

IP-Connectivity Access Network: refers to the collection of network entities and interfaces that provides the underlying IP transport connectivity between the UE and the IMS entities. An example of an "IP-Connectivity Access Network" is GPRS.

IP SM GW (IP short message gateway): An IP SM GW is an AS providing the support of Short Message Service of the IMS domain. See more details in TS 23.204 [56].

Local Service Number: A local service number is a telephone number in non-international format. A local service number is used to access a service that may be located in the home network of the user (home local service number) or the roamed network of the user (geo-local service number).

Media Flow: One or more IP flows carrying a single media instance, e.g. an audio stream or a video stream. In the context of this specification the term Media Flow is used instead of IP Flow regardless of whether the actual IP packet corresponds to media plane information (e.g. audio RTP flow) or control signalling (e.g. RTCP or SIP Signalling).

MPS: Based on TS 22.153 [77]. Multimedia Priority Service allows authorized users to obtain and maintain radio and network resources with priority, also during national security or emergency situations when PLMN congestion may occur.

MPS session: A session (e.g. voice, video, data session) for which priority treatment is applied for allocating and maintaining radio and network resources.

MPS for Messaging: Within the scope of this specification, IMS Immediate Messaging and IMS Session-based Messaging delivered with MPS priority.

MPS-subscribed UE: A UE having a USIM with MPS indication set and having an MPS subscription in the HPLMN.

NAT-PT/NAPT-PT: NAT-PT uses a pool of globally unique IPv4 addresses for assignment to IPv6 nodes on a dynamic basis as sessions are initiated across the IP version boundaries. NAT-PT binds addresses in IPv6 network with addresses in IPv4 network and vice versa to provide transparent routing between the two IP domains without requiring any changes to end points, like the UE. NAT-PT needs to track the sessions it supports and mandates that inbound and outbound data for a specific session traverse the same NAT-PT router.

NAPT-PT provides additional translation of transport identifier (e.g. TCP and UDP port numbers, ICMP query identifiers). This allows the transport identifiers of a number of IPv6 hosts to be multiplexed into the transport identifiers of a single assigned IPv4 address. See IETF RFC 2766 [33] for more details.

Network Address Translation (NA(P)T): method by which IP addresses are mapped from one group to another, transparently to end users. Network Address Port Translation, or NA(P)T is a method by which many network addresses and their TCP/UDP (Transmission Control Protocol/User Datagram Protocol) ports are translated into a single network address and its TCP/UDP ports. See RFC 3022 [65] for further details.

Outbound: Managing Client Initiated Connections in the Session Initiation Protocol (Outbound) defines behaviours for User Agents, registrars and proxy servers that allow requests to be delivered on existing connections established by the User Agent. See RFC 5626 [48] for further details.

Preferred Circuit Carrier Access: An IMS service that allows a specific long distance circuit carrier to be selected for a long distance call.

Preferred Circuit Carrier Selection: An IMS service that allows the subscriber to select a long distance circuit carrier per call when dialling a call origination.

Rich Call Data (RCD) server: It refers to a server in a third party network storing IMS subscriber specific RCD information.

Rich Call Data (RCD) server address: It refers to the address of an RCD server.

Rich Call Data (RCD) information: It refers to a collection of properties associated with an IMS subscriber. Such properties can be caller name, Email address, telephone number, job title and characteristics of an organization. Details of the properties in the context of RCD can be found in draft-ietf-sipcore-callinfo-rcd-12 [107].

Rich Call Data (RCD) URL: It refers to the URL from where RCD information of a specific IMS subscriber stored at an RCD server can be retrieved from.

Rich Call Data (RCD) properties: It refers to the RCD related parameters (e.g. RCD server address, RCD URL, RCD information) stored in HSS.

Service User: According to TS 22.153 [77].

Stand-alone Non-Public Network: A non-public network not relying on network functions provided by a PLMN.

STUN: Simple Traversal of UDP Through NAT (STUN), provides a toolkit of functions. These functions allow entities behind a NAT to learn the address bindings allocated by the NAT, to keep those bindings open and communicate with other STUN-aware devices to validate connectivity. See RFC 5389 [47] for further details.

STUN Keep-alive: Is a usage of STUN, to keep NAT bindings open.

STUN Relay: Is a usage of STUN, that allows a client to request an address on the STUN server itself, so that the STUN server acts as a relay. See IETF RFC 5766 [46] for further details.

Subscriber: A Subscriber is an entity (comprising one or more users) that is engaged in a Subscription with a service provider. The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of users authorized to enjoy these services and also to set the limits relative to the use that users make of these services.

Transport address: A unique identifier of transport-layer address, i.e. a combination of a network address, protocol identifier and port number. For example, an IP address and a UDP port.

Standalone IMS Data Channel Session: An IMS session with only data channel media component(s) defined in TS 26.114 [76] without accompanying audio/video/messaging media.

NEXT CHANGE

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [68] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [68].

5GS	5G System

API	Application Program Interface

APN	Access Point Name

AS	Application Server

A2P	Application to Person

BCSM	Basic Call State Model

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Bearer Service

CAMEL	Customised Application Mobile Enhanced Logic

CAP	Camel Application Part

CDR	Charging Data Record

CN	Core Network

CS	Circuit Switched

CSCF	Call Session Control Function

CSE	CAMEL Service Environment

DC	Data Channel

DCAR	Data Channel Application Repository

DC AS	Data Channel Application Server

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

DHCP	Dynamic Host Configuration Protocol

DNN	Data Network Name

DNS	Domain Name System

ECN	Explicit Congestion Notification

ENUM	E.164 Number Mapping

GGSN	Gateway GPRS Support Node

GLMS	Group and List Management Server

GMLC	Gateway Mobile Location Centre

GRUU	Globally Routable User Agent URI

GUP	Generic User Profile

HSS	Home Subscriber Server

IBCF	Interconnection Border Control Function

I-CSCF	Interrogating-CSCF

IETF	Internet Engineering Task Force

IM	IP Multimedia

IMC	IMS Credentials

IMS	IP Multimedia Core Network Subsystem

IMS ALG	IMS Application Level Gateway

IMSI	International Mobile Subscriber Identifier

IN	Intelligent Network

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IP-CAN	IP-Connectivity Access Network

IP-SM-GW	IP Short Message Gateway

ISDN	Integrated Services Digital Network

ISIM	IMS SIM

ISP	Internet Service Provider

ISUP	ISDN User Part

IWF	Interworking Function

NP	Number portability

MAP	Mobile Application Part

MGCF	Media Gateway Control Function

MGF	Media Gateway Function

MRB	Media Resource Broker

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

NAI	Network Access Identifier

NAPT	Network Address Port Translation

NAT	Network Address Translation

NA(P)T-PT	Network Address (Port-Multiplexing) Translation-Protocol Translation

II-NNI	Inter-IMS Network to Network Interface

OSA	Open Services Architecture

P2A	Person to Application

P2P	Person to Person

P-CSCF	Proxy-CSCF

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PDN	Packet Data Network

PDP	Packet Data Protocol e.g. IP

P-GRUU	Public Globally Routable User Agent URI

PLMN	Public Land Mobile Network

PSI	Public Service Identity

PSTN	Public Switched Telephone Network

QoS	Quality of Service

RAB	Radio Access Bearer

RCD	Rich Call Data

RFC	Request for Comments

SCS	Service Capability Server

S-CSCF	Serving-CSCF

SDP	Session Description Protocol

SGSN	Serving GPRS Support Node

SLF	Subscription Locator Function

SNPN	Stand-alone Non-Public Network

SSF	Service Switching Function

SS7	Signalling System 7

SIM	Subscriber Identity Module

SIP	Session Initiation Protocol

S-GW	Signalling Gateway

TAS	Telephony Application Server

T-GRUU	Temporary Globally Routable User Agent URI

THIG	Topology Hiding Inter-network Gateway

TrGW	Transition Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

URL	Universal Resource Locator

USIM	UMTS SIM

NEXT CHANGE

AC.2.2.X	Data Channel Application Server (DC AS)

The DC AS is the external function that supports the following functionalities:

-	The DC AS interacts with the DCSF and the MF for data channel traffic handling.

-	The DC AS optionally supports data channel interworking between DCMTSI UE and MTSI UE.

END OF CHANGES