3GPP TSG-SA WG2#168	S2-2503702

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	New Solution for KI#1: UP based UE data collection solution for AI/ML NR air interface operation

Document for:	Discussion/Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2

Abstract of the contribution: This contribution proposes the solution for KI#1 to support UE data Collection for option 2.

# 1.	Discussion

This contribution proposed a new solution based on newly defined entity UDCF in 5GC to support UP based UE data collection for AIML UE side model training.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-04:

# >>>>BEGINNING OF CHANGES (all new text)<<<<

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #y> |
| 6.x | x |  |

## 6.X	Solution #X: Hybrid solution using UDCF to support UP based UE data collection for NR AIML air interface operation

### 6.X.1	Description

#### 6.x.1.1	General description

This solution addresses KI#1.

Based on the LS in RP-24331, SA2 is required to study transfer of data over UP for option 2 and study if the solution fulfil the visibility and controllability requirements from RAN2.

In order to do that, a new NF named UDCF (UE Data Collection Function) is proposed as a new 5GC entity with the following functionality:

- The UP path termination entity in 5GC for UE data transfer from UE

- Receiving the UE data collection request from DCCF

- Requests data collection to UE

- Receiving “raw” data reporting from UE

- Expose UE “raw” data to DCCF or expose the UE “raw” data to OAM depending on the MNO choice for its AIML data management and processing architecture.

The interfaces for UDCF are described in Figure 6.x.1-1.

Figure 6.x.1-1: Interfaces for UDCF

AF is the UE server for UE-side model training.

The same UP procedure about UE data collection between UE and UDCF can be used for both architecture option 2 or option 3. The reference of architecture Options 2 and 3 is referred to the data collection options described in RP-242389.

In Option 2, SA2 will further study the exposure procedure from UDCF, to DCCF using new interface Nu1 and from DCCF to AF. In option 3, the UDCF can expose the collected data to OAM, using Nu2 and SA5 can further study the rest of the functionality for an “OAM centric data collection architecture” and specifically how to expose the UE data to the UE server for UE-side model training.

In both cases it is proposed that the same UDCF NF can be used for both option 2 and option 3 as described in Figure 6.x.1-2.

Figure 6.x.1-2: UDCF based architecture to support option 2 and option 3

#### 6.x.1.2	Solution Principles

The following principles for this solution are proposed as below:

- New UDCF is proposed as a UP path termination entity in 5GC for “raw” UE data transfer

- When the 5GC-centric variant is used:

-	AF requests the data for specific UE from DCCF

-	DCCF check if the requested UE collected data are already stored in ADRF or not. If the requested UE collected data are available in ADRF, DCCF exposes the data from ADRF to AF. If the requested UE collected data are not available in ADRF, DCCF sends data for the specific UE from UDCF.

-	UDCF informs the UDCF address to UE via NAS signalling. UE establishes the UP connection via UPF to UDCF.

-	UDCF sends data collection request to UE via application layer signaling, i.e. HTTP signalling. The security about the application layer signalling between UE and UDCF will be discussed in SA3.

-	UE collects data according to RAN1/RAN2’s decisions and sends the report to UDCF.

Editor’s Note: Details of the data formats and data parameters in the UE-to-UDCF will be align with RAN1/2’s decision.

- 	UDCF sends the received “raw” data to DCCF, DCCF further sends to AF. DCCF may also store the collected UE collected data in ADRF.

- When the OAM-centric variant is used:

-	Details between the UDCF and OAM will be defined by SA5.

### 6.X.2	Procedures

Figure 6.x.2.1-1 describes the procedure about AF subscribes UE collected data via UDCF for the 5GC-centric variant:

Figure 6.x.2-1	AF subscribes UE collected data from UDCF (5GC-centric variant)

- AF sends the Nnef_EventExposure_Subscribe request message to request the UE collected data. This message includes requested UE collected data, Target of reporting (GPSI(s), or any UE), location information, S-NSSAI and reporting time info.

- NEF discovers the DCCF from NRF for UE data collection request.

If AF provides GPSI for specific UE, NEF maps the GPSI to SUPI and discovers the DCCF for the specific UE.

If AF provides any UE, NEF should discover the UE list from AMF based on S-NSSAI location info that provided by AF in step 1.

NEF discovers DCCF by using the requested UE collected data, location information and S-NSSAI.

- DCCF checks user consent about the UE collected data collection.

- If user consent is granted, DCCF determines if any available parameters that stored in ADRF, if yes, DCCF will retrieve data from ADRF and report to AF via NEF. The following steps are skipped.

If there is no available data stored in ADRF, DCCF will discover an UDCF for UE collected data.

UDCF discovery from NRF. UDCF registers the supported UE collected data, S-NSSAI, DNN and location information as NF profile in NRF. DCCF will discover the UDCF for specific UE with these information.

5.	DCCF sends the requested UE collected data, UE ID, reporting time info to the UDCF for each UE.

6.	UDCF invokes the Namf_Communication_N1N2Transfer message to include the UDCF UP address to AMF.

7.	AMF sends the NAS message to UE which includes the UDCF UP address.

8.	UE establishes the UP connection to UDCF by using the UDCF UP address that receives in step 8.

9.	UE sends the NAS response to AMF.

10.	AMF sends response to UDCF.

11.	UDCF sends the data request to UE, UE performs data collection as defined in RAN1 and RAN2, UE reports the collected data to UDCF. The data collection and reporting between UE and UDCF is via a UP based signalling, for example, HTTP. The security about the interaction between UE and UDCF will be determined in SA3.

12.	UDCF reports the received AF subscribes UE collected data to DCCF.

13.	DCCF reports data to NEF. DCCF may further store the UE collected data in ADRF.

14. NEF forwards the data to AF.

Figure 6.x.2.1-2 describes the procedure about AF subscribes UE collected data via UDCF for the OAM-centric variant:

Figure 6.x.2-2	AF subscribes UE collected data from UDCF (OAM-centric variant)

- AF request the UE collected data to OAM. Details of the procedure will be defined by SA5.

2.	OAM sends the requested UE collected data, UE ID, reporting time info to the UDCF for each UE.

3- 8.	Same steps as steps 5-11 in Figure 6.x.2-1.

9.	UDCF reports the received AF subscribed UE collected data to OAM.

10. OAM reports data to AF. Details of the procedure will be defined by SA5.

### 6.X.3	Impacts on services, entities and interfaces

UDCF

- New 5GC entity to support UE “raw” data collection and exposure either to DCCF or OAM

DCCF

- Enhance to support data exposure to AF

- Enhance to support UDCF discovery

UE

- Receiving UDCF UP address via NAS signalling

- Establishing the UP connection to UDCF

- Receiving UE collected data collection request from UDCF via UP signaling

- Reporting UE collected data to UDCF via UP protocol e.g. HTTP

OAM

- Can receive “raw” UE data collected by UDCF. Details on further procedures up to SA5.

# >>>>END OF CHANGES<<<<