3GPP TSG-WG SA2 Meeting #168	S2-2503703

Goteborg, SE, 7 – 11 Apr, 2025	Revision of S2-250XXXX

Source:	China Mobile

Title:	Update for Functional Entities

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT/ Rel-19

Abstract: This contribution is proposed to support different security solutions considering different security capabilities supported by the AIoT devices.

1. Introduction/Discussion

This contribution is proposed to support different security solutions considering the security capabilities supported by an AIoT device.

According to the conclusion from SA3,  the network may provide different network layer security solutions dependent of the exact security capabilities supported by an AIoT device, as depicted in TR 33.713.

The network layer security solution to be specified includes the following security capabilities: authentication (one-way and mutual), protection of the AIoT data (confidentiality, integrity and replay protection) and identifier privacy over the air interface.

NOTE: The exact security capabilities to be supported by a device depend on the functionalities supported by the device. Which device functionality mandates which security capabilities is to be clarified during the normative work. Whether every device supports every possible device functionality or not is out of SA3 scope.

To handle security solutions, it consumes more power and the complexity of the device increases, leading to an increase in the AIoT device cost. To decrease cost, the lowest-end AIoT devices may not have any security capability. Other devices may support one or a combination of the following capabilities: authentication, protection of the AIoT data and identifier privacy over the air interface. In addition, the data security and identifier privacy may be not necessary in some secure cases, e.g. customer’s private network. For a wide area use case, the AIOTF may be configured with different security solutions per AF or per area. For example, no security solution is used in a warehouse, while authentication, data protection and privacy are all used in another warehouse, which depends on the service requirements.

Therefore, it is necessary to consider the security capabilities of an AIoT device and the service requirements to decide whether to enable authentication, data security and privacy. Accordingly, the network shall store the related information, decide the security solution and transport parameters to the AIoT devices, which proposes new requirements for the Functional Entities in 4.5 in TS 23.369.

2. Text Proposal

It is proposed to approve the following changes.

FIRST CHANGE

4.5	Functional Entities

4.5.1	AIoT Device

Editor’s note:	The definition of AIoT Device needs to align with the definition at RAN WGs.

4.5.2	AIoT Readers

Editor’s note:	AIoT RAN and AIoT Reader will be aligned later with RAN WGs. It is FFS whether to specify the relationship between AIoT RAN and AIoT Reader and which term to use in this TS.

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS protocol with AIoT Device.

-	Connectivity with an AIOT RAN via a direct interface reference point or via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the trusted AF's AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information when AIOTF connects the AIoT RAN indirectly via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving AIoT device profile data from ADM.

-	Retrieving AF subscription data from ADM.

-	Optionally AIoT Device context management.

-    	Determine the security solution based on  device’s security capabilities, AF ID and area information, and provide related parameters to the AIoT device.

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs of 3rd party for AIoT services.

-	Interacting with AF of 3rd party and AIOTF.

-	Selection of AIOTF for AIoT services.

-	Authorization of the untrusted AF’s AIoT operation request.

4.5.5	AF

The AF performs the following functions to support AIoT services:

-	Interacting with NEF for AIoT related service exposure for 3rd party .

-	Interacting with AIOTF for AIoT related service exposure for trusted AF.

4.5.6	NRF

In addition to the functions defined in TS 23.501 [3], the NRF performs the following functions:

-	Support of new NF types AIOTF and their corresponding NF profiles. The NF profile includes the AIOTF ID or IP address, NF type and the serving area information of the AIOTF.

-	Support of AIOTF discovery based on parameters such as target area information.

4.5.7	AMF

The AMF performs the following functions when the AIoT RAN and the AIOTF communicate indirectly via an AMF:

-	Relaying signalling for AIoT service between AIoT-RAN and AIOTF.

-	Providing transparent transport for AIoT NAS messages between AIoT Device and AIOTF.

-	Providing transport for AIOT RAN Node’s serving area information between AIOT RAN and AIOTF.

Editor’s note:	Further functions on AMF is FFS.

4.5.8	UDR

In addition to the functions defined in TS 23.501 [3], the UDR may support the following functions:

-	Storage of AIoT device profile data.

4.5.9	ADM

The ADM supports the following functions:

-	Management of AIoT device profile data.

- The profile data includes security capabilities supported by the AIoT device.