3GPP TSG-WG SA2 Meeting #168	S2-2503711

# Goteborg, Sweden, April 07 – 11, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 1142 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on QCI for satellite access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Packet delay budget for satellite access with regenerative payload for QCI value 10 needs to be clarified, similar to NOTE 18 in Table 5.7.4-1 of TS 23.501. Correspondingly, it needs to clarify that NOTE 17 in Table 6.1.7-A only applies to satellite access with transparent payload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarifications on packet delay budget for QCI value 10 considering satellite access with regenerative payload. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Access network packet delay budget for satellite access with regenerative payload may be wrongly calculated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

6.1.7.2	Standardized QCI characteristics

This clause specifies standardized characteristics associated with standardized QCI values. The characteristics describe the packet forwarding treatment that an SDF aggregate receives edge-to-edge between the UE and the PCEF (see figure 6.1.7-1) in terms of the following performance characteristics:

1	Resource Type (GBR or Non-GBR);

2	Priority;

3	Packet Delay Budget;

4	Packet Error Loss Rate;

5	Maximum Data Burst Volume (for some GBR QCIs);

6	Data Rate Averaging Window (for some GBR QCIs).

Figure 6.1.7-1: Scope of the Standardized QCI characteristics for client/server (upper figure) and peer/peer (lower figure) communication

The standardized characteristics are not signalled on any interface. They should be understood as guidelines for the pre-configuration of node specific parameters for each QCI. The goal of standardizing a QCI with corresponding characteristics is to ensure that applications / services mapped to that QCI receive the same minimum level of QoS in multi-vendor network deployments and in case of roaming. A standardized QCI and corresponding characteristics is independent of the UE's current access (3GPP or Non-3GPP).

The one-to-one mapping of standardized QCI values to standardized characteristics is captured in table 6.1.7-A and table 6.1.7-B. The main differences between the two parts are that, in contrast to Part A, Part B of Table 6.1.7 describes QCIs for which the Packet Error Loss Rate calculation includes those packets that are not delivered within the Packet Delay Budget; and, it provides additional information on the Data Rate Averaging Window as well as the Maximum Data Burst Volume that needs to be delivered within the Packet Delay Budget.

Table 6.1.7-A: Standardized QCI characteristics

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE 13) | Packet Error Loss Rate (NOTE 2) | Example Services |
| --- | --- | --- | --- | --- | --- |
| 1 (NOTE 3) |  | 2 | 100 ms (NOTE 1, NOTE 11) | 10-2 | Conversational Voice |
| 2 (NOTE 3) | GBR | 4 | 150 ms (NOTE 1, NOTE 11) | 10-3 | Conversational Video (Live Streaming) |
| 3 (NOTE 3, NOTE 14) |  | 3 | 50 ms (NOTE 1, NOTE 11) | 10-3 | Real Time Gaming, V2X messages Electricity distribution - medium voltage (e.g. clause 7.2.2 of TS 22.261 [51]) Process automation - monitoring (e.g. clause 7.2.2 of TS 22.261 [51]) |
| 4 (NOTE 3) |  | 5 | 300 ms (NOTE 1, NOTE 11) | 10-6 | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 3, NOTE 9, NOTE 12) |  | 0.7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 (NOTE 3, NOTE 12) |  | 2 | 100 ms (NOTE 1, NOTE 10) | 10-2 | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 3, NOTE 12) |  | 1.5 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Mission Critical Video user plane |
| 75 (NOTE 14) |  | 2.5 | 50 ms (NOTE 1) | 10-2 | V2X messages |
| 71 |  | 5.6 | 150ms (NOTE 1, NOTE 16) | 10-6 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 72 |  | 5.6 | 300ms (NOTE 1, NOTE 16) | 10-4 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 73 |  | 5.6 | 300ms (NOTE 1, NOTE 16) | 10-8 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 74 |  | 5.6 | 500ms (NOTE 1, NOTE 16) | 10-8 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 76 |  | 5.6 | 500ms (NOTE 1, NOTE 16) | 10-4 | "Live" Uplink Streaming (e.g. TS 26.238 [53]) |
| 5 (NOTE 3) |  | 1 | 100 ms (NOTE 1, NOTE 10) | 10-6 | IMS Signalling |
| 6 (NOTE 4) |  | 6 | 300 ms (NOTE 1, NOTE 10) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 (NOTE 3) | Non-GBR | 7 | 100 ms (NOTE 1, NOTE 10) | 10-3 | Voice, Video (Live Streaming) Interactive Gaming |
| 8 (NOTE 5) |  | 8 | 300 ms (NOTE 1) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file |
| 9 (NOTE 6) |  | 9 |  |  | sharing, progressive video, etc.) |
| 10 |  | 9 | 1100 ms (NOTE 1, NOTE 17, NOTE X) | 10-6 | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access with these characteristics |
| 69 (NOTE 3, NOTE 9, NOTE 12) |  | 0.5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling, MC Video signalling) |
| 70 (NOTE 4, NOTE 12) |  | 5.5 | 200 ms (NOTE 7, NOTE 10) | 10-6 | Mission Critical Data (e.g. example services are the same as QCI 6/8/9) |
| 79 (NOTE 14) |  | 6.5 | 50 ms (NOTE 1, NOTE 10) | 10-2 | V2X messages |
| 80 (NOTE 3) |  | 6.8 | 10 ms (NOTE 10, NOTE 15) | 10-6 | Low latency eMBB applications (TCP/UDP-based); Augmented Reality |
| NOTE 1: A delay of 20 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. This delay is the average between the case where the PCEF is located "close" to the radio base station (roughly 10 ms) and the case where the PCEF is located "far" from the radio base station, e.g. in case of roaming with home routed traffic (the one-way packet delay between Europe and the US west coast is roughly 50 ms). The average takes into account that roaming is a less typical scenario. It is expected that subtracting this average delay of 20 ms from a given PDB will lead to desired end-to-end performance in most typical cases. Also, note that the PDB defines an upper bound. Actual packet delays - in particular for GBR traffic - should typically be lower than the PDB specified for a QCI as long as the UE has sufficient radio channel quality. NOTE 2: The rate of non congestion related packet losses that may occur between a radio base station and a PCEF should be regarded to be negligible. A PELR value specified for a standardized QCI therefore applies completely to the radio interface between a UE and radio base station. NOTE 3: This QCI is typically associated with an operator controlled service, i.e. a service where the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. In case of E-UTRAN this is the point in time when a corresponding dedicated EPS bearer is established / modified. NOTE 4: If the network supports Multimedia Priority Services (MPS) then this QCI could be used for the prioritization of non real-time data (i.e. most typically TCP-based services/applications) of MPS subscribers. NOTE 5: This QCI could be used for a dedicated "premium bearer" (e.g. associated with premium content) for any subscriber / subscriber group. Also in this case, the SDF aggregate's uplink / downlink packet filters are known at the point in time when the SDF aggregate is authorized. Alternatively, this QCI could be used for the default bearer of a UE/PDN for "premium subscribers". NOTE 6: This QCI is typically used for the default bearer of a UE/PDN for non privileged subscribers. Note that AMBR can be used as a "tool" to provide subscriber differentiation between subscriber groups connected to the same PDN with the same QCI on the default bearer. NOTE 7: For Mission Critical services, it may be assumed that the PCEF is located "close" to the radio base station (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence delay of 10 ms for the delay between a PCEF and a radio base station should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that QCI-65 and QCI-69 are used together to provide Mission Critical Push to Talk service (e.g. QCI-5 is not used for signalling for the bearer that utilizes QCI-65 as user plane bearer). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these QCIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This QCI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this QCI value. NOTE 13: Packet delay budget is not applicable on NB-IoT or when Enhanced Coverage is used for WB-E-UTRAN (see TS 36.300 [19]). NOTE 14: This QCI could be used for transmission of V2X messages as defined in TS 23.285 [48]. NOTE 15: A delay of 2 ms for the delay between a PCEF and a radio base station should be subtracted from the given PDB to derive the packet delay budget that applies to the radio interface. NOTE 16: For "live" uplink streaming (see TS 26.238 [53]), guidelines for PDB values of the different QCIs correspond to the latency configurations defined in TR 26.939 [54]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PELR combinations are needed these configurations will have to use non-standardised QCIs. NOTE 17: When UE is accessing the network via satellite access supporting transparent payload, the worst case one way propagation delay between the UE and gNB for GEO satellite is expected to be ~270 ms, ~ 21 ms for LEO at 1200 km and ~13 ms for LEO at 600 km. The UL scheduling delay that needs to be added is typically a two way propagation delay e.g. ~540 ms for GEO, ~42 ms for LEO at 1200 km and ~26 ms for LEO at 600 km. Based on that, QCIs that require access network PDB lower than the sum of these values are not applicable when the specific types of satellite access are used (see TS 36.300 [19]). QCI value10 can accommodate the worst case access network PDB for GEO satellite type. NOTE X: When UE is accessing the network via satellite access supporting regenerative payload, a static value of ~290 ms for GEO satellite, ~41 ms for LEO at 1200 km and ~33 ms for LEO at 600 km for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. |  |  |  |  |  |

Table 6.1.7-B: Standardized QCI characteristics

| QCI | Resource Type | Priority Level | Packet Delay Budget (NOTE B1) | Packet Error Loss Rate (NOTE B2) | Maximum Data Burst Volume (NOTE B1) | Data Rate Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 82 (NOTE B6) | GBR | 1.9 | 10 ms (NOTE B4) | 10-4 (NOTE B3) | 255 bytes | 2000 ms | Discrete Automation (TS 22.278 [38], clause 8 bullet g and TS 22.261 [51], table 7.2.2-1, "small packets") |
| 83 (NOTE B6) |  | 2.2 | 10 ms (NOTE B4) | 10-4 (NOTE B3) | 1354 bytes (NOTE B5) | 2000 ms | Discrete Automation (TS 22.278 [38], clause 8 bullet g and TS 22.261 [51], table 7.2.2-1, "big packets") |
| 84 (NOTE B6) |  | 2.4 | 30 ms (NOTE B7) | 10-5 (NOTE B3) | 1354 bytes (NOTE B5) | 2000 ms | Intelligent Transport Systems (TS 22.278 [38], clause 8, bullet h and TS 22.261 [51], table 7.2.2). |
| 85 (NOTE B6) |  | 2.1 | 5 ms (NOTE B8) | 10-5 (NOTE B3) | 255 bytes | 2000 ms | Electricity Distribution- high voltage (TS 22.278 [38], clause 8, bullet i and TS 22.261 [51], table 7.2.2 and Annex D, clause D.4.2). |
| NOTE B1: The PDB applies to bursts that are not greater than Maximum Data Burst Volume. NOTE B2: This Packet Error Loss Rate includes packets that are not successfully delivered over the access network plus those packets that comply with the Maximum Data Burst Volume and GBR requirements but which are not delivered within the Packet Delay Budget. NOTE B3: Data rates above the GBR, or, bursts larger than the Maximum Data Burst Volume, are treated as best effort and, in order to serve other packets and meet the PELR, this can lead to them being discarded. NOTE B4: A delay of 1 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE B5: This Maximum Data Burst Volume value is set to 1354 bytes to avoid IP fragmentation on an IPv6 based, IPSec protected GTP tunnel to the eNB (the value is calculated as in Annex C of TS 23.060 [12] and further reduced by 4 bytes to allow for the usage of a GTP-U extension header). NOTE B6: This QCI is typically associated with a dedicated EPS bearer. NOTE B7: A delay of 5 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE B8: A delay of 2 ms for the delay between a PCEF and a radio base station should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. |  |  |  |  |  |  |  |

The Resource Type determines if dedicated network resources related to a service or bearer level Guaranteed Bit Rate (GBR) value are permanently allocated (e.g. by an admission control function in a radio base station). GBR SDF aggregates are therefore typically authorized "on demand" which requires dynamic policy and charging control. A Non GBR SDF aggregate may be pre-authorized through static policy and charging control.

The Maximum Data Burst Volume, if defined for the QCI (see Table 6.1.7-B), is the amount of data which the RAN is expected to deliver within the part of the Packet Delay Budget allocated to the link between the UE and the radio base station as long as the data is within the GBR requirements. If more data is transmitted from the application, delivery within the Packet Delay Budget cannot be guaranteed for packets exceeding the Maximum Data Burst Volume or GBR requirements.

The Data Rate Averaging Window, if defined for the QCI (see Table 6.1.7-B), is the 'sliding window' duration over which the GBR and MBR for a GBR SDF aggregate shall be calculated (e.g. in the RAN, PDN-GW and UE).

The Packet Delay Budget (PDB) defines an upper bound for the time that a packet may be delayed between the UE and the PCEF. For a certain QCI the value of the PDB is the same in uplink and downlink. The purpose of the PDB is to support the configuration of scheduling and link layer functions (e.g. the setting of scheduling priority weights and HARQ target operating points). Except for QCIs 82 and 83, the PDB shall be interpreted as a maximum delay with a confidence level of 98 percent. For services using QCI 82 or 83, a packet delayed by more than the PDB is included in the calculation of the PELR if the packet is within the Maximum Data Burst Volume and GBR requirements.

NOTE 1:	The PDB denotes a "soft upper bound" in the sense that an "expired" packet, e.g. a link layer SDU that has exceeded the PDB, does not need to be discarded (e.g. by RLC in E-UTRAN). The discarding (dropping) of packets is expected to be controlled by a queue management function, e.g. based on pre-configured dropping thresholds.

The support for SRVCC requires QCI=1 only be used for IMS speech sessions in accordance to TS 23.216 [28].

NOTE 2:	Triggering SRVCC will cause service interruption and/or inconsistent service experience when using QCI=1 for non-IMS services.

NOTE 3:	Triggering SRVCC for WebRTC IMS session will cause service interruption and/or inconsistent service experience when using QCI=1. Operator policy (e.g. use of specific AF application identifier) can be used to avoid using QCI 1 for a voice service, e.g. WebRTC IMS session.

Services using a Non-GBR QCI should be prepared to experience congestion related packet drops and, except for QCI 80, 98 percent of the packets that have not been dropped due to congestion should not experience a delay exceeding the QCI's PDB. This may for example occur during traffic load peaks or when the UE becomes coverage limited. See Annex J for details. Packets that have not been dropped due to congestion may still be subject to non congestion related packet losses (see PELR below). Owing to its low latency objective, services using QCI 80 should anticipate that more than 2 percent of packets might exceed the PDB of QCI 80.

Except for services using QCI 82 or 83 services using a GBR QCI and sending at a rate smaller than or equal to GBR can in general assume that congestion related packet drops will not occur and 98 percent of the packets shall not experience a delay exceeding the QCI's PDB. Exceptions (e.g. transient link outages) can always occur in a radio access system which may then lead to congestion related packet drops even for services using a GBR QCI and sending at a rate smaller than or equal to GBR. Packets that have not been dropped due to congestion may still be subject to non congestion related packet losses (see PELR below). For services using QCI 82 or 83 a packet which is delayed by more than the PDB but is within the Maximum Data Burst Volume and GBR requirements, is counted as lost when calculating the PELR.

Every QCI (GBR and Non-GBR) is associated with a Priority level (see Table 6.1.7). The lowest Priority level value corresponds to the highest Priority. The Priority levels shall be used to differentiate between SDF aggregates of the same UE and it shall also be used to differentiate between SDF aggregates from different UEs. Via its QCI an SDF aggregate is associated with a Priority level and a PDB. Scheduling between different SDF aggregates shall primarily be based on the PDB. If the target set by the PDB can no longer be met for one or more SDF aggregate(s) across all UEs that have sufficient radio channel quality then the QCI Priority level shall be used as follows: in this case a scheduler shall meet the PDB of an SDF aggregate on QCI Priority level N in preference to meeting the PDB of SDF aggregates on next QCI Priority level greater than N, until the priority N SDF aggregate's GBR (in case of a GBR SDF aggregate) has been satisfied.

Other aspects related to the treatment of traffic exceeding an SDF aggregate's GBR are out of scope of this specification.

When required by operator policy, the eNodeB can be configured to use the ARP priority level in addition to the QCI priority level to determine the relative priority of the SDFs in meeting the PDB of an SDF aggregate. This configuration applies only for high priority ARPs as defined in clause 6.1.7.3.

NOTE 4:	The definition (or quantification) of "sufficient radio channel quality" is out of the scope of 3GPP specifications.

NOTE 5:	In case of E-UTRAN a QCI's Priority level and when required by operator policy, the ARP priority level may be used as the basis for assigning the uplink priority per Radio Bearer (see TS 36.300 [19] for details).

The Packet Error Loss Rate (PELR) defines an upper bound for the rate of SDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in E-UTRAN) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in E-UTRAN). Thus, the PELR defines an upper bound for a rate of non congestion related packet losses. The purpose of the PELR is to allow for appropriate link layer protocol configurations (e.g. RLC and HARQ in E-UTRAN). For a certain QCI the value of the PELR is the same in uplink and downlink.

NOTE 6:	The characteristics PDB and PELR are specified only based on application / service level requirements, i.e. those characteristics should be regarded as being access agnostic, independent from the roaming scenario (roaming or non-roaming) and independent from operator policies.

* * * End of Changes * * * *