3GPP TSG-SA2 Meeting #168	S2-2503744

07 - 11 April 2025, Goteborg, SE                                       (revision of S2-25xxxxx)

Source:	CATT, vivo

Title:	FS_5GSAT_Ph4_ARC Key issues for WT#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper is to propose new key issue for FS_5GSAT_Ph4_ARC.

1Discussion

The Work Task#1 of the FS_5GSAT_Ph4_ARC SID is:

-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	Both CP and UP based NB-IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

-	WT-1.2 whether and how to define IMS enhancement to utilize NB-IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

-	WT-1.3 Support of IMS emergency call via NB-IoT NTN.

-	WT-1.4 How to obtain UE Geo-location for IMS emergency call and other regulatory services.

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	There will be NO mobility enhancement developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

Based on the WT, we have to study how to support IMS voice call over GEO via NB-IoT NTN connecting to EPC.

2 Proposal

It is proposed to agree the new key issues for WT#1 into TR 23.700-19 on FS_5GSAT_Ph4_ARC as followed.

* First change *

5.X	Key Issue #X: Support of QoS for IMS voice service over GEO satellite

5.X.1	Description

Voice calls using the IMS (IP Multimedia Subsystem) platform can ensure the quality of service (QoS) of voice calls by guaranteeing key factors such as the transmission delay and bandwidth of voice signals within the wireless mobile communication system. Currently, IMS voice operates as VoIP, requiring the establishment of a PDN connection to the IMS APN. This setup involves two DRBs: one for IMS signaling with QCI=5 (non-GBR) and another for IMS voice transmission with QCI=1 (GBR). However, with GEO satellite access, the significantly longer propagation delay of285ms compared to other access technologies impacts key parameters, such as the Packet Delay Budget, which is defined as 100 ms for QCI=1/5 and may need to be revisited.

To support IMS voice call over GEO via NB-IoT NTN connecting to EPC, the following issue is proposed to be studied:

-	Whether and how to support QoS for IMS voice service over GEO satellite.

NOTE 1:	Both CP and UP based NB-IoT NTN solutions will be studied in this KI, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 3:	There will be NO mobility enhancement developed for the NB-IoT NTN in this KI.

5.Y	Key Issue #Y: Support of IMS enhancement to utilize NB- IoT

5.Y.1	Description

To support IMS voice call over GEO via NB-IoT NTN, the existing NB-IoT feature only supports the transmission of small data with high delay requirements, and cannot meet the requirements of low latency and high bandwidth for voice transmission. Besides, as described in TS 22.261 [a], mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size, and to restrict call setup time within 30s needs to be supported.

To support IMS voice call over GEO via NB-IoT NTN connecting to EPC, the following issue is proposed to be studied:

-	Whether and how to define IMS enhancement to utilize NB-IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

NOTE 1:	Both CP and UP based NB-IoT NTN solutions will be studied in this KI, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

NOTE 3:	There will be NO mobility enhancement developed for the NB-IoT NTN in this KI.

NOTE 4:	Depending on operators’ policies, IMS enhancements defined in this KI may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

5.Z	Key Issue #Z: Support of IMS emergency call via NB-IoT NTN

5.Z.1	Description

IMS emergency call is a critical regulatory requirement in IMS service. NB-IoT GEO satellite access currently lacks support for emergency sessions, limiting its applicability in critical communication scenarios. To support IMS emergency call over GEO via NB-IoT NTN connecting to EPC, the following issue is proposed to be studied:

-	How to support IMS emergency call over GEO via NB-IoT NTN.

NOTE 1:	Both CP and UP based NB-IoT NTN solutions will be studied in this KI, checkpoint will be set before the normative work to align with RAN scope.

## 5.W	Key Issue #W: Positioning for IMS emergency call and regulatory services

### 5.W.1	Description

When a UE requests emergency PDN connection for emergency call and regulatory services, the MME is responsible to trigger positioning procedure to obtain UE geo-location and provide the location to the GMLC. The UE geo-location is used by the LRF associated with the GMLC to route the emergency session to the correct PSAP/emergency centre.

This key issue aims to study how to obtain UE geo-location for IMS emergency call and other regulatory services when the UE initiates IMS emergency call over GEO via NB-IoT NTN connecting to EPC. The following issue is proposed to be studied:

-	How to obtain UE Geo-location for IMS emergency call and other regulatory services.

* End of change *