3GPP TSG-SA WG2 Meeting #168	S2-2503790

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	KI#2: New Solution on AM policy control based on EIF notification

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on AM policy control that the PCF may utilize the notification including Energy Consumption information from the EIF and update the AM policy for a UE based on the received information.

# 1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 Study includes the discussion on solutions that can support policy update based on the notification information from the EIF related to the network energy consumption for a UE/PDU Session per UE/QoS flow per UE. A new solution is proposed to support AM policy control by the PCF based on the energy consumption information received from the EIF.

# 2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change (all new text) * * * *

## 6.X	Solution #X: Access and mobility control based on energy information from EIF

### 6.X.1	Key Issue mapping

This solution is to address access and mobility control that the PCF updates AM policy for a UE based on energy related information received from the EIF.

### 6.X.2	Functional Description

The solution assumes the energy saving indication for per-UE is stored in the subscription data of a UE. The PCF and/or the AMF can retrieve the information during the AM Policy Association or Registration procedure. In case of the dynamic policy control, the PCF retrieves from the UDR and stores the energy saving indication information for UE and provides a policy to control the energy use by a UE during the policy association establishment or modification with the AMF. The PCF can subscribe to the EIF and receive the energy consumption information in a required granularity to control the energy consumed in the network to serve the UE. The PCF may decide, based on the energy related information received from the EIF, operator policy, energy saving indication, and local configuration, if network energy saving is required. The PCF may provide an updated AM policy for UE to the AMF to apply a changed policy for energy saving.

### 6.X.3	Procedures

Figure 6.x.3-1 illustrates registration procedure considering energy related information from the EIF.

Figure 6.36.3-1: Registration procedure considering energy related information

1.	UE sends a Registration Request message to the AMF to attach to the network.

2.	The AMF performs Subscription Retrieval procedure with UDM and receives subscription data for the UE. The subscription data may include energy saving indication information for the UE (e.g. energy control per UE/Network Slice/PDU Session). The AMF may use this information together with operator policy and local configurations to decide control parameters for the UE, e. g. RFSP, service area restriction, etc.

3.	The AMF performs Policy Association Establishment procedure with PCF if dynamic policy is supported. The PCF may provide the energy policy in the AM policy information for the UE based on the energy saving indication and energy related information from the EIF or OAM. The AMF may use the information to set the control parameters for the UE, e.g. RFSP, service area restriction, etc.

4.	The remaining steps for Registration procedure as defined in steps 17 to 24 in Figure 4.2.2.2.2-1 of TS 23.502 [3] proceeds.

5.  The PCF may subscribe to the EIF for notification of the energy-related information related to the UE.

6. The EIF may send the updated energy related information as requested by the PCF in the step 9.

7.	The PCF may decide to change the AM policy for energy saving, e.g., to trigger the aggressive saving mode when the energy consumption in the network increases beyond the defined threshold happens, based on the energy related information included in the notification from the EIF, operator policy, local configuration, etc.

8.	The PCF performs AM Policy Modification procedure with AMF and provides the updated policy for the UE including energy related policy information.

9.	The AMF stores the updated AM policy and provides the updated parameters for access and mobility control of the UE when the Registration procedure is initiated by the UE.

* * * End of Change * * * *