SA WG2 Meeting #168	S2-2503791

Goteberg, Sweden, 07-11 April, 2025

# Title:	AIoT Device Information Configure Service

# Source:	InterDigital Inc., Lenovo

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	Proposal for additional NEF service for AIoT device information configuration.

1. Discussion

In addtion to existing AIoT related NEF services captured in 7.4 , it is also necessary to provide the NEF service that allows the AF to configure the device information (i.e. device profile data such as device identifier, device location, device frequency band, etc) in the 5GC, especially for AIoT devices managed by a 3rd party, which will help the MNO to manage the AIoT devices.

2. Proposal

It is proposed to adopt the proposed text for Clause 7.4 “NEF services” in the draft TS 23.xyz.

* * * * First Change (All new text) * * * *

## 7.4	NEF services

### 7.4.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_AIoTDeviceInfoConfigure | Insert | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

### 7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation.

Input, Optional: Approximate number of AIoT Devices.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

### 7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Disable).

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

### 7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required: AF Transaction ID, a list of AIoT Device ID(s) or Failure Cause in case of Failure, Information obtained from each target AIoT Device.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: None.

Editor’s note:	The NEF AIoT service operation definition needs to be revisited to align with AIOTF services.

### 7.4.5	Nnef_AIoTDeviceInfoConfigure

Service operation name: Nnef_AIoTDeviceInfoConfigure

Description: The consumer uses the service to configure AIoT device information in AIoT ADM.

Input, Required: AF Identifier, AIoT device identifier(s).

Input, Optional: AIoT device profile data, device location, device frequency band, etc.

Output, Required: Operation execution result indication.

Output, Optional: None.

* * * * End of Change * * * *