SA WG2 Meeting #168	S2-2503792

Goteberg, Sweden, 07-11 April, 2025

# Title:	Update to Inventory procedure regarding AIOTF selection

# Source:	InterDigital Inc.

# Document for:	Approval

Agenda Item:	19.14.2

Abstract:	The contribution proposes text to address the AIOTF selection open issue in the Inventory procedure.

1. Discussion

There is an EN regarding AIOTF selection in the existing Inventory procedure (Step 2).

Editor's note:	How the NEF selects the AIOTF needs further definition.

If the Target Area information is available in the Inventory Request, the NEF may query the NRF for the AIOTF(s) that support the Target Area, assuming the AIOTFs may register its supported areas as part of its NF profile with the NRF. If the NRF returns multiple AIOTFs that support the Target Area, the NEF may select one or multiple AIOTF(s) based on its implementation.

If the Target Area information is not available and AIoT Device ID information is available, the NEF may query the ADM for the Last-known AIOTF, and if it is available, selects the Last-known AIOTF.

2. Proposal

It is proposed to adopt the proposed change to Clause 6.2.2 “Inventory Procedure” in TS 23.369.

* * * * First Change (All new text) * * * *

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices])  service operation request to the NEF.

Editor's note:	The parameters for the inventory service operation need further definition.

2.	The NEF may select an AIOTF based on NEF local configuration or, if the Target Area information is available, the NEF  may query the NRF for the AIOTF(s) that support the Target Area. If there are multiple AIOTFs that support the Target Area, the NEF may select one or multiple AIOTF(s) based on NEF implementation. If the Target Area information is not available and AIoT Device ID information is available, the NEF may query the ADM for the Last-known AIOTF, and if it is available, select the Last-known AIOTF.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target area information], [AIoT Device ID identification information], [Approximate number of AIoT Devices]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a correlation ID corresponding to this AF service operation request, as well as the AIoT Device Identification information to be included in the paging message sent by AIoT RAN.

AIOTF performs Reader Selection, see clause 5.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the correlation ID, the AIoT Device Identification information to be included in the paging message, and assistance information to the selected AIoT RAN.

Editor's note:	AIoT Device Identification information needs further definition.

Editor's note:	Whether the Inventory Request sent to AIoT RAN includes indication about whether there will be a follow up command or not needs to be determined.

8.	The AIoT RAN sends an Inventory Response to the AIOTF with the correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly.

9.	Upon reception of the Inventory Request message from the AIOTF, the AIoT Reader(s) will execute the inventory operation.

If an AIoT device matches the AIoT Device Identification information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its AIoT Device ID.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	AIoT RAN sends one or more Inventory Report messages to the AIOTF including the correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s).

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the AIoT_Notify message including a list of AIoT Device ID(s). If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

Editor's note:	Whether only one, or more than one report is generated by the AIOTF to send to the AF needs clarification.

13.	The NEF informs the AF of the outcome of the AIoT_Inventory request by sending the AIoT_Notify message including the AIoT Device ID(s).

* * * * End of Change * * * *