SA WG2 Meeting #168	S2-2503795

07 - 11 April, 2025, Goteborg

Source:	Samsung

Title:	WT#2 use case: Prevention of unexpected service experience degradation

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a use case to cover Work Task#2 of the FS_AIML_CN_Ph2 SID.

# 1	Discussion

This paper discusses a use case for FS_AIML_CN_Ph2 (SP-250413) Work Task #2 (WT#2), as described below.

Whether and how to enhance the 5GC analytics to support efficient performance user plane. Note 4: This may be based on the analysis of the user plane traffic (e.g. traffic patterns, packet information) and the reaction of the network per the analysis is based on the existing policy framework. Note 5: The study will not address how UPF detects wrong or malformed or duplicated packets. This WT shall have neither RAN or UE impacts. Solutions shall minimize the load on Control Plane NFs due to too much reporting from UPF.

A related use case for the WT#2 is provided in accordance with the SA2 guidance (SP-250418).

# 2	Proposal

It is proposed to include the following changes in TR 23.700-04 V.0.0.

* * * * Start of Changes * * * *

### 5.X.1	Use Case #X: Prevention of unexpected service experience degradation

When rate limit control (e.g., session AMBR) is enforced, some DL packets can be dropped due to the buffer status at UPF and DL packet arrival rate beyond the applied rate limit at UPF. For certain cases, a packet like SYN+ACK during TCP handshake can be unfortunately dropped. If a user enjoying the web browser suddenly experiences severe impact on service experience degradation such as connection failure with error messages like “server not responding” or “request time out”. To analyze and mitigate this user plane traffic behavior or event, 5GC analytics can be enhanced to generate statistics what kind of traffic was dropped when a specific traffic management such as rate limit is applied for a given PDU Session. For this purpose, the newly introduced feature in Rel-19, “Handling of Payload Headers”, can be utilized to collect payload header information and its related UPF behavior in conjunction with the existing input data at NWDAF. Potential mitigation method can be investigated to enable bypassing, exempting, or prioritizing of a certain type of packet from rate limit control based on the corresponding new NWDAF-provided analytics.

In TS 23.288, NWDAF can collect the various data related to the user plane packet traffic, but data collection related to “Handling of Payload Header” and its use on the enhanced analytics service have not been standardized.

* * * * End of Changes * * * *