3GPP TSG-SA2 Meeting #168	S2-2503801

Goteborg, SE, Apr. 7th – 11th 2025

Source:	Huawei, HiSilicon

Title:	KI#2, New Sol: Service authorisation and revocation

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The service authorization and revocation for FS_Sensing_ARC are proposed.

# 1. Introduction

This pCR proposes the solution for Key issue#2 service authorisation and revocation.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

# * * * * Second change (All text new) * * * *

## 6.X	Solution #X: sensing service authorization and revocation

### 6.X.1	Description

This solution addresses key issue#2 regarding authorization and revocation for the AF who invokes the sensing service.

Specifically, in terms of service authorization, when AF requests the sensing service, 5GC needs to perform authorization for the AF to ensure that only authorized AF can access and use the sensing service, since it is critical to maintain the security of the 5G system and the efficiency usage of the network resource.

In terms of service revocation, when the AF no longer needs sensing service, the AF can actively request to cancel the corresponding sensing service. In doing so the sensing-related resource can be released in time avoiding unnecessary resource wasted.

### 6.X.2	Procedure for service authorization

Figure 6.X.2-1: service authorization procedure.

NOTE:	This sub-clause provides the details of service authorization and invocation procedure, and corresponds to step 1-2 and 4-7 of the sensing general framework (in Sol#X).

- The AF sends Nnef_SensingExposure_Subscribe request message to the NEF. The request contains the sensing service requirement(s) (e.g. Target Sensing Service Area, one-time report/periodical report/event-triggered, etc), sensing service type and corresponding information to obtain required Sensing Result from network.

- Upon receiving request from AF, the NEF authorizes the AF for usage of the sensing service based on AF ID. If the authorization is not granted, the rest of the procedure (step 5-6) is skipped and NEF responds to the AF the failure of the sensing service authorization in step 8.

- If the authorization succeeds, then the NEF sends the Nsf_SensingSubscription_Subscribe request message to the Sensing Function. The request message contains the sensing service requirement (e.g., Target Sensing Service Area, one-time report/periodical report/event-triggered, etc), sensing service type and corresponding information.

- Upon receiving request from NEF, the Sensing Function may perform authorization based on the sensing service requirement, for example, the Sensing function determines whether the sensing can be performed in the requested Target Sensing service area. If the authorization not granted, the rest of the Sensing procedure (step 8 -15 in in Sol#X) is skipped and Sensing Function responds to the NEF the failure of the sensing service authorization in step 7. If the authorization succeeds, the Sensing Function responds to the NEF the Success of the sensing service authorization in step 7, and the Sensing Function performs the Sensing procedure to obtain the Sensing result .

- The Sensing Function sends Nsf_SensingSubscription_Subscribe response message to the NEF, the response message indicates whether the sensing service subscription is successful or not

- The NEF sends sensing service subscription response message to the AF, the response message indicates whether the Nnef_SensingExposure_Subscribe response is successful or not.

### 6.X.3	Procedure for service revocation

Figure 6.X.3-1: service revocation procedure

- The AF sends a Nnef_SensingService_Unsubscribe request message to the NEF indicating the request to revoke the sensing service.

- Upon receiving the unsubscribe request from AF the NEF sends the Nsf_SensingService_Unsubscribe request message to the Sensing Function to cancel subsequent reporting of the sensing results.

- The Sensing Function cancels the reporting of related Sensing results. The Sensing function determines the cancellation of the Sensing procedure.

- The Sensing Function sends Nsf_SensingService_Unsubscribe response message to the NEF to indicate that the service has been cancelled.

- The NEF sends Nsf_SensingService_Unsubscribe response message to the AF to indicate that the services has been cancelled.

### 6.X.4	Impacts on services, entities and interfaces

AF:

-	Support sensing request and revocation service

NEF:

-	Support sensing service authorization and revocation

Sensing function:

-	Support sensing service authorization and revocation

# * * * * End of changes * * * *