3GPP TSG-SA2 Meeting #168	S2-2503812

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0552 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | End UE RSC Bounded Routing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 23.304, clause 5.14.3.2, Relay UE has distinguished the routing logic with different RSC. But this rule applies to End UE as well. This CR addresses the End UE for the routing proxy according to the same principle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the routing difference to different RSC for End UE in clause 5.14.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The End UE will have wrong routing behavior when multiple RSCs are used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

#### 5.14.3.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used. The 5G ProSe Multi-hop UE-to-UE Relays supporting the same RSC establishe IP based Layer-2 links with each other and form a relay cloud to provide IP connectivity for 5G ProSe End UEs connected to any of the 5G ProSe Multi-hop UE-to-UE Relays.

NOTE:	If a 5G ProSe Multi-hop UE-to-UE Relay support multiple RSCs, different Layer-2 links will be established, with each associated with a specific RSC. The 5G ProSe UE-to-UE Relay operates as separate logical entities in the separate relay clouds of different RSCs, i.e. no routing table sharing and no traffic forwarding across different relays clouds of different RSCs. The 5G ProSe End UE should have separate IP address space, separate routing with different RSCs as well.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets).

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connectivies in the relay cloud to provide multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relay acts as a MANET router. The exact MANET routing protocols to be supported, e.g. Neighborhood Discovery Protocol (NHDP) [36], OLSRv2 [35], are associated with the RSCs based on configuration.

Based on configuration associated with the RSC, the 5G ProSe End UE obtains the IP address/prefix in two options:

-	Each of the 5G ProSe End UE may be configured with a routable IP address/prefix associated with the RSC. This IP address/prefix does not change when the 5G ProSe End UE changes the connections with 5G ProSe Multi-hop UE-to-UE Relays.

-	If the 5G ProSe End UE does not have an IP address/prefix configured for the RSC, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay it connects to. In this case, the 5G ProSe End UE may need to change the IP address/prefix when it changes its connection to another 5G ProSe UE-to-UE Relay.

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries. 5G ProSe Multi-hop UE-to-UE Relay needs to support DNS operations.

Once the 5G ProSe Multi-hop UE-to-UE Relay becomes aware of the 5G ProSe End UE's IP address/prefix, the 5G ProSe Multi-hop UE-to-UE Relay propagates the related DNS information (i.e. User Info ID and associated IP address/prefix) to all other 5G ProSe Multi-hop UE-to-UE Relays connected to the same MANET using a dedicated MANET Discovery Info message. The MANET Discovery Info message is diffused throughout the MANET using Multipoint Relay flooding (MPR flooding) as defined in IETF RFC 7181 [35], if available. If MPR flooding is not available, then the MANET Discovery Info message is diffused using simple flooding i.e. by transmitting a copy of the MANET Discovery Info message on all interfaces except the one on which the message has been received.

The MANET Discovery Info message is based on the generalized MANET message format defined in IETF RFC 5444 [37]. The 5G ProSe Multi-hop UE-to-UE Relay initiating the sending of the MANET Discovery Info message controls the propagation depth of the MANET Discovery Info message based on the configured "maximum number of hops per RSC" as specified in 5.1.5.1a, by using the built-in mechanism defined in IETF RFC 5444 [37].

When MANET protocol determines the route for IP forwarding, layer-2 link QoS and hop counts can be refelected, e.g. with the Link Metrics value as defined in OLSRv2 [35]. The 5G ProSe Multi-hop UE-to-UE Relay can also enforce the configured hop limit based on the link metrics when building the routing tables, e.g. ignore the route that exceeds the threshold derived from hop limit.

If 5G 5G ProSe Multi-hop UE-to-UE Relay has traffic for its own, it can use any IP address it owns for the transmission.

* * * * End of Changes * * * *