3GPP TSG-SA WG2#168	S2-250xxxx

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-14: FS_Sensing-ARC new key issues

Document for:	Discussion/Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: This contribution proposed the key issues for TS 23.700-14 for study on Integrated Sensing and Communication (ISAC) based on the approved SID.

# 1.	Discussion

According to the approved study item on Study on Stage 2 for Integrated Sensing and Communication (SP-250401), there are several general WTs below that need to be reflected as the key issues in the TR:

-	WT-2: Service authorization and revocation. This includes investigating mechanisms for authorization and revocation sensing service requests from the service consumer.

-	WT-3: Discovery and selection of sensing entities based on service requirements triggered by the service request and capability of the sensing entities.

-	WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

Therefore, this contribution proposed the related key issues, with additional considerations for these issues.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

## 5.X	Key Issue #X: Architectural Enhancement to support ISAC service authorization and revocation

### 5.X.1	Description

The key issue is to address the service authorization and revocation for the ISAC operation. There are two types of authorization and revocation involved in the ISAC operation:

-	Service authorization and revocation of a ISAC session request; and

-	Service authorization and revocation for a sensing entity.

For the service authorization and revocation of the ISAC session request, the study needs to address the following aspects:

-	the criteria for the service authorization and revocation, e.g.: based on identification of the entity initiated the ISAC session; the session target area or identity of a target object; quality level of the sensing results, etc.

-	which entity performs the service authorization and revocation;

-	where to store the service authorization and revocation data, and how to update the authorization data.

For the service authorization and revocation for the sensing entities, the study needs to address the following aspects:

-	which entity performs the sensing entity authorization and revocation;

-	granularity of the sensing entity authorization, e.g. based on a location, based on a time period, or based on certain sensing methods;

-	storage and management of the authorization data for the sensing entities.

# >>>>NEXT CHANGE<<<<

## 5.Y	Key Issue #Y: Discovery and selection of sensing entities for an ISAC session

### 5.Y.1	Description

The key issue is to address the discovery and selection of the sensing entities for the ISAC operation. The discovery and selection of sensing entities can be per ISAC session. The study needs to address the following aspects:

- 	which entity(/entities) performs the sensing entity discovery and selection;

-	the criteria for the sensing entity selection, e.g. based on the sensing target/area, method, QoS of sensing, etc.

-	the procedures for the discovery of sensing entities for an ISAC session.

# >>>>NEXT CHANGE<<<<

## 5.Z	Key Issue #Z: Efficient exposure of ISAC data or results

### 5.Z.1	Description

The key issue is to address the efficient exposure of ISAC data or results. Comparing to other type of non-data services, e.g. location service, the ISAC service may involve a large amount of data. Given that sensing operation shares the same system as that for data communications, the study needs to address the following aspects:

- how to efficiently transport and exposure the ISAC related data without affecting the operation of communication services, i.e. between RAN and CN, and from CN to external entities;

- investigate the feasibility and impacts of transferring sensing data/results from RAN to CN sensing NF without going through AMF.

# >>>>END OF CHANGES<<<<