3GPP TSG-SA2 Meeting #168	S2-2503822

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1451 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | An optimized method for AF to obtain the DN-facing UE IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | AF correlates the UE Data Collection with NWDAF request using DN-facing UE IP address of the UE ID (SUPI or GPSI) provided by the NWDAF. - The procedure defined for the AF to obtain UE private IP address is described in clause 6.2.8.2 4.2. - If NAT is enabled, the procedure defined for the AF to obtain UE public IP address is described in clause 6.2.8.2.4.5. In this CR, we propose an optimized procedure which can be used by the AF to obtain the DN-facing UE IP address regardless of NAT being enabled or disabled. Also in the existing procedure, the AF needs to interact with multiple NFs like UDM, SMF, UPF in a sequence to obtain the UE public IP address. The existing procedure neither addresses the requirement for future PDU sessions nor is it optimized for multiple UEs. The proposal in this CR is to have a single service offered by an NF which can internally chain the services to get the required information. The AF can invoke this service for a requested UE ID to directly get the DN-facing IP address irrespective of NAT status being known to the AF. - The Nudm_EventExposure service of UDM can be used to create subscriptions to get information related to PDU sessions for multiple UEs and for future PDU sessions. - A new event, “DN-facing UE IP address”, is proposed to be added in Nudm_EventExposure service that could provide the consumer with the DN-facing UE IP address for UE(s)’s PDU session identified by DNN and/or S-NSSAI. For the new event, the UDM shall invoke the Nsmf_EventExposure service of SMF to fetch the DN-facing UE IP address. - With the addition of new event, “DN-facing UE IP address”, in Nsmf_EventExposure service; SMF will invoke the Nupf_EventExposure service of UPF to get UE public IP address provided NAT exposure is enabled for the serving UPF. - For AF interacting with the core through NEF, Nnef_UEAddress_Subscribe service operation is enhanced to provide DN-facing UE IP address(es) for UE(s) identified by GPSI/External Group Identifier. Thus proposed procedure allows the AF to create a single subscription with UDM with input parameters as UE ID, DNN and/or S-NSSAI to fetch the DN-facing UE IP address. And it also supports fetching DN-facing UE IP address for multiple UEs and for future PDU session(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a new clause 6.2.8.2.4.x which describes the procedure used by AF to correlate UE data collection and NWDAF request using DN-facing UE IP address regardless of NAT status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved stage-2 would be: - missing optimization in the AF procedure to obtain DN-facing UE IP address. - missing support for future PDU session or for group of UEs in the AF procedure to obtain DN-facing UE IP address. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.2.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change***

#### 6.2.8.2.x	AF correlates UE data collection and NWDAF request using DN-facing UE IP address regardless of NAT status

This procedure applies regardless of NAT status between the UE and the AF.

If the AF receives the Naf_EventExposure_Subscribe request including Target for Event Reporting set to a UE’s ID (GPSI or SUPI) and the AF does not locally store the DN-facing UE IP address; the AF creates a subscription by invoking Nudm_EventExposure service for the event "DN-facing UE IP address" by providing the UE’s ID, S-NSSAI and DNN. Whenever UDM identifies the matching PDU sessions establishment for the input parameters provided, it invokes Nsmf_EventExposure service of serving SMF for the event "DN-facing UE IP address" by providing PDU session ID, UE ID. If the serving UPF supports NAT exposure, the serving SMF requests the UE public IP address by invoking Nupf_EventExposure_Subscribe service operation for the UE NAT Mapping Information event.

Figure 6.2.8.2.4.x-1: AF correlates UE data collection and NWDAF request using the DN-facing UE IP address

For an AF authorized by NEF, the following steps 0-8 apply. For AF that interacts directly, the following steps 0 and 2-7 apply, with GPSI replaced by SUPI and NEF replaced by AF.

- NWDAF subscribes to the AF by using Naf_EventExposure_Subscribe service described in clause 5.2.19.2.2 of 3GPP TS 29.502 [3], with the Target for Event Reporting set to GPSI.

- The AF creates a subscription for the DN-facing UE IP address by invoking the Nnef_UEAddress_Subscribe service operation by providing GPSI/external identifier and AF identifier.

- The NEF creates a subscription with UDM by invoking the Nudm_EventExposure_Subscribe service operation for the DN-facing UE IP address event by providing SUPI/group identifier, DNN, S-NSSAI to the UDM.

- When the PDU session is established, the UDM requests the serving SMF by invoking the Nsmf_EventExposure_Subscribe service operation for the DN-facing UE IP address event by providing PDU session ID and SUPI.

- The serving SMF then verifies if NAT information exposure is enabled for serving UPF. If NAT information exposure is enabled, the SMF requests the UPF to provide the UE public IP address by invoking the Nupf_EventExposure_subscribe service operation for the UE NAT Mapping Information event described in clause 5.2.1.3.6 of 3GPP TS 29.564 [51]. However, if NAT information exposure is not supported by the UPF then SMF will send a failure message with the UE private IP address and an indication that UE private IP address is being sent to the UDM.

- The serving UPF will respond with the UE public IP address only if NAT is enabled.

- The SMF responds to the UDM with the DN-facing UE IP address which is:

- the UE public IP address if provided by the UPF or

- the UE private IP address which is available in the SMF.

- UDM notifies the DN-facing UE IP address to the NEF.

- NEF notifies the DN-facing UE IP address to the AF.

* * * End of Change * * *