3GPP TSG-SA WG2 Meeting #168	S2-2503832

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	KI#2: New Solution on policy control based on Energy Consumption Information for Network Slice

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution on policy control that the PCF may utilize the notification including Energy Consumption information of the network slice from the EIF and update the AM policy for one or more UE(s) using the network slice based on the received information.

1.	Discussion

One of the key issues for Rel-20 EnergySys Phase2 Study includes the discussion on solutions that can support policy update based on the notification information from the EIF related to the network energy consumption for a network slice. A new solution is proposed to support AM policy control by the PCF based on the energy consumption information received from the EIF.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change (all new text) * * * *

6.X	Solution #X: Access and mobility control based on energy information for network slices from EIF

6.X.1	Key Issue mapping

This solution is to address access and mobility control that the PCF updates AM policy for one or more UEs for the network slice based on energy related information for the network slice received from the EIF.

6.X.2	Functional Description

The solution assumes the EIF can collect information to calculate energy consumption information for the network and expose the energy consumption information for the network slice. Also, the PCF can obtain the energy consumption information for the network slice including the indication that the energy saving needs to be applied. Based on the energy consumption information, the PCF updates the AM policies for the UEs to reduce the energy consumption for the network slice.

6.X.3	Procedures

AMF

PCF

UE

EIF

1. Determine that energy consumption information has changed

4. AM Policy Association Modification

3. Select UEs

5. Update parameters based on AM Policy

Figure 6.x.3-1 illustrates registration procedure considering energy related information from the EIF.

Figure 6.36.3-1: Registration procedure considering energy related information

- The EIF determines that the energy consumption information for S-NSSAI has changed. The energy consumption information may include the indication that the energy saving needs to be applied to the S-NSSAI.

- The EIF provides energy consumption information and the S-NSSAI to the PCF.

- To reduce the energy consumption on the S-NSSAI, the PCF determines the UEs for which the AM policy need to be updated. The PCF determines to update the AM policy for the UE for which the energy saving indicator is stored in the subscription information.

- The PCF performs AM Policy Modification procedure with AMF and provides the updated policy for the UE including energy related policy information.

- The AMF stores the updated AM policy and provides the updated parameters for access and mobility control of the UE.

6.X.4	Impacts on existing services, entities and interfaces

EIF:

-	Collects information to calculate energy consumption information for Network Slice.

-    Expose energy consumption information for Network Slice.

PCF:

-	Obtains the energy consumption information for Network Slice from the EIF and update the AM policy for the UEs to reduce energy consumption on the Network Slice

* * * End of Change * * * *